Kingston Zoning By-law Number 2022-62

# Part 1 of 5: Sections 1 to 19

Table of Contents

Section 1:	Administration	Part 1, Page 3

Section 2:	Interpretation of This By-law	Part 1, Page 11

Section 3:	Definitions	Part 1, Page 20

Section 4:	General Provisions	Part 1, Page 65

Section 5:	Overlay Provisions	Part 1, Page 99

Section 6:	Specific Use Provisions	Part 1, Page 112

Section 7:	Parking, Loading and Bike Parking Provisions	Part 1, Page 122

Section 8:	Rural Zones (AG, RU, RUR, LSR, RC and HAM)	Part 1, Page 146

Section 9:	Rural Industrial Zones (RM1, RM2 and MX1)	Part 1, Page 159

Section 10:	Mixed Use Zones (WM1, WM2, DT1 and DT2	Part 1, Page 166

Section 11:	Urban Residential Zones (UR1 to UR9)	Part 1, Page 188

Section 12:	Urban Multi-Residential Zones (URM to URM10)	Part 1, Page 211

Section 13:	Heritage Zones (HCD1 to HCD3)	Part 1, Page 230

Section 14:	Institutional Zones (IN1, IN2, G1 and G2)	Part 1, Page 242

Section 15:	Commercial Zones (CN, CA, CD, CR, CG and CW)	Part 1, Page 251

Section 16:	Employment Zones (M1 to M5)	Part 1, Page 263

Section 17:	Transportation and Utilities Zones (TA, TR and TU)	Part 1, Page 274

Section 18:	Open Space Zones (OS1, OS2 and DR)	Part 1, Page 279

Section 19:	Environmental Zones (EPA)	Part 1, Page 284

Section 20:	Legacy Exceptions (LXXX)	Part 2, Page 1

Section 21:	Exceptions (EXXX)	Part 3, Page 1

Section 22:	Holding Conditions (HXXX)	Part 4, Page 1

Section 23:	Temporary Zones (-T)	Part 5, Page 1

Section 24:	Amendments to this By-law	Part 5, Page 2

(By-Law Number 2022-62; 2024-332)

Kingston Zoning By-Law Number 2022-62

A by-law to regulate the use of lands and the size and location of buildings within the City of Kingston, pursuant to Section 34 of the Planning Act.

This By-Law was Given all Three Readings and Passed: April 26, 2022

The Council of The Corporation of the City of Kingston enacts as follows:

# Section 1:	Administration

### Title of This By-law

    - This By-law is composed of the text in Sections 1 to 24 and the following schedules attached to and forming part of this By-law, and may be cited as the “Kingston Zoning By-law” or “ZBL 2022-62”:

      - Schedule 1, Zoning Map;

      - Schedule 2, Parking Areas;

      - Schedule 3, Required Ground Floor Commercial;

      - Schedule 4, Road Classification;

      - Schedule A, Floodplain Overlay;

      - Schedule B, Source Water Protection Overlay;

      - Schedule C, Airport Noise Exposure Overlay;

      - Schedule D1, Urban Constraint Areas and Second Rural Unit Holding Overlay;

      - Schedule D2, Third Rural Unit Holding Overlay;

      - Schedule D3, Fourth Urban Residential Unit Holding Overlay;

      - Schedule E, Exception Overlay; and

      - Schedule F, Holding Overlay.

      - Schedule G, Non-Residential Conversion Holding Overlay;

      - Schedule H, Express Transit Area Overlay;

      - Schedule J, Servicing Allocation Policy Overlay

(By-Law Number 2022-62; 2024-332; 2024-333)

    - Any references to “this By-law” mean the Kingston Zoning By-law.

### Area to Which This By-law Applies

    - The provisions of this By-law apply to all lands within the municipal boundaries of the City of Kingston as shown on Schedule 1, other than the lands identified as “Not Subject to this By-law” and “N/A” on Schedule 1.

    - All lands under waterbodies within the municipal boundaries of the City of Kingston are subject to this By-law in addition to the requirements of both federal and provincial legislation.

### Compliance with Applicable Law

    - No person may use or develop any lot or building within the City of Kingston except in full compliance with all of the provisions of this By-law.

    - This By-law is not to be interpreted as reducing or mitigating any other by-law, provision, regulation, or restriction lawfully imposed by the City or any other public authority having jurisdiction.

    - Nothing in this By-law serves to relieve any person from the obligation to comply with the provisions and requirements of any by-law of the City in force from time to time or the obligation to obtain any licence, permit, authority, or approval required under the by-laws of the City or of any other public authority having jurisdiction.

    - In the event of a conflict between this By-law and any City by-law, this By-law prevails to the extent of the conflict.

    - No other by-law, provision, regulation or Act is to be interpreted as reducing or mitigating any provision of this By-law, unless the other by-law, provision, regulation or Act was specifically intended to affect zoning and the public authority responsible for the by-law, provision, regulation or Act has the jurisdiction to do so.

### Effective Date of This By-law

    - This By-law comes into force and takes effect on the day it was passed, if no appeals are filed, or where one or more appeals are filed, in accordance with the provisions of the Planning Act.

### Repeal of Existing By-laws

    - The following former zoning by-laws are repealed in their entirety upon the date this By-law comes into force and effect:

      - By-law Number 3078 of the former City of Kingston, as amended;

      - By-law Number 9087A of the former City of Kingston, as amended;

      - By-law Number 8950 of the former City of Kingston, as amended;

      - By-law Number 541 of the former Township of Kingston, as amended; and

      - By-law Number 3077 of the former City of Kingston, as amended.

    - The following former zoning by-laws are repealed upon the date this By-law comes into force and effect, except for those portions of the following former zoning by-laws that apply to lands identified as “Not Subject to this By-law” and “N/A” on Schedule 1 of this By-law:

      - By-law Number 8499 of the former City of Kingston, as amended;

      - By-law Number 96-259 of the former City of Kingston, as amended;

      - By-law Number 76-26 of the former Township of Kingston, as amended;

      - By-law Number 97-102 of the former Township of Kingston, as amended; and

      - By-law Number 32-74 of the former Township of Pittsburgh, as amended.

    - Despite Clauses 1.5.1. and 1.5.2., the applicable former zoning by-laws apply to assist in the interpretation of any minor variance referred to in Clause 1.7.1. in the context of this By-law. Where a conflict exists between the provisions of this By-law and the applicable former zoning by-laws in respect of the interpretation of any minor variance referred to in Clause 1.7.1., the provisions of this By-law prevail.

### Transition Provisions

#### Complete Applications for a Building Permit

    - Nothing in this By-law prevents the development or use of a lot or a building for which a complete application for a building permit was received by the City on or before the date of passing of this By-law, if the development or use complies, or the building permit application is amended to comply, with the provisions of the applicable former zoning by-law as it read immediately prior to the passing of this By-law.

#### Other Types of Complete Applications

    - Where a complete application was received by the City on or before the date of passing of this By-law for the development or use of a lot or one or more buildings, approval may be granted in the context of the applicable former zoning by-law as it read immediately prior to the passing of this By-law, for one or more of the following applications where they are consistent with the intent of the complete application:

      - Any application under Section 45 of the Planning Act;

      - Site plan control approval pursuant to Section 41 of the Planning Act;

      - Consent pursuant to Section 53 of the Planning Act;

      - Draft plan of subdivision pursuant to Section 51 of the Planning Act or a description under the Condominium Act, 1998, S.O. 1998, c.19;

      - Payment in lieu of parking agreement pursuant to Section 40 of the Planning Act; and

      - A part lot control exemption pursuant to Section 50 of the Planning Act.

    - Where the development or use of a lot or one or more buildings qualifies under Clause 1.6.2., a building permit may be issued after final approval is received for all required applications if the development or use complies, or the building permit application for the development or use is amended to comply, with the provisions of the applicable former zoning by-law as it read immediately prior to the passing of this By-law.

    - Nothing in this By-law applies so as to continue the exemption provided by Clauses

1.6.1. and 1.6.2. beyond the issuance of the final building permit upon which the exemptions are founded.

    - Clauses 1.6.1., 1.6.2., 1.6.3. and 1.6.4. are repealed in their entirety three years after the date of passing of this By-law.

### Minor Variances

#### Continuation of Approved Variances

    - During the three-year period commencing on the date of passing of this By-law and expiring on the date specified in Clause 1.6.5., a building permit may be issued in

the context of the applicable former zoning by-law as it read immediately prior to the passing of this By-law for any development subject to one or more approved minor variances under the provisions of the applicable former zoning by-law.

    - After the three-year period in Clause 1.6.5. expires, minor variances approved under the provisions of the applicable former zoning by-law may be relied upon only if the zoning provision respecting the minor variance is the same or more permissive in this By-law than in the applicable former zoning by-law.

### Non-Conformity and Non-Compliance

#### Legal Non-Conforming Uses

    - A use that is not permitted by this By-law, but which was lawfully used for such purpose on the date of passing of this By-law, is considered a legal non-conforming use. Nothing in this By-law applies to prevent a legal non-conforming use, so long as it continues to be used for that purpose.

#### Legal Non-Complying Buildings

    - A building that does not meet the provisions of this By-law, but which lawfully existed on the date of passing of this By-law, is considered to be a legal non- complying building. Nothing in this By-law applies to prevent a legal non- complying building so long as it continues to exist.

    - If a lot contains a legal non-complying building, nothing in this By-law applies to prevent the further development of such lot, provided the development:

      - Does not further increase the extent or degree of non-compliance with the provisions of this By-law; and

      - Complies with all other applicable provisions of this By-law.

    - If a lot contains a legal non-complying building, nothing in this By-law applies to prevent the development of an accessory building on the lot, provided that the development complies with all applicable provisions of this By-law.

#### Legally Existing Lots

    - A lot in existence on the date of passing of this By-law that does not meet the minimum lot area or lot frontage provisions of the underlying Zone is permitted to be used and buildings are permitted to be developed provided the use conforms with this By-law and the buildings comply with all other provisions of this By-law, including Subsection 4.8. requiring frontage on a street.

(By-Law Number 2022-62; 2024-332)

    - A lot in an AG Zone or RU Zone that is reduced in size through a severance application and is less than 40 hectares in lot area is permitted to be used and developed for agricultural uses provided any new buildings comply with all other provisions of this By-law, including Subsection 4.8, requiring frontage on a street.” A lot in a HAM Zone or RUR Zone in existence on the date of passing of this By-law that does not meet the minimum lot area or lot frontage provisions of the underlying Zone is permitted to be used and buildings are permitted to be developed provided the use conforms with this By-law and the buildings comply with all other provisions of this By-law, including Subsection 4.8. requiring frontage on a street.

(By-Law Number 2022-62; 2024-332)

### Conveyances to Public Authorities

    - No person is deemed to have contravened any provision of this By-law by reason of the fact that any part or parts of any lot has or have been conveyed to or acquired by the City or any public authority.

    - Where any portion of a lot is taken by the City or any public authority for the purpose of a street widening, sight triangle, turning lane or other similar use, existing buildings are not deemed to be legal non-complying as a result of the reduced setbacks or reduced lot area.

    - Where any portion of a lot is taken by the City or any public authority for the purpose of a street widening, sight triangle, turning lane or other similar use, new development must be calculated on the entire lot area including the area taken by the City or any public authority. For clarity, this includes provisions that are measured based on lot area, such as floor space index or lot coverage.

    - Where any portion of a lot is taken by the City or any public authority for the purpose of a street widening, sight triangle, turning lane or other similar use, new

development must comply with setbacks to the lot lines as they exist at the time of development.

### Administration of This By-law

    - Unless otherwise stated, this By-law is administered and enforced by the Director.

#### Technical Revisions to this By-law

    - Provided that the purpose, intent, meaning and substance of this By-law is not affected, the Director may undertake the following technical revisions without a zoning by-law amendment:

      - Changing numbering, cross-references and the arrangement of text, tables and schedules;

      - Revisions to the base mapping and parcel fabric updated from the Ontario Land Registry Office;

      - Correcting lot and feature boundary errors;

      - Revisions to Schedule 4, Road Classification, when new roads are constructed or when the road alignment has changed;

      - Amendments to correct or clarify existing items that are not part of this By-law in Clauses 2.2.3., 2.2.4. and 2.2.5., or revisions to add new items or delete existing items listed in Clauses 2.2.3., 2.2.4. and 2.2.5.;

      - Correcting grammatical, mathematical or typographic errors and altering punctuation; and

      - Replacing a description of a date or time with an actual date or time.

### Remedies

    - Where any matter or thing is required to be done by a person under the provisions of this By-law, Council may direct that in default of it being done by said person, such matter may be done by the City at the sole expense of the person and the expense thereof with interest may be recovered by the City in accordance with the provisions of the Municipal Act, 2001, S.O. 2001, c. 25.

### Validity or Severability

    - If a decision of a court of competent jurisdiction declares that one or more of the provisions of this By-law, including anything contained in the Schedules, are invalid and the judgment does not affect the validity of the remaining portions of this By- law, then the remaining portions are in full force and effect.

### Enforcement and Penalties

    - Every person who contravenes this By-law is guilty of an offence, and on conviction is liable:

      - On a first conviction, to a fine of not more than $25,000.00; and

      - On a subsequent conviction, to a fine of not more than $10,000.00 for each day or part thereof upon which the contravention has continued after the day on which the person was first convicted.

    - Despite Clause 1.13.1., every corporation that contravenes this By-law is guilty of an offence, and upon conviction is liable:

      - On a first conviction, to a fine of not more than $50,000.00; and

      - On a subsequent conviction, to a fine of not more than $25,000.00 for each day or part thereof upon which the contravention has continued after the day on which the corporation was first convicted.

    - In addition to any other remedy or any penalty provided by law, where a conviction has been entered, any court of competent jurisdiction thereafter may make an order prohibiting the continuation or repetition of the offence by the person convicted.

    - Subject to and in accordance with the provisions of the Planning Act, where the Director believes on reasonable grounds that this By-law is being contravened, the Director or any person acting under their instructions may, at all reasonable times, enter and inspect any property on or in respect of which they believe the contravention is occurring.

# Section 2:	Interpretation of This By-law

### Language and Meaning

#### General

    - In this By-law, if words, terms or phrases are formatted in a bold font, they have the meaning provided in Section 3, Definitions. The definitions and interpretations set out in Section 3 apply and any words not specifically defined in this By-law carry their ordinary meaning.

    - Definitions are given in this By-law to aid in the understanding and the implementation of the true spirit, intent, and meaning of this By-law. They are not to be used to avoid an obligation imposed by this By-law or any requirement enacted in a substantive provision of this By-law.

    - Despite the tense used in a provision:

- Every provision of this By-law must be applied to the circumstances as they exist at the time in question;

- Every obligation imposed by this By-law is a continuing one so long as either the use, the circumstances, the reason for the obligation, or the events which caused, precipitated or gave rise to the obligation continue; and

- Any reference to legislation or provisions or regulations or sections thereof approved by another public authority includes any amendments to or successions thereof.

#### Singular and Plural Words

    - In this By-law, unless otherwise specifically indicated:

- Words used in the singular include the plural and vice versa;

- This By-law is gender neutral and, accordingly, any reference to one gender includes all genders; and

- Word variations, for example: comply, complying, compliance, complies, have a similar meaning.

#### Including or Excluding

    - In this By-law, lists following the words “including” or “excluding” are not intended

to be exhaustive.

#### Minimum and/or Maximum Requirements

    - The provisions of this By-law must be held to be the minimum requirements, unless the word “maximum” is used, in which case the maximum requirement applies. If both a minimum and a maximum requirement are specified, then both provisions apply.

#### Intent and Rules of Interpretation

    - This By-law is remedial in nature and must be interpreted in a benevolent manner and read and applied in a way that will ensure the effective implementation of its provisions.

    - All provisions of this By-law that apply to a lot must be complied with. The intended hierarchy of provisions in Sections 1 through 24, is as follows (from the highest ranking to the lowest ranking):

- Sections 1, 2 and 3: Administration, Interpretation of this By-law and Definitions;

- Sections 6 and 22, and Schedule F: Holding Overlay and Holding Conditions;

- Sections 5, 20 and 21, and Schedules A to E: Overlay Provisions, Legacy Exceptions and Exceptions;

- Schedule 1: Zoning Map;

- Schedules 2, 3 and 4: Non-Overlay Schedules;

- Section 23: Temporary Zones;

- Sections 8 through 19: Provisions for Specific Zone Categories;

- Section 6: Specific Use Provisions;

- Section 4: General Provisions;

- Section 7: Parking, Loading and Bike Parking Provisions; and

- Section 24: List of Amendments to this By-law.

    - Despite Clause 2.1.8., where two or more provisions of this By-law are applicable, all provisions must be complied with or, where it is not possible to comply with all applicable provisions, the most restrictive provision must be complied with.

    - Where a use takes place outside of a building but a provision of this By-law regulates that use inside of a building, the provision applies as though the actual area occupied by the use is in a building.

    - Where a provision is tied to the use of a lot or building, such provision applies where any portion of the lot or building is used for such purpose.

    - Where a performance standard is applied to a use, such standard applies to the entire building in which the use is located.

    - Where an accessory use is specifically listed in connection with a principal use, the list of accessory uses is not intended to be exhaustive but has been included to provide clarity related to that specific accessory use only.

### Parts of This By-law

#### Items that are Part of This By-law

    - Schedules, Appendices, Figures, and text contained in clauses, subclauses, paragraphs, subparagraphs and items form part of this By-law.

    - Tables form part of this By-law and are used throughout to present permitted uses and provisions in a concise format and are structured with columns (vertical) and rows (horizontal) with titles.

#### Items that are not Part of This By-law

    - Titles, headings, subheadings, diagrams, footnotes, indices, table of contents, illustrations, and references to former enactments or enabling legislation do not form part of this By-law and are editorially inserted for convenience of reference only.

    - Notes that are included in square brackets, for example “[Note: See Diagram 3.19.9.]”, do not form part of this By-law and are editorially inserted for reference only.

    - Text included in electronic, interactive mapping of the Exception Overlay in Schedule E or the Holding Overlay in Schedule F is provided for convenience purposes only and does not form part of this By-law. The text that forms part of this By-law and corresponds with the applicable Exception Overlay and Holding Overlay mapping is included in Section 20, Section 21 and Section 22 of this By-law, as applicable.

### Structure of This By-law

#### System of Division

    - The system of division and numbering of the provisions of this By-law are as follows:

Section 1. - Title Subsection 1.1. – Heading Text - Subheading

Clause 1.1.1. – Text Subclause 1. - Text

Paragraph (a) Subparagraph (i)

Item (1)

    - This By-law is separated into Parts for the purpose of publishing the information in a format that is accessible and formatted to accommodate future amendments. Despite being separated into Parts, all Sections and Schedules referenced in Clause

1.1.1. form part of this By-law.

### Establishment of Zones

    - This By-law establishes the Zones listed in Table 2.4.1. and places all lands subject to this By-law in one or more of the Zones in accordance with Schedule 1.

#### Table 2.4.1. - List of Zones

| Zone Name | Zone Code |
| --- | --- |
| Prime Agricultural Area Zone | AG |
| General Rural Area Zone | RU |

| Zone Name | Zone Code |
| --- | --- |
| Rural Residential Zone | RUR |
| Limited Service Rural Residential Zone | LSR |
| Rural Commercial Zone | RC |
| Hamlet Zone | HAM |
| Rural Industrial Zone | RM1 |
| Rural Heavy Industrial Zone | RM2 |
| Mineral Resource and Extraction Zone | MX1 |
| Williamsville Zone 1 | WM1 |
| Williamsville Zone 2 | WM2 |
| Downtown Zone 1 | DT1 |
| Downtown Zone 2 | DT2 |
| Mixed Zone 1 | MU1 |
| Mixed Zone 2 | MU2 |
| Mixed Zone 3 | MU3 |
| Urban Residential Zone 1 | UR1 |
| Urban Residential Zone 2 | UR2 |
| Urban Residential Zone 3 | UR3 |
| Urban Residential Zone 4 | UR4 |
| Urban Residential Zone 5 | UR5 |
| Urban Residential Zone 6 | UR6 |
| Urban Residential Zone 7 | UR7 |
| Urban Residential Zone 8 | UR8 |
| Urban Residential Zone 9 | UR9 |
| Urban Residential Zone 10 | UR10 |
| Urban Residential Zone 11 | UR11 |
| Urban Residential Zone 12 | UR12 |
| Urban Residential Zone 13 | UR13 |
| Urban Multi-Residential Zone 1 | URM1 |
| Urban Multi-Residential Zone 2 | URM2 |

| Zone Name | Zone Code |
| --- | --- |
| Urban Multi-Residential Zone 3 | URM3 |
| Urban Multi-Residential Zone 4 | URM4 |
| Urban Multi-Residential Zone 5 | URM5 |
| Urban Multi-Residential Zone 6 | URM6 |
| Urban Multi-Residential Zone 7 | URM7 |
| Urban Multi-Residential Zone 8 | URM8 |
| Urban Multi-Residential Zone 9 | URM9 |
| Urban Multi-Residential Zone 10 | URM10 |
| Urban Multi-Residential Zone 11 | URM11 |
| Heritage Zone 1 – Village of Barriefield | HCD1 |
| Heritage Zone 2 – Market Square | HCD2 |
| Heritage Zone 3 – Old Sydenham | HCD3 |
| Institutional Minor Zone | IN1 |
| Institutional Major Zone | IN2 |
| Correctional Facility Zone | G1 |
| Military Installation Zone | G2 |
| Neighbourhood Commercial Zone | CN |
| Arterial Commercial Zone | CA |
| District Commercial Zone | CD |
| Regional Commercial Zone | CR |
| General Commercial Zone | CG |
| Marine Commercial Zone | CW |
| Harbour Zone | HB |
| Business Park Zone | M1 |
| General Industrial Zone | M2 |
| Heavy Industrial Zone | M3 |
| Employment Service Zone | M4 |
| Waste Management Zone | M5 |
| Airport Zone | TA |

| Zone Name | Zone Code |
| --- | --- |
| Transportation and Railway Zone | TR |
| Utility Installation or Corridor Zone | TU |
| Protected Open Space Zone | OS1 |
| General Open Space Zone | OS2 |
| Development Reserve Zone | DR |
| Environmental Protection Area Zone | EPA |

By-Law (2022-62; 2025-118)

### Zone Boundaries

    - Schedule 1, the Zoning Map, identifies different areas, called Zones, into which this By-law divides the City and shows the Zone codes given to these areas. The Zones may be cited by either their Zone code or their Zone name.

    - Where the boundary of any Zone shown on Schedule 1:

      - Follows a street, utility right-of-way, railway right-of-way, or watercourse, it is considered to follow the centre line of such street, utility right-of-way, railway right-of-way, or watercourse;

      - Substantially follows lot lines shown on Schedule 1 or the electronic consolidation of Schedule 1, it is considered to follow such lot lines;

      - Follows a street and, if the street is subsequently closed, the land in the closed street is considered to be included in the Zone of the adjoining land and if such street forms a boundary between two or more different Zones, it is considered to follow the centre line of that closed street; and

      - Passes through a lot and the distance is not indicated, it is considered to be located as measured using the scale of Schedule 1.

    - Where a lot is divided into more than one Zone, the Zone boundary is not treated as a lot line and each portion of the lot must be used in accordance with the provisions of the underlying Zone. In the case of a conflict, the more restrictive provision applies.

    - Despite Clause 2.5.3., where a Legacy Exception or Exception only applies to a portion of a lot, the boundary of such Legacy Exception or Exception is treated as a lot line for the purpose of zoning interpretation.

    - Where any lot or building is used for more than one purpose as provided in this By-law, the lot or building must comply with the provisions of this By-law relating to each use. In the case of a conflict, the more restrictive provision applies.

    - Despite Clause 2.5.1., all land within the municipal boundary of the City located under a waterbody is deemed to be zoned EPA where it meets the definition of waterbody.

(By-Law Number 2022-62; 2024-332)

### Zone Labels and Provisions

#### Subzones

    - Subzones are created by adding a period and upper case letter to the primary Zone code on the Zoning Map (for example: “UR3.B”), and have the effect of modifying the uses or the provisions of the primary Zone to the extent set out in the text of the provisions for that Subzone.

#### Holding Zones

    - Holding Overlays are created by identifying specific lands on Schedule D, Schedule F, Schedule G and Schedule J of this By-law and have the effect of restricting the development or use of a lot or building in accordance with the corresponding provisions until this By-law has been amended to remove the lot from the Holding Overlay in accordance with Section 36 of the Planning Act.

(By-Law Number 2022-62; 2024-332)

#### Temporary Zones

    - Temporary Zones are created by adding a hyphen and upper case "T" followed by a number (for example: "UR2-T1") to the Zone code on the Zoning Map, and have the effect of applying temporary use provisions pursuant to Section 39 of the Planning Act. Lands designated in this manner are subject to all provisions of the underlying Zone except as otherwise provided by the Temporary Zone provisions. Upon the expiry date of the Temporary Zone provisions, the temporary use of such lots or buildings, as may be specified, must cease, and use of the Temporary Zone symbol must be discontinued. The provisions of the corresponding Temporary Zone are included in Section 23 of this By-law.

#### Height Limit

    - The maximum permitted height is established in the underlying Zone, Subzone, Legacy Exception Overlay, Exception Overlay or provision, in metres and/or number of storeys. Where height is established in both metres and storeys, this By-law is interpreted to mean the lesser of the permitted height in both metres and storeys, unless otherwise indicated. Height may also be shown in terms of height in metres above sea level, indicated by the term ‘a.s.l.’ following the number in parenthesis.

#### Tables

    - The Tables in Sections 8 to 19 inclusive present the principal zoning provisions for permitted uses in each Zone. The permitted uses are listed, and the applicable provisions are provided in the rows associated with each permitted use. The columns provide the type of provision that is associated with the permitted use associated with each row.

#### Additional Provisions

    - Additional provisions are presented in conjunction with the permitted uses and zoning provisions that are presented in the Tables with each Zone. The references to an additional provision in the permitted uses and zoning provisions presented in the Tables are indicated with small numbers in superscript adjacent to the upper right of the large numbers in the Tables. In some cases where a number of additional provisions may apply, the Table may include a note directing the reader to the additional provisions below the Table.

# Section 3:	Definitions

## A

    - Accessible Space means a designated parking space identified by prescribed pavement markings and signage which is reserved for the exclusive use of persons with disabilities.

    - Accessible Aisle means a designated area immediately adjacent to one or two

accessible spaces, which provides access to and from the accessible spaces.

    - Accessory means subordinate and naturally, customarily and normally incidental to and exclusively devoted to a principal use or building, and located on the same lot.

      - Accessory House means an accessory building that is used for the purpose of one dwelling unit, which is accessory to a house, semi-detached house, or townhouse in the urban area.

(By-Law Number 2022-62; 2024-332)

    - Additional Rural Unit means a dwelling unit, which is accessory to a principal dwelling unit in the rural area, and is located on the same lot as the principal dwelling unit. An additional rural unit is either a second rural unit or a third rural unit.

(By-Law Number 2022-62; 2024-332)

    - Affordable Unit means:

For affordable rental housing, a dwelling unit intended for use as a rented residential premises for which:

- The rent is no greater than the lesser of:

  - the income-based affordable rent for the dwelling unit set out in the “Affordable Residential Units for the Purposes of the Development Charges Act, 1997”, as it is amended from time to time; or

  - the average market rent identified for the dwelling unit set out in the “Affordable Residential Units for the Purposes of the Development Charges Act, 1997”, as it is amended from time to time; and

- The tenant is dealing at arm’s length with the landlord; and

- The affordable unit is secured through an agreement registered on title of the property during the affordability term requiring:

  - An affordability term set for a minimum of 20 years; and

  - Following the initial occupancy, during the affordability term, the rent must not increase by more than the annual Residential Tenancies Act guideline increase.

For affordable ownership housing, a dwelling unit not intended for use as a rented residential premises for which:

- The price is no greater than the lesser of:

  - the income-based affordable purchase price for the dwelling unit (in the year in which the unit is sold) set out in the “Affordable Residential Units for the Purposes of the Development Charges Act, 1997”, as it is amended from time to time; or

  - 90% of the average purchase price identified for the dwelling unit (in the year in which the unit is sold) set out in the “Affordable Residential Units for the Purposes of the Development Charges Act, 1997”, as it is amended from time to time; and

- The tenant is dealing at arm’s length with the landlord.

(By-Law Number 2022-62; 2024-332)

    - Agricultural Sales Establishment means the use of any lot or building having as its principal use the storage and display of agricultural implements for sale, rent, or lease and may include facilities for the repair and maintenance of such implements as an accessory use.

    - Agricultural Use means the use of any lot or building for the growing of crops, including nursery, biomass, and horticultural crops; raising of livestock; raising of other animals for food, fur, or fibre, including poultry and fish; aquaculture; apiaries; agro-forestry; maple syrup production; and riding stables, including, but not limited to livestock facilities, manure storages, value-retaining facilities; or other agricultural activities such as the packaging, selling, sorting or storage of goods grown or raised on the lands.

(By-Law Number 2022-62; 2024-332)

    - Agriculture-Related Use means the use of any lot or building for farm-related commercial and/or farm-related industrial uses that are directly related to agricultural uses in the area, support agricultural uses, benefit from being in close proximity to agricultural uses and provide direct products and/or services to agricultural uses as a primary activity. Examples of agriculture-related uses may include, but are not limited to

- Storage and distribution centres serving farm operations in the area;

- Farmers markets primarily selling locally grown products;

- Processing of produce grown in the area (for example, cider-making, or the canning, quick-freezing and packing of food);

- Grain dryers;

- Agricultural research centres;

- Wineries using locally grown grapes;

- Abattoirs for processing and selling meat from animals raised in the area;

- Flour mills for locally grown grain;

- Farm equipment repair stores;

- Livestock assembly yards or stock yards;

- Agricultural auction grounds; and

- Farm supplier store (for example, feed, seeds, fertilizer).

    - Agri-tourism means the use of any lot or building which is complementary to a principal agricultural use on a lot and which provides education and active activities to experience and enjoy the agricultural way of life in the rural area. Such activities may include farm machinery and equipment exhibitions, farm-tours, petting zoos, corn mazes, hayrides, sleigh rides, processing demonstrations, pick your own produce/products, farm themed playgrounds, educational facilities that focus on farming instruction or other similar activities. Agri-tourism may include accessory retail sales, but excludes restaurants and all other uses that are considered under the broader on-farm diversified use definition.

    - Airport Facilities means all facilities related to aircraft landing and take-off, including runways, taxiways, aprons, hangars and navigational aids. Facilities may

also include terminals, maintenance, warehousing, manufacturing, distribution, aviation based research and development, training, communications, environmental reporting, aviation-related retail commercial, charter operations, air courier service, management or other related operations of the airport or airport-related tenants and may include automobile rental, retail stores, food concessions and restaurants as accessory uses.

    - Amenity Area means an area exterior to a residential building, or an interior area common to all dwelling units within a residential building, which is designed and intended primarily for the leisure and recreation of the occupants of the building.

    - Angular Plane means an imaginary upwardly inclined plane set at an angle to the horizontal, commencing either at intersection with a vertical plane, such as a build- to-plane, at a prescribed elevation above grade, which may coincide with a storey or a distance from grade, or at grade, which may coincide with a lot line.

    - Animal Care means the use of any lot or building for medical, grooming, training or similar services for animals, but does not include a kennel or an animal shelter.

    - Animal Shelter means the use of any lot or building for the care of lost, abandoned or neglected animals and operated by a public authority or semi-public authority or by a not-for-profit organization.

    - Apartment Building means a multi-unit residential building that is not otherwise defined in this By-law. Apartment building also includes development with multiple multi-unit residential buildings on one lot, such as multiple townhouses or stacked townhouses when they are located on one lot.

(By-Law Number 2022-62; 2024-332)

      - Arm’s Length has the same meaning as arm’s length in section 251 of the Income Tax Act (Canada) with necessary modifications.;

(By-Law Number 2022-62; 2024-332)

    - Attic means uninhabitable space between the roof and the ceiling of the top storey

or between a partial wall and a sloping roof.

    - Automobile Body Shop means the use of any lot or building for the painting and/or repairing of the exterior and/or the undercarriage of motor vehicle bodies, but does not include a salvage yard.

    - Automobile Repair Shop means the use of any lot or building for the servicing and repair of motor vehicles that may also be operated in conjunction with a

towing service and other similar uses, including the sale of the required components.

    - Automobile Sales Establishment means the use of any lot or building for the display and sale of new or used motor vehicles and may include accessory uses, including outdoor storage for the display and sale of motor vehicles, the servicing and repair of motor vehicles, an automobile body shop, the sale of motor vehicle parts and products and the leasing or renting of motor vehicles.

(By-Law Number 2022-62; 2024-332)

## B

    - Bachelor Dwelling Unit means a dwelling unit within which the common areas and areas intended for sleeping are combined into a single room. For the purpose of this By-law, a bachelor dwelling unit is deemed to contain one bedroom.

    - Backyard Hen means a female chicken that is at least four months old and is subject to a hen coop permit issued in accordance with the Animal Control By-law.

    - Backyard Hen Coop means an accessory building where backyard hens are kept and which is constructed in accordance with the Animal Control By-law and contains lockable roofs and doors.

    - Backyard Hen Run means a secure building attached to a backyard hen coop that allows backyard hens to access outside.

    - Balcony means an unenclosed or partially enclosed platform that is attached to and only directly accessible from within a building and is located above the first storey. A balcony includes associated guards, fencing, walls, visual screen and other associated features.

    - Banquet Hall means the use of any lot or building in which facilities are provided for the gathering of people for the purpose of catering to banquets, weddings, receptions or similar functions for which food and beverages are prepared and served, but excludes a restaurant.

    - Basement means that portion of a building where the ceiling is less than 1.0 metre above finished grade.

    - Basement Storey means a storey that is located below the first storey.

    - Bay Window means a window that projects outward from an exterior wall of a

building but does not include any gross floor area of the dwelling unit.

    - Bedroom means any room in a dwelling unit that is not:

- A common area, being:

  - A living room open to all occupants of the unit; or

  - A dining room open to all occupants of the unit;

- An area used for sanitary purposes, such as a washroom;

- An area used for cooking purposes, such as a kitchen;

- An area occupied solely by mechanical equipment, such as furnaces, hot water heaters, or laundry equipment;

- A circulation space, such as a stairway or hallway;

- A room less than 6 square metres in area where there are built-in cabinets and/or closets;

- A room less than 7 square metres in area where there are no built-in cabinets and/or closets; or

- A room without a window or similar alternative source of natural light when the door is closed.

    - Bike Space means an unobstructed area that is designed to be used for the parking or storage of one bicycle.

    - Block means an area of land comprised of one or more lots that is bounded on all sides by a street.

    - Building means anything that is comprised of components joined together and that stands more or less permanently in one place. A building includes all components such as walls, roof, floors, structural systems, columns, plumbing, fixtures, service systems, private sewage systems, decks, porches, canopies, architectural features, chimneys, mechanical systems and any component that is attached to a building. The following are considered to be buildings:

1.	A shipping container, sea can or similar storage container when placed on the ground for any purpose other than loading and unloading a shipment in

conjunction with an industrial or commercial use for up to maximum of 28 consecutive calendar days.

(By-Law Number 2022-62; 2024-332)

    - Building Depth means the horizontal distance between the required front setback and the principal building’s rear wall, measured perpendicular to the front setback.

    - Building Frontage means the building façade that fronts on a street line where access to the building is available.

    - Building Supply Store means the use of any lot or building for the retail sale or rental of equipment, construction supplies and accessories, including outdoor storage of building materials, which may include: lumber; millwork; cement; siding; roofing; plumbing or electrical supplies; heating, cooling or ventilation supplies; fireplaces; windows; paints; wall coverings; and floor coverings.

    - Build-to-Plane means a vertical plane which runs parallel to, and at a specified distance from, a street line. The location of the build-to-plane is established by measuring from, and perpendicular to, the street line to the nearest part of any main wall of any building on a lot.

    - Bunkhouse means a detached accessory building that is designed to provide seasonal sleeping accommodations and which may contain a washroom, but does not contain a kitchen.

## C

    - Cabin means a building intended only for seasonal accommodation and which may contain a washroom or a kitchen or both.

    - Campground means the use of any lot or building for providing overnight or short term accommodation for recreational vehicles, travel trailers, motor homes, and tents, but not a mobile home, and includes accessory services and facilities normally incidental and subordinate to such a use including common washroom lavatory and bathing facilities, indoor and/or outdoor recreation areas, picnic areas and an entrance kiosk.

    - Call Centre means the use of any lot or building established to transmit or receive a high volume of phone calls to provide technical support, customer service, sales or similar client services.

    - Canopy means a roof-like architectural feature projecting more than 0.3 metres from the exterior face of a building.

    - Car-share means the practice where a number of people share the use of one or more cars that are owned by a car-sharing organization and where such organization may require that the use of cars be reserved in advance, charge fees based on time and/or kilometers driven, and set membership requirements of the car-sharing organization.

    - Car-share Space means a parking space that is reserved for the exclusive use of a

#### car-share vehicle.

    - Carwash means the use of any lot or building for the washing of one or more

vehicles at any one time.

    - Casino Gaming Facility means the use of any lot or building for the purpose of providing slot machine or table game wagering or betting, authorized and licensed by the Province of Ontario.

    - Cataraqui Source Protection Plan means the Cataraqui Source Protection Plan developed under the Clean Water Act, 2006, S.O. 2006, c.22, or any successor documents that implement the source protection plan requirements of the Clean Water Act.

    - Catering Service means the use of any lot or building for the preparation of meals in full or in part for consumption at a location other than the premises in which the meal is prepared.

    - Cemetery means the use of any lot or building, or part thereof for the interment of human remains and which may include a crematorium, mausoleum, or columbarium as licensed under the Funeral, Burial and Cremation Services Act, 2002, S.O. 2002, c. 33.

    - Centre Line means an imaginary line which equally divides the width of a right-of- way allowance, including a street.

    - Chief Building Official means the municipal official appointed pursuant to the Building Code Act, 1992, S.O. 1992, c. 23 and includes their authorized representatives.

    - City means the Corporation of the City of Kingston.

    - Classroom means a room primarily used for scheduled teaching purposes which does not require special equipment of a kind that makes the room unsuitable for general instruction (for example laboratory, benches).

    - Club means the use of any lot or building by clubs, groups, or organizations for the purposes of providing for meeting places, social functions, and regular membership gatherings.

    - Commercial Motor Vehicle means any vehicle which displays commercial lettering or commercial licence plates and also includes construction equipment, which is

self-propelled or designed to be towed, a taxi, a delivery vehicle, and a driving school vehicle, in addition to any “commercial motor vehicle” as defined under the Highway Traffic Act, R.S.O. 1990, c. H.8.

    - Commercial Parking Lot means the principal use of any lot or building for the parking of motor vehicles, with or without a fee being charged. Commercial parking lot includes drive aisles, parking spaces and components necessary to support the use, and excludes any area where motor vehicles for sale or repair are kept or stored.

    - Common Element means a lot or building forming part of the common elements of a Plan of Condominium pursuant to the Condominium Act, 1998. Common elements may include private roads, common walkways, common sidewalks and common amenity areas within a Plan of Condominium.

    - Community Garden means the use of any lot or building managed and maintained by a group of individuals for the purpose of cultivation of plants for personal use or consumption and includes other forms of urban agriculture such as tree planting projects or similar uses. Permitted accessory uses include the sale or donation of produce grown in the community garden.

    - Community Centre means the use of any lot or building where members of the public are provided with health services, social support services, cultural, social or recreational programs or life/work skills training programs and where neither overnight care nor living accommodation is available. A day care centre is a permitted accessory use to a community centre.

    - Complementary Use means the use of any lot or building which is separate from the principal use of the lot, the principal use of an adjacent lot or a lot within the same Zone or Zone category and is not subordinate or incidental to such use, but is a use which provides support and services to the principal use on the subject lot, adjacent lot or lot within the same Zone or Zone category.

    - Conference Centre means the use of any lot or building where the primary focus is to host large gatherings for specific events such as conferences, conventions, exhibitions, seminars, workshops, rallies, trade shows, specific day retail sales and may include banquet halls, meeting rooms, restaurants, theatres or a hotel where they support the primary focus of hosting large gatherings.

    - Conservation Use means the use of any lot for the protection of natural heritage features for the purpose of long-term protection of the natural heritage resource.

    - Construct means to do anything in the erection, installation, extension or alteration or repair of a building and includes the installation of a building unit fabricated or moved from elsewhere.

    - Contractor’s Yard means the use of any lot or building by a construction company, landscaper, or contractor for the storage and maintenance of equipment and materials used or rented by the contractor or company, and includes facilities for the administration or management of the business and the stockpiling or storage of supplies used by the business.

    - Convert means to change the use of an existing lot or building, or a part thereof, to another use.

    - Co-Living Unit means the use of a building intended for residential accommodation where private bedrooms and/or living spaces share the use of one common kitchen and may share common washroom facilities or living spaces.

    - Cornice means an exterior horizontal projection or ornamental moulding along the top of a building, wall, arch or column.

    - Corner Lot means a lot situated at the intersection of and abutting two or more streets, or parts of the same street, where the inside angle of intersection or projected angle at the intersection of the tangents of the street lines is less than 135 degrees, except lots at the start of cul-de-sacs where the angle may exceed 135 degrees. [Note: See Diagram 3.12.17.]

    - Correctional College means the use of any lot or building for the training of correctional service staff.

    - Correctional Institution means the use of any lot or building for a correctional institution as defined by the Ministry of Correctional Services Act, R.S.O. 1990, c. M.22.

    - Council means the Municipal Council of the Corporation of the City of Kingston.

    - Creativity Centre means the use of any lot or building as the workplace of a photographer, artist, craftsperson or any other similar creative field, and includes galleries for the display of art for viewing and purchase, and any accessory instruction facilities where such creative field is taught.

## D

    - Day Care Centre means the use of any lot or building licensed pursuant to the Child Care and Early Years Act, 2014, S.O. 2014, c.11, Sched. 1, for the purpose of providing temporary care for or supervision of children for a child’s safety, well- being or development, in the absence of the child’s parent or guardian and for a continuous period that does not exceed 24 hours. A day care centre may also include care offered or supplied on a regular schedule to adults for a portion of a day, but which does not provide overnight accommodation.

    - Deck means a building component that is an uncovered and unenclosed or partially enclosed platform, which may or may not be attached to one or more walls of a building. A deck includes its associated guards, fencing, walls, visual screens, stairs and other associated features. A deck may or may not have a foundation.

    - Density means the ratio of the number of bedrooms, dwelling units or co-living units to one net developable hectare of lot area or per lot.

    - Department Store means the use of any lot or building that sells several lines of merchandise, which may include two or more of the following lines of merchandise, where no one particular line comprises the primary offering: apparel, jewelry, cosmetics and toiletries, furniture, home furnishings, appliances, electronics, sporting goods, toys, photo equipment, hardware, auto accessories, and other household goods and services. Accessory uses to a department store include, but are not limited to, a garden centre.

    - Develop or Development means to:

- Create a new lot;

- Create a new use;

- Convert to a different use;

- Alter landscaped open space, a landscaped berm, a planting strip or any other landscaping feature required pursuant to this By-law;

- Construct driveways, drive aisles, parking spaces, parking lots or loading spaces on a lot; and

- Alter, enlarge, erect, build, construct, reconstruct, relocate, renovate or restore

buildings or parts thereof;

Develop or development does not include:

- Activities that create or maintain infrastructure authorized under an environmental assessment process;

- Works subject to the Drainage Act, R.S.O. 1990, c. D.17; or,

- Underground or surface mining of minerals or advanced exploration on mining lands in significant areas of mineral potential where advanced exploration has the same meaning as under the Mining Act, R.S.O. 1990, c. M.14.

    - Director means the City’s Director, Planning Services, or their designate, or in the event of organizational changes, the Director of the appropriately titled department.

    - Ditch means a small to moderate excavation created to channel water.

    - Dormer means a roofed building component, often containing a window, that pierces through the plane of a sloping roof.

    - Drive Aisle means an internal roadway immediately adjacent to a parking space in a parking lot which provides vehicular access to 2 or more parking spaces, except for tandem parking spaces, and is not a driveway.

    - Drive-Through means a building component that provides or dispenses products or services through an attendant or a window or an automated machine to patrons remaining in motor vehicles, including associated stacking lane, speaker system, microphone system, signage, order board or other similar facilities, but does not include a service station or carwash.

    - Driveway means a defined area providing access for motor vehicles from a street to facilities such as a parking lot, commercial parking lot, parking space, loading space, private garage, but excludes a drive aisle.

    - Reserved

(By-Law Number 2022-62; 2024-332)

    - Dwelling Unit means the use of a building, comprised of one or more habitable rooms designed to provide at least one washroom and kitchen for residential accommodation. This definition may include a short term rental as defined in the Short Term Rental By-law where it is comprised of one or more habitable rooms designed to provide at least one washroom and kitchen for residential accommodation. This definition excludes bunkhouses, recreational vehicles, travel trailers, tent trailers or motor homes.

## E

    - Effective Ratio means a theoretical ratio that is established for a given use or building, which is used as the basis for calculating the required number of accessible spaces to be provided on a lot.

    - Elementary School means the use of any lot or building for academic instruction typically offered from kindergarten to grade 8 including public, private or separate schools, but does not include a training facility. Permitted accessory uses include day care centres.

    - Electric Vehicle means a motor vehicle that uses electricity for propulsion, and that

can use an external source of electricity to charge the vehicle’s batteries.

    - Electric Vehicle Ready means a parking space designed and constructed to be ready for the future installation of electric vehicle supply equipment through the installation of conduits that enable the installation of electrical components in the future or through other similar means.

    - Electric Vehicle Supply Equipment means a complete assembly consisting of conductors, connectors, devices, apparatus, and fittings installed specifically for the purpose of power transfer and information exchange between a branch electric circuit and an electric vehicle.

    - End-of-Trip Bike Facility means a dedicated area in a non-residential building

where showers, clothing lockers and private change rooms are provided for cyclists.

    - Entertainment Establishment means the use of any lot or building for the provision of entertainment or amusement without the necessity of active participation by the user and includes such uses as an arena, movie theatre, cinema, concert hall, playhouse, arcade and bingo, or similar use when the principal focus of the use is spectating for entertainment.

    - Equipment Rental means the use of any lot or building providing machinery, equipment and tools for rent at a limited period of time, mainly to construction contractors but also to industry and individual consumers.

    - Exterior Lot Line means the lot line, other than a front lot line, of a corner lot

which abuts a street. [Note: See Diagram 3.12.17.]

    - Exterior Setback means the setback between the exterior lot line and the nearest part of any building on the lot. [Note: See Diagram 3.19.8.]

    - Exterior Yard means a yard extending from the front yard to the rear yard

between the exterior lot line and a building on the lot. [Note: See Diagram 3.19.8.]

## F

    - Factory Outlet means the use of any lot or building as an accessory to a manufacturing use offering goods for sale which are manufactured entirely on the same lot as the factory outlet.

    - Fairgrounds means the use of any lot or building where fairs, circuses or exhibitions are held primarily outdoors, and includes any accessory and temporary buildings.

    - Feedmill means the use of any lot or building as a commercial or industrial scale mill for the processing, blending, grinding and mixing of grains, seeds and concentrates.

    - Fence means a physical barrier or partition made of wood, metal or other substance that is constructed for any purpose, such as marking the boundary of a property, enclosing a property, providing privacy, preventing access by people or animals, or dividing a property into sections, and includes every door, gate and other closure that forms part of a fence, and which may be regulated by the City of Kingston By- law Number 2003-405, A By-law to Regulate Fences, or any successor legislation.

    - Ferry Terminal means the use of any lot or building for the docking of ferry boats which may carry persons, cargo, motor vehicles or other vehicles across a waterbody, including accessory administrative offices, food concessions, retail stores or other similar uses.

    - Financial Institution means the use of any lot or building wherein money management services are provided and includes a bank, trust company, credit

union, financial company, mortgage company, loan company, cheque cashing company, or investment company.

    - Finished Grade means the average elevation of the ground surface at the base of the main wall, measured at the four most distant points representing the outermost corners of the building.

When used in reference to a round building or another building that does not have corners, means the average elevation of the ground surface at the base of the building, measured around the perimeter.

    - First Storey means the storey with its floor closest to finished grade and which has a floor level that is located at or above finished grade.

    - Fitness Centre means the use of any lot or building in which facilities are provided for fitness or athletic activities such as body-building, endurance training, yoga, exercise and fitness classes, or other similar uses where the principal focus is fitness. Fitness centres may include associated facilities such as a sauna, a swimming pool and a solarium and accessory uses such as a food concession and retail store.

    - Flat Roof means a roof where at least 50% of the area of the roof, when viewed from a horizontal plane, has a pitch less than 10 degrees from the horizontal.

    - Floodplain means the area, usually lowlands, adjoining a waterbody and which has been or may be subject to flooding hazards as defined by the Cataraqui Region Conservation Authority and as identified as “Floodplain” on Schedule A.

    - Floor Space Index means the gross floor area of all buildings on a lot divided by the lot area. “FSI” means floor space index.

    - Food Truck means the use of any lot for a vehicle, whether motorized or manually propelled, from which refreshments and/or food are sold for public consumption, including carts, wagons, trailers and motor vehicles.

    - Forestry Use means the use of any lot or building for raising and harvesting of trees, including the raising and cutting of fuel wood, pulp, wood, lumber, Christmas trees and other products, but excludes a garden centre or building product outlet.

    - Former Zoning By-law means:

- By-law Number 3078 of the former City of Kingston, as amended;

- By-law Number 9087A of the former City of Kingston, as amended;

- By-law Number 8950 of the former City of Kingston, as amended;

- By-law Number 541 of the former Township of Kingston, as amended;

- By-law Number 3077 of the former City of Kingston, as amended;

- By-law Number 8499 (“8499”) of the former City of Kingston, as amended;

- By-law Number 96-259 (“96-259”) of the former City of Kingston, as amended;

- By-law Number 76-26 (“76-26”) of the former Township of Kingston, as amended;

- By-law Number 97-102 (“97-259”) of the former Township of Kingston, as amended; and

- By-law Number 32-74 (“32-74”) of the former Township of Pittsburgh, as amended.

    - Freehold means a lot or building where the ownership does not include any

#### common element.

    - Front Lot Line means, in the case of an interior lot, the line dividing the lot from the street. Where an interior lot includes more than one lot line separating the lot from the street, including straight and/or curved portions of a lot line, all lot lines separating the lot from the street are the front lot line.

In the case of a corner lot, the shorter lot line abutting a street is deemed the front lot line and the longer lot line abutting a street is deemed an exterior lot line. Where a corner lot includes a front lot line and exterior lot line that do not intersect at one point (for example, a rounded or chamfered area), the front lot line must be measured using the hypothetical point of intersection of the extension of the front lot line and the extension of the exterior lot line. Any portions of lot lines that form part of an irregular corner, such as rounded or chamfered lot lines, are not considered front lot lines or exterior lot lines.

In the case of a corner lot which is also a through lot, the front lot line is the lot line opposite to a lot line that does not abut a street.

In the case of a through lot or a corner lot whose street lines are the same length, the lot line where the principal vehicular access to the lot is provided is deemed to be the front lot line.

In the case of a waterfront lot with no street line, the front lot line is the lot line contiguous with the waterbody. In the case of a waterfront lot with a street line, the front lot line is the street line.

In the case of one lot that occupies an entire block, every lot line is a front lot line.

Where an existing lot does not have a lot line which is also a street line, the lot line which is closest to and most parallel to the nearest street line is considered the front lot line. [Note: See Diagram 3.12.17.]

    - Front Setback means the setback between the front lot line and the nearest part of any building on the lot.

Where a corner lot includes a front lot line and exterior lot line that do not intersect at one point, the front setback must be determined by measuring the hypothetical point of intersection of the extension of the front lot line and the extension of the exterior lot line. [Note: See Diagram 3.19.8.]

    - Front Yard means a yard extending across the full width of the lot between the front lot line and a principal building on the lot. Where the main wall facing the front lot line is not parallel to the front lot line, the front yard is projected as a linear extension of the main wall to the lot lines. [Note: See Diagram 3.19.8.]

(By-Law Number 2022-62; 2024-332)

    - Funeral Establishment means the use of any lot or building established for the purpose of temporarily placing human remains and cremated human remains, so that persons may attend and pay their respects.

## G

    - Garage Sale means the use of any lot or building for the sale of household goods by an occupant of a dwelling unit, on the same lot as the dwelling unit.

    - Garden Centre means the use of any lot or building for the retail sale of trees, shrubs or plants, and which may include the accessory sale of soil, planting materials, fertilizers, lawn and garden tools, ornaments, and similar materials.

    - Gasoline Pump means a building for the dispensing of vehicle fuels including gasoline, diesel, propane and natural gas but does not include a kiosk or any other accessory building.

    - Reserved

(By-Law Number 2022-62; 2024-332)

    - Golf Course means the use of any lot or building for the purpose of playing golf and may include accessory uses such as a restaurant, food concession, driving range, the sale or rental of golf equipment, or a banquet hall.

    - Gravel Pit means the use of any lot or building for open excavation made for the removal of any soil, earth, clay, marl, sand, gravel or unconsolidated rock or mineral in order to supply material for construction, manufacturing or industrial purposes, but excludes an excavation incidental to the development of a building for which a building permit has been granted by the City, or an excavation incidental to the construction of any public works. This definition includes a wayside pit or wayside quarry.

    - Grocery Store means the use of any lot or building devoted to the sale of perishable and non-perishable food including baked goods, fruits and vegetables, meat or butcher products and dairy products and may include an accessory food concession, delicatessen and retail store.

    - Gross Floor Area means the total floor area of all floors of a building above finished grade, measured between the outside of the exterior walls or between the outside of exterior walls and the centre line of party walls dividing the building from another building, but excluding:

- Areas of enclosed malls used as a common area between stores;

- Areas used for mechanical equipment, electrical equipment or similar service areas such as garbage or recycling rooms;

- Areas used for stairways and elevator shafts;

- Areas used as storage lockers;

- Areas used for loading spaces, bike spaces and parking spaces;

- Any floor area with a floor to ceiling height of less than 1.8 metres;

- Area in an attic having headroom of 2.1 metres or less for at least half the

attic floor area, unless otherwise specified; and

- Balconies, porches, decks and mezzanines.

    - Gross Leasable Area means the total area of all floors in a building or part of a building usable for tenant occupancy and the tenants' exclusive use, including basements, mezzanines and integral storage areas, measured from the centre line of joint partitions and from outside wall faces, but not including public or common areas, such as parking spaces and parking lots, walls, corridors, stairways, elevators or machine or equipment rooms.

    - Group Home means the use of a lot or building to provide supervised living accommodation as per the requirements of its residents, licensed and/or funded by the Province of Ontario or the Government of Canada, generally limited to 10 persons or fewer, exclusive of staff, living together as a single housekeeping unit. For the purpose of this By-law, a group home is considered to be a dwelling unit.

## H

    - Habitable Room means any room in a dwelling unit that is capable of being used by one or more persons for living, sleeping, eating, food preparation or sanitation and includes a bedroom.

    - Habitation Unit means the use of a residential building or mixed use building associated with a post-secondary institution intended for residential accommodation by one person. A habitation unit may be located within a co- living unit or dwelling unit.

    - Heavy Equipment or Truck Repair Shop the use of a lot or building for the repair or servicing of heavy equipment or trucks and may include accessory uses such as wash facilities and driver services.

    - Heavy Industrial Use the use of a lot or building for:

- Production, manufacture, fabrication, assembly, or similar processes, where the process(es) have an elevated probability of fugitive emissions such as noise, odour, dust or vibration, including where one or more process(es) occurs outside of a building; and/or

- Production or use or storage of flammable, explosive or other hazardous materials.

    - Height, when used in reference to an accessory building, mixed use building, apartment building or a building with non-residential uses, means the vertical distance measured from finished grade to the highest point of the building.

When used in reference to any feature that is not a building, means the vertical distance from the ground surface at the base of the feature to the highest point of the feature.

When used in reference to the first storey, means the vertical distance measured from finished grade to the top of the ceiling of the first storey.

For principal buildings with a residential use, excluding a mixed use building or

apartment building, means the vertical distance from finished grade to:

- In the case of a building with a flat roof, the highest point of the building;

- In the case of a sloped roof, the average level between the eaves and highest point of the building; and

- For all other roof types, including a quonset hut or a dome, the highest point of the building.

- For the purpose of this definition, a roof with a slope 60 degrees or more to the horizontal and which is adjacent to occupied portions of a building is considered to be a main wall. Where the soffit projects more than 0.5 metres from the main wall on the horizontal, the slope is calculated at a point that is

0.5 metres from the main wall.

#### Diagram 3.8.5. – Roof with Slope of 60 Degrees or More

    - Heritage Building means a building that is designated under the Ontario Heritage Act, R.S.O. 1990, c. O.18, or that is certified to be of significant architectural or historical value by a recognized, non-profit public organization whose primary object is the preservation of structures of architectural or historical significance and the certification has been accepted by the Chief Building Official.

    - High Water Mark means the highest water level that has been maintained for a sufficient duration (on an annual basis) as to leave physical evidence upon the landscape marking the boundary between that water level and upland areas. The boundary may be identified by:

- An examination of the bed and bank of the waterbody, to determine where the presence and action of water has been so common and usual and long continued in all ordinary years to mark upon the bed or bank a character distinct from that of the abutting upland; and/or

- A distinction between either open water or dominant aquatic/wetland vegetation, and dominant upland vegetation.

    - Home Day Care means the use of a portion of a dwelling unit for the purpose of providing temporary care for or supervision of children for a child’s safety, well- being or development, in the absence of the child’s parent, for a continuous period

that does not exceed 24 hours, and in accordance with the Child Care and Early Years Act.

    - Home Office means the use of a portion of a dwelling unit, including an attached private garage or a detached accessory building located on the same lot as the dwelling unit, as an office space for a person residing in the dwelling unit where no customers, clients or in-person meetings are conducted on the lot. Home office includes remote-working and work from home arrangements where business is conducted virtually or over the phone.

    - Home Occupation means a use of a portion of a dwelling unit, including an attached private garage or a detached accessory building located on the same lot as the dwelling unit, as an occupation, business, trade, home day care or craft for a person residing in the dwelling unit that is subordinate to the principal use of the dwelling unit. Home occupation excludes a home office.

    - Horizontal Bike Space means a bike space that is provided in a horizontal format, excluding a stacked bike space.

    - Hospital means the use of any lot or building that is established for the purposes of the treatment of patients and that is approved as a public hospital under the Public Hospitals Act, R.S.O. 1990, c. P.40 and may include a gift shop, cafeteria and other similar accessory uses normally associated with a hospital.

    - Hotel means the use of any lot or building for the temporary lodging of the travelling public or for recreation purposes and may include a banquet hall, meeting rooms, licensed lounge, restaurant, convenience store and gift shop as accessory uses, but excludes any short term rental as defined in the Short Term Rental By-law.

    - House means a ground oriented residential building that is used for the purpose one or more principal dwelling units on one lot. This includes, but is not limited to, a house, rowhouse, tiny house, mobile home and other similar housing types. House also includes buildings that are appear similar to a semi-detached house and townhouse, when the entire building is located on one lot.

(By-Law Number 2022-62; 2024-332)

## I

    - Industrial Repair Shop means the use of any lot or building for the repair of industrial articles through the use of machining, welding or fabrication.

    - Institutional Use means the use of any lot or building by any organization, group, or association for government, religious, charitable, educational, benevolent, health or welfare purposes and not for profit or gain, but does not include uses otherwise defined herein.

    - Intake Protection Zone means the area of land and water that contributes source water to a drinking water system intake within a specified distance, period of flow time (for example, two hours), and/or watershed area.

    - Interior Lot means a lot other than a corner lot. [Note: See Diagram 3.12.17.]

    - Interior Lot Line means a lot line, other than a rear lot line that does not abut a

street. [Note: See Diagram 3.12.17.]

    - Interior Setback means the setback between the interior lot line and the nearest part of any building on the lot. [Note: See Diagram 3.19.8.]

    - Interior Yard means a yard extending from the front yard to the rear yard

between the interior lot line and a building on the lot. [Note: See Diagram 3.19.8.]

## J

    - Reserved

## K

    - Kennel means the principal use of any lot or building where the predominant economic activity consists of day boarding, overnight boarding or breeding of domestic household pets.

## L

    - Laboratory means the use of any lot or building where experiments, tests or investigations are conducted and/or where drugs, chemicals, or other substances or articles pertinent to such experiments, tests or investigations are manufactured or otherwise prepared for use on the lot. This definition excludes a research establishment.

    - Landscaped Berm means an outdoor area on a lot that has been designed for safety purposes as an earthen berm with side slopes not steeper than 2.5 to 1, adjoining and parallel to a railway right-of-way with returns at the ends.

    - Landscaped Open Space means an outdoor area on a lot that is used for:

- Soft landscaping that allows water to permeate into the ground (vegetation such as trees, shrubs, hedges, ornamental plantings, grass, groundcover or other similar plantings);

- Hard landscaping (brick, gravel, pavers, rocks, stones, walkways, fences or

#### outdoor patios);

- Exterior stairs;

- Porches without a perimeter foundation;

- Decks without a perimeter foundation;

- Swimming pools; and

- An area above a private sewage system. Landscaped Open Space excludes:

  - Driveways, drive aisles, parking spaces, parking lots, loading spaces or anywhere a vehicle is parked or driven;

  - Decks or porches that have a perimeter foundation; and

  - Any area beneath, above or within any building (excluding a private sewage system).

    - Laundry Store means the use of any lot or building for the purpose of receiving articles or goods of fabric to be subjected to the process of dry cleaning, dry dyeing or cleaning elsewhere and for the pressing and/or distribution of any such articles or goods which have been subjected to any such process. A laundry store includes a laundromat, where one or more clothes washing and drying machines are used.

    - Library means the use of any lot or building for the collection of literary, artistic, musical and similar reference materials in the form of books, manuscripts, recordings and films for the purposes of study, reference and recreation.

    - Light Industrial Use means the use of any lot or building for production, processing, manufacturing, fabrication, assembly, or similar processes, including those that are technologically advanced or innovative, within a self-contained building where the processes have a low probability of fugitive emissions such as

noise, odour, dust or vibration. This definition does not include any other use

defined herein.

    - Livestock means beef cattle, birds, dairy cattle, deer and elk, fur-bearing animals, game animals, goats, horses, poultry, ratites, sheep, swine and other animals as identified in minimum distance separation, but excludes backyard hens.

    - Livestock Barns means one or more permanent buildings located on a lot which are intended for housing livestock, and are structurally sound and reasonably capable of housing livestock.

    - Livestock Facility means the use of any lot or building for livestock barns and manure storage, including all unoccupied livestock barns and unused manure storage.

    - Loading Space means an unobstructed open, enclosed or partially enclosed area for the purpose of loading or unloading vehicles in conjunction with a permitted use or building on the same lot.

    - Long-term Bike Space means a bike space that is intended for use of an occupant of a dwelling unit or employee of a business.

    - Lot means a single parcel, tract of land or parcel of tied land, in each case that may be conveyed in compliance with the provisions of the Planning Act or the Condominium Act, 1998, excluding a unit, as that term is defined in the Condominium Act, 1998, or a common element. Where a lot is divided in a stratified manner to allow for separate ownership of different levels of a building, it is still considered to be one lot for the purpose of this By-law, with the lot lines that exist at the level of the ground being the applicable boundaries for zoning interpretation purposes.

(By-Law Number 2022-62; 2024-332)

    - Lot Area means the total surface area taken on a horizontal plane within all lot lines

of a lot, excluding:

- The area below the high water mark of a waterbody;

- Any lands which may be subject to the Floodplain Overlay;

- Any lands which have been or which will be dedicated to the City for public streets, public sidewalks, public open space, parks or public community facilities including, but not limited to libraries, fire stations, and recreation facilities; and

- Any lands zoned Environmental Protection Area.

    - Lot Coverage means the percentage of the lot area covered by:

- A building;

- A deck that has a perimeter foundation; and

- A porch that has a perimeter foundation.

Lot Coverage excludes private sewage systems, uncovered steps, patios, swimming pools, decks, porches, balconies and bay windows as well as canopies and overhanging eaves which are 2.0 metres or more in height above the finished grade.

    - Lot Depth means the horizontal distance between the front lot line and rear lot line perpendicular to the front lot line. If the front lot line and rear lot line are not parallel, lot depth means the length of a straight line joining the midpoint of the front lot line with the midpoint of the rear lot line. If there is no rear lot line, lot depth means the length of a straight line joining the midpoint of the front lot line with intersection of the interior lot lines and/or exterior lot line.

    - Lot Frontage means the linear distance measured between the two points of intersection of the interior lot lines and/or exterior lot lines with the front lot line.

In the case of a pie-shaped or an irregularly shaped lot where the lot narrows at the street line, lot frontage is the linear distance measured between two points on the interior lot lines and/or exterior lot lines at a linear distance that is 6 metres from the front lot line measured along the interior lot lines and/or exterior lot lines.

Where a corner lot includes a front lot line and interior lot line and/or exterior lot line that do not intersect at one point, the interior lot line and/or exterior lot line is deemed to extend to its hypothetical point of intersection with the extension of the front lot line.;

(By-Law Number 2022-62; 2024-332)

    - Lot Line means a line delineating any legal boundary of a lot.

#### Diagram 3.12.17. – Lot Types and Lot Lines

    - Deleted

(By-Law Number 2022-62; 2024-332)

## M

    - Main Wall means all portions of the exterior front, side and/or rear wall of a building and all structural components of the exterior walls essential to support the roof.

(By-Law Number 2022-62; 2024-332)

    - Marina means the use of any lot or building as a commercial venture providing transient and/or seasonal docking and mooring facilities where boats are berthed, stored, serviced, repaired or launched. A marina may include a yacht club, land based facilities for the winter storage of boats, boat servicing establishments, sale or rental of boats and boat accessories, the sale of marine fuels and lubricants, waste water pumping facilities, washroom and shower facilities, laundry store, boat launching ramp, boat lift/boat lifting equipment, administrative offices and marine-

related instructional facilities. Accessory uses may include a restaurant or a hotel

that supports the marina.

    - Marine Facility means an accessory building that is used for the purpose of taking a boat into or out of a waterbody, or to moor, berth or store a boat and which abuts a shoreline. This definition includes vertical storage for non-motorized boats, a boat slip, boat launch ramp, boat lift, boat port, dock or marine railway and areas with enclosed walls and a roof such as a boathouse. Marine facility excludes any building used as a residential use, a general living area or sleeping accommodation.

    - Mechanical Penthouse means an enclosed or partially enclosed building component which houses mechanical, ventilation, electrical or other similar equipment only, other than a bulkhead, sited on the roof of a building.

    - Military Installation means the use of any lot or building for military purposes, including armories, staff colleges, military dwelling units, training facilities, administrative offices, dining areas, enclosed storage areas, residential accommodation, recreation facilities or museums.

    - Mineral Aggregate Operation means the use of any lot or building for purposes that are under licence or permit in accordance with the Aggregate Resources Act,

R.S.O. 1990, c. A.8, including associated facilities used in extraction, transport, beneficiation, processing or recycling of mineral aggregate resources and derived products such as asphalt and concrete, or the production of secondary related products, but does not include a wayside pit or wayside quarry.

    - Minimum Distance Separation means formulae and guidelines developed by the Province of Ontario, as amended from time to time, to separate uses so as to reduce incompatibility concerns about odour from livestock facilities.

    - Mixed Use Building means a building that is used for the purpose of one or more

dwelling units and one or more permitted non-residential uses.

    - Mobile Home means a house containing one dwelling unit that is designed to be made mobile, and constructed or manufactured to provide year-round living accommodations, but does not include recreational vehicles, travel trailers, tent trailers or motor homes. A mobile home may include porches or sunrooms as accessory buildings.

(By-Law Number 2022-62; 2024-332)

    - Mobile Home Park means an area under single ownership and used exclusively for the siting of mobile homes, with accessory commercial facilities and recreational uses for residents, including any buildings in or on such mobile home park.

    - Model Home means a house, semi-detached house or townhouse used temporarily for the purpose of an office and/or show room and/or sales centre to promote the sale of residential units within a draft approved plan of subdivision under the Planning Act or a description under the Condominium Act, 1998 proposed for registration.

    - Motor Vehicle means an automobile, motorcycle and motor-assisted bicycle unless otherwise indicated in the Highway Traffic Act, R.S.O. 1990, c. H.8. and any other vehicle propelled or driven otherwise than by muscular power, but does not include trains or other motorized vehicles running only upon rails, or a motorized snowmobile, boat, personal watercraft, trailer, camper, motor home, all-terrain vehicle, traction engine, farm tractor or road-building machine, as defined in the Highway Traffic Act.

    - Municipal Services means a lot that is serviced by both municipal water and municipal sanitary sewer.

    - Museum means the use of any lot or building that is open to the public and in which a collection of objects illustrating science, art, history or related types of information is kept for display and storage. Museums may include retail stores and food concessions as accessory uses.

## N

    - Non-Residential Building means all buildings occupied by non-residential uses

only.

    - Non-Residential Use means all uses that are not residential uses, excluding public uses, parks, or any use in an Open Space Zone or EPA Zone.

## O

    - Occasional Use means the use of any lot or building at limited and infrequent intervals.

    - Office means the use of any lot or building for conducting the affairs of businesses, professions, services, media studios, industries, governments, or other similar activities, in which the chief product of labour is the processing of

information rather than the production and distribution of goods. Office excludes a

#### wellness clinic.

    - On-Farm Diversified Use means the use of any lot or building which is complementary to the principal agricultural use on a lot. On-farm diversified uses may include but are not limited to agri-tourism or other similar uses that produce value-added agricultural products. Examples of on-farm diversified uses may include, but are not limited to:

- Value-added uses that use feedstock from outside the surrounding agricultural area (for example, processor, packager, winery, cheese factory, bakery abattoir);

- Office, creativity centre, personal service shop, day care centre that exceed

home occupation permissions;

- Sawmill, welding or woodworking shop, manufacturing/fabrication, equipment repair that exceed home occupation permissions;

- Retail store; and

- Restaurant, tasting room or cooking classes.

    - Outdoor Patio means a surfaced, open space of land at grade or on the roof of a building which is used as an accessory extension of the principal commercial or hospitality use of the lot or building.

    - Outdoor Storage means the use of any lot, outside of a building, for the storage of goods, materials and equipment or the display and sale of goods, materials and equipment, outside of a building, but excludes a parking lot, a loading space, a salvage yard or an outside area on a permitted agricultural use for the storage of agricultural equipment.

## P

    - Parapet means the low protective or ornamental wall above the cornice of a

#### building.

    - Park means the use of any lot consisting largely of open space, which may include outdoor recreational areas, sports fields, playgrounds, playfield, food concession, beach, outdoor theatres, or other similar uses in a manner that is generally accessible to the public.

    - Parking Lot means the accessory use of any lot or building for the parking of more than one motor vehicle in a configuration where a driveway is not in line with and does not provide direct access to the parking space. Parking lot includes drive aisles and parking spaces and excludes any area where motor vehicles for sale or repair are kept or stored. Parking lot excludes tandem parking spaces that are in line with and directly accessed from a driveway.

    - Parking Structure means a building for the parking of four or more motor vehicles, but excludes a private garage accessory to a house, semi-detached house, or townhouse.

(By-Law Number 2022-62; 2024-332)

    - Parking Space means an unobstructed area dedicated solely for the purpose of parking of a motor vehicle with unimpeded access directly from a street, drive aisle or driveway.

    - Partial Services means a lot that is serviced by either municipal water or municipal sanitary sewer, but not both.

    - Passive Recreation means the use of an outdoor area of any lot for recreational purposes such as walking, running, cycling, hiking, bird watching or other similar activities. Sports fields and playgrounds are not considered passive recreation.

    - Patio means a surfaced, open area of land at grade on a lot that is used for the purpose of amenity and provided with a stable, hard surface treatment. Patio excludes driveways, drive aisles, parking spaces, parking lots, loading spaces or anywhere a vehicle is parked or driven.

    - Person means any human being, association, firm, partnership, corporation, agent or trustee, and their heirs, executors or other legal representatives of a person to whom the context can apply according to the law.

    - Personal Service Shop means the use of any lot or building in which services involving the care of persons or their apparel are offered and includes a barber, hairdresser, beautician, aesthetician, tailor, dressmaker, shoemaker, tanning salon, spa or similar service establishments as licensed by the City. The sale of merchandise is only permitted as an accessory use to the principal service provided.

    - Pigeon means a bird from the family Columbidae that is subject to a pigeon loft permit issued in accordance with the Animal Control By-law.

    - Pigeon Loft means an accessory building where pigeons are kept.

    - Pipeline means a system of pipes, pumps, valves and control devices for the long- distance transportation of oil or gas.

    - Place of Worship means the use of any lot or building for the regular assembly of

persons for the practice of religious worship, services, or rites.

    - Planning Act means the Planning Act, R.S.O. 1990, c. P.13.

    - Planting Strip means the area of a lot used or intended to be used for the sole purpose of planting a row of trees or a continuous hedgerow of evergreens or shrubs and may include supplementary planting of ornamental shrubs, flowering shrubs, flower beds or a combination thereof.

    - Porch means a building component that is an unenclosed or partially enclosed platform covered by a roof, and which is attached to a main wall of a building. A porch includes all associated guards, fencing, walls, visual screens, columns, roof, stairs and other associated features. A porch may or may not have a foundation.

    - Post-Secondary Institution means a university, college of applied arts and technology or other post-secondary institution established pursuant to the Ministry of Training, Colleges and Universities Act, R.S.O. 1990, c. M.19.

    - Principal means a main or primary purpose for which any lot is used. Principal building means the building where a principal use is located.

    - Private Garage means a building which is designed or used for the sheltering of vehicles and storage of household equipment accessory to the principal residential use. A private garage includes a carport.

    - Private Services means a lot that is not serviced by municipal water or municipal sanitary sewer.

    - Private Sewage System means a building that is a sewage system as defined in O. Reg. 332/12 under the Building Code Act, 1992.

(By-Law Number 2022-62; 2024-332)

    - Private Street means a private right-of-way that is used by motor vehicles which is not deeded to the City or any other public authority as a public right-of-way, and which has been legally established through a plan of subdivision under the Planning Act or a description under the Condominium Act, 1998.

A private right-of-way with deeded access for motor vehicles in an LSR Zone is also considered to be a private street.

    - Production Studio means the use of any lot or building for producing photography, live broadcasts, motion pictures, audio or video recordings or transmissions or similar uses.

    - Public Authority means any Federal, Provincial, Regional or Municipal agency including any commission, ministry, board authority, or department established by such agency.

    - Public Market means the use of any lot or building where produce, meat, flowers, fruit, crafts, paintings and other items, largely produced by the vendor, are sold to the public at retail by vendors from individual stalls or stands.

    - Public Use means the use of any lot or building for a public service by a public authority including the City, any conservation authority established by the Province of Ontario, any utility company, or any railway company authorized under the Canada Transportation Act, S.C. 1996 c. 10. Public use includes the use of any lot or building owned by a public authority by any other third party, including where the party is not a public authority, for any purpose authorized by the applicable public authority.

## Q

    - Reserved

## R

    - Rear Lot Line means the lot line opposite to, and most distant from, the front lot line. In the case of a pie shaped lot or a lot where there is no lot line opposite to, and most distant from, the front lot line, there is no rear lot line but rear yard setback and other provisions calculated from a rear lot line must be taken from the point of intersection of the interior lot lines and/or exterior lot lines. [Note: See Diagram 3.12.17.]

    - Rear Setback means the setback between the rear lot line and the nearest part of any building on the lot.

In the case of a lot with no rear lot line, the rear setback is the setback from the point of intersection of the interior lot lines and/or exterior lot lines and the

nearest part of any building on the lot at an angle that is perpendicular to the main wall facing the rear lot line. [Note: See Diagram 3.19.8.]

    - Rear Wall means the portion of the main wall of the principal building that is farthest from the front setback. Projections, bay windows and chimney breasts of

0.5 metres or less from the rear wall are excluded.

    - Rear Yard means a yard extending across the full width of the lot between the rear lot line and a building on the lot. Where the main wall facing the rear lot line is not parallel to the rear lot line, the rear yard is projected as a linear extension of the main wall to the lot lines.

In the case of a lot with no rear lot line, the rear yard is the yard extending from the main wall facing the point of intersection of the interior lot lines and/or exterior lot lines to the point of intersection of such lot lines. [Note: See Diagram 3.19.8.]

    - Recreation Facility means the use of any lot or building for athletic or recreation activities, which may include a community centre, club, ice or roller skating rink, curling rink, indoor paintball facility, axe throwing, racquet club, swimming pool, golf driving range, billiard parlour and bowling alley or other similar uses where the principal focus of the use is the participation in athletic or recreation activities.

    - Recreational Vehicle Sales Establishment means the use of any lot or building for the storage and display of boats, trailers, campers, motor homes, all-terrain vehicles and other similar equipment for sale, rent or lease, but excludes mobile homes or motor vehicles. Accessory uses may include facilities for the repair and maintenance of such recreational vehicles.

    - Renovate means the repair, strengthening, or restoration of a building, but does not include its replacement.

    - Repair Shop means the use of any lot or building for the servicing, repairing or refurbishing of goods, appliances, furniture and small engines, excluding the repair of internal combustion engines, motor vehicles or other similar products.

    - Research Establishment means the use of any lot or building for research, data collection and manipulation, and/or technical development of information or devices for application, excluding a laboratory.

    - Residential Building means a house, semi-detached house, townhouse, stacked townhouse and apartment building, as well as a mixed use building containing a dwelling unit an accessory house, additional rural unit and a tiny house.

(By-Law Number 2022-62; 2024-332)

Diagram 3.18.10. – Deleted

(By-Law Number 2022-62; 2024-332)

    - Residential Use means a dwelling unit, and a co-living unit,

(By-Law Number 2022-62; 2024-322)

    - Restaurant means the use of any lot or building in which the principal business is the preparation and serving of food and/or beverages to the public for consumption on or off the premises, and which may include the preparation of food in a ready to consume state for consumption off the premises. A restaurant includes a take-out restaurant, a bakery, and other similar uses.

    - Retail Store means the use of any lot or building for the sale, rental or lease of goods, merchandise, substances or commodities directly to the general public including pharmacies, convenience stores, markets, and other similar stores, but excludes uses that are otherwise defined herein.

    - Right-of-Way means an area of land that is legally described in a registered deed for the provision of public or private access.

    - Rural Area means the area delineated as “Rural Area” on Schedule 4 .

    - Rural Use means the use of any lot in the rural area for personal, non-commercial purposes and the personal, general enjoyment of rural lands including hunting, fishing, bird watching, canoeing, kayaking, hiking, snowmobiling, all terrain vehicles, seasonal camping or other similar uses.

## S

    - Salvage Yard means the use of any lot or building where goods, wares, merchandise, articles or things are processed for further use and where such goods, wares, merchandise, articles or things are stored wholly or partly in the open. This definition may include a junk yard, a scrap metal yard and an automobile wrecking establishment.

    - Second Rural Unit means an additional rural unit, which is the first accessory dwelling unit in chronological order of development located on the same lot as the principal dwelling unit.

(By-Law Number 2022-62; 2024-332)

    - Secondary School means the use of any lot or building for academic instruction typically offered from grades 9 to 12 including private, public or separate schools, but does not include a training facility. Permitted accessory uses include day care centres.

    - Self-Service Storage Facility means the use of any lot or building for the purpose of providing individual secured storage units and other spaces to persons accessing such units or spaces for the purpose of storing and removing property.

    - Semi-Detached House means a ground oriented residential building that is used for the purpose of two dwelling units and configured in such a manner that the dwelling units are divided vertically beside each other and with each principal dwelling unit being located on its own independent lot. This definition also includes a semi-detached house that has been converted to accommodate more than one dwelling unit on a lot in the principal building, subject to the maximum number of dwelling units permitted by the applicable Zone.

(By-Law Number 2022-62; 2024-332)

    - Sensitive Use means residential uses, day care centres, home day cares, parks, elementary schools, secondary schools, post-secondary institutions, special needs facilities, hospitals and any other similar uses.

    - Separation Distance means the horizontal distance between a use or building and a specific use, lot, building or other specified feature. Separation distance is measured as the most direct path between the two specified points, without regard to roads, walkways, sidewalks, or other surface transportation features.

(By-Law Number 2022-62; 2024-332)

      - Service Station means the use of any lot or building where motor vehicle fuel is kept for sale, including a gasoline pump, and/or a facility where oil changes or other minor maintenance tasks are performed on motor vehicles and may also include the following additional functions:

        - The sale of oil, grease, antifreeze, tire tubes, tire accessories, electrical light bulbs, spark plugs, and batteries for motor vehicles, etc.;

        - The sale of convenience commercial goods and food as an accessory use;

        - Carwash as an accessory use;

        - Electrical charging stations for electric vehicles;

        - Minor repairs essential to the actual operation of motor vehicles including ignition adjustment and tire inflation; and/or

        - The sale of propane as an accessory use.

(By-Law Number 2022-62; 2024-332)

    - Setback means the horizontal distance between a lot line and the nearest part of any building on the lot, excluding such features that are specifically permitted to project into required setbacks. Setback includes front setback, rear setback, interior setback and exterior setback.

(By-Law Number 2022-62; 2024-332)

#### Diagram 3.19.8. – Setbacks & Yards

    - Shoreline means any lot line or part thereof which abuts a waterbody.

    - Short-term Bike Space means a bike space that is intended for use by visitors to a

residential building or customers of a business.

    - Sight Triangle means an unobstructed triangular area of land on a corner lot

where the front lot line and exterior lot line intersect.

    - Sign means any device, object or visual medium used to convey information by way of color, form, graphic, illumination, symbol or writing, displayed or intended to be displayed on any lot or building to attract attention to a specific subject matter for identification, information or advertising purposes, and which is regulated by the Signs By-law.

    - Reserved

(By-Law Number 2022-62; 2024-332)

    - Special Needs Facility means the use of any lot or building for housing providing a group living arrangement for people who have specific needs beyond economic needs and that is not a group home, including but not limited to, needs such as mobility requirements or support functions required for daily living. This includes any dedicated facilities for such use and may include a wellness clinic as an accessory use.

    - Specific Day Retail Sales means the accessory use of any lot or building for an event that brings together members of the public or an industry for the purpose of selling or buying goods or services, including entering into contracts for the sale and/or purchase of goods or services.

    - Stacked Bike Space means a horizontal bike space that is positioned above or below another horizontal bike space.

    - Stacked Townhouse means a residential building that is used for the purpose of five or more dwelling units and configured in a manner that a portion of the dwelling units are located entirely or partially above the other portion of the dwelling units, and where each dwelling unit has its own independent external access outside.

(By-Law Number 2022-62; 2024-332)

    - Stadium means the use of any lot where outdoor or partially enclosed facilities are provided primarily for the gathering of persons for civil, political, travel, religious, social, educational, recreational, or similar purposes or for the consumption of food or drink, excluding outdoor seating at a sports field or any other use otherwise defined herein.

    - Stepback means the horizontal distance from the exterior wall of a specified storey to the exterior wall of the storey immediately below it. The horizontal distance must be measured in the direction that is opposite to the lot line, ensuring that the stepback moves towards the centre of the lot.

    - Storey means occupied space of a building between the top of any floor and the top of the floor next above it, or between the top of the floor and the ceiling above the floor, if there is no floor above it. Any portion of a building partly below ground is deemed a storey where any part of its ceiling is 1.0 metre or more above finished grade. An attic is not a storey. A mezzanine level is not a storey where the floor area of the mezzanine is equal to 40% or less of the gross floor area of the storey immediately below it. Where the floor area of a mezzanine level exceeds 40% of the gross floor area of the storey immediately below it, it is a storey.

    - Street means a public street or highway in accordance with the terms of the Municipal Act, but does not include unopened road allowances. For the purpose of this By-law, a private street is considered a street.

    - Street Line means the boundary between a street and a lot.

    - Streetwall means the wall of a building or portion of a wall facing a street line.

    - Streetwall Height means the vertical distance between the top of the streetwall

and the finished grade.

    - Swale means a graded or engineered landscape feature appearing as a linear, shallow, open channel for the purpose of moving or holding water.

    - Swimming Pool means a building that is a tank, pool, or artificial body of water which may be used for swimming or wading purposes and which has a possible maximum depth of water greater than 0.60 metres. A swimming pool includes a hot tub or whirlpool and includes all of its associated and unenclosed mechanical equipment. A swimming pool excludes a pond, reservoir, stormwater management facility or any natural body of water.

## T

    - Tandem Parking Spaces means two parking spaces where one parking space is only accessed by passing through the other parking space from a drive aisle or driveway.

    - Third Rural Unit means an additional rural unit, which is the second accessory dwelling unit in chronological order of development located on the same lot as the principal dwelling unit.

(By-Law Number 2022-62; 2024-332)

    - Tiny House means a house containing one dwelling unit that is designed to be portable and provide year-round living accommodations, but does not include recreational vehicles, travel trailers, tent trailers or motor homes.

(By-Law Number 2022-62; 2024-332)

    - Through Lot means a lot bounded on opposite sides by a street. However, if the lot qualifies as being a corner lot and a through lot, such lot is deemed to be a corner lot for the purposes of this By-law. [Note: See Diagram 3.12.17.]

    - Tourism Use means the use of any lot or building that has been designed and equipped for the purposes of tourism and may include visitor reception, tourist information, ticket and toll booths or interpretation facilities.

    - Towing Compound means the use of any lot or building for the temporary storage of towed motor vehicles and may include enclosed outdoor storage, but excludes a salvage yard, automobile body shop or automobile repair shop.

    - Townhouse means a ground oriented residential building that is used for the purpose of three or more dwelling units and configured in such a manner that the dwelling units are divided vertically beside each other, with each dwelling unit having its own independent external access outside and with each principal dwelling unit being located on its own independent lot, including both freehold and common element lots. This definition includes back-to-back townhouses and also includes a townhouse that has been converted to accommodate more than one dwelling unit on a lot in the principal building, subject to the maximum number of dwelling units permitted by the applicable Zone.

(By-Law Number 2022-62; 2024-332)

    - Trade Show means the accessory use of any lot or building for an event held to bring together members of a particular industry to display, demonstrate, and promote their latest products and services.

    - Training Facility means the use of any lot or building in which training or educational services are offered, but does not include an elementary school, secondary school or post-secondary institution. A training facility may include but is not limited to the instruction of a trade, skill, service (such as driving), administration, dance, calisthenics, business, aviation, art, language, hairdressing, music, culture or sport.

    - Transformer Station means the use of any lot or building for the generation, transmission or distribution of electricity.

    - Transportation Depot means the use of any lot or building where buses, taxis, trucks, tractor trailers or other similar commercial vehicles are dispatched, rented, leased, maintained, stored or parked for commercial purposes, and includes any technical training facility directly associated with these vehicles, but excludes uses otherwise defined herein.

    - Transportation Terminal means the use of any lot or building where buses or trains pick up and discharge fare-paying passengers, which may include accessory offices, retail stores, food concessions or restaurants.

    - Deleted

(By-Law Number 2022-62; 2024-332)

## U

    - Urban Area means the area delineated as “Urban Area” on Schedule 4.

    - Use, as a noun, means the purpose for which any lot or building is arranged, designed or intended to be occupied or maintained. As a verb, use means the doing or permitting of anything by the owner or occupant of any lot or building directly or indirectly or by or through any trustee, tenant, servant or agent acting for or with the knowledge and consent of the owner or occupant, for the purpose of making use of the said lot or building.

## V

    - Vehicle means a motor vehicle, or other device, including motorized construction equipment, farm equipment, motor home, motorized mobility device, snowmobile, boat, recreational vehicle, and also including a trailer or farm implement or any other device which is capable of being driven, propelled or drawn by any kind of power, but excludes a bicycle or any other device powered solely by means of human effort.

    - Vertical Bike Space means a bike space that is provided in a vertical format where no bike spaces are immediately above or below.

    - Visitor Space means a parking space dedicated for the exclusive use of motor vehicles driven by people who do not reside in a dwelling unit on the lot.

    - Visual Screen means a human-made or natural area on a lot that provides a visual barrier in such a manner that the item being screened is not visible from a specified point or area.

## W

    - Walking Distance means the linear distance between a specific use, lot, or building and another specified feature along an active transportation route. Walking distance is measured as the most direct path between the two specified features along streets, public sidewalks, publicly accessible walkways or other surface transportation features that are accessible to the public.

(By-Law Number 2022-62; 2024-332)

    - Walkway means a hard surface treated path that provides pedestrian and/or active transportation access to the exterior entrance of a building.

    - Warehouse means the use of any lot or building primarily for the storage and/or distribution of goods or materials and which may include a wholesale establishment as an accessory use.

    - Waste Disposal Area means the use of any lot or building providing for the long- term storage or destruction of solid waste.

    - Waste Processing Site means the use of any building for the principal purpose of sorting and processing waste to create a new product or raw material on site, and may include a recycling centre.

    - Waste Transfer Station means the use of any building for the principal purpose of collection and storage of waste for shipment, and may include limited sorting or preparation of that waste to facilitate its shipment.

    - Water Frontage means the straight line horizontal distance between the two most widely separated points on any one shoreline of a lot.

    - Water Supply Plant means the use of any lot or building approved by the Ministry of Environment, Conservation and Parks, where water is treated for human consumption.

    - Waterbody means a lake, canal, pond, wetland, river, watercourse, seasonal watercourse or municipal drain as defined by the Drainage Act, but does not include an artificially constructed swale or ditch intended for intermittent and minor surface drainage of an area or a lot.

(By-Law Number 2022-62; 2024-332)

    - Wastewater Treatment Facility means the use of any lot or building approved by the Ministry of Environment, Conservation and Parks, where domestic and/or industrial sewage waste is treated by a private individual or corporation.

    - Wayside Pit or Wayside Quarry means the use of any lot or building as a temporary pit or quarry opened and used by or for a public authority solely for the purpose of a particular project or contract of street construction and not located on the street right-of-way.

    - Wellhead Protection Area means an area of land surrounding a well where human activities may need to be regulated to protect the quality and quantity of groundwater that supplies that well.

    - Wellness Clinic means the use of any lot or building by physicians, dentists, physiotherapists, chiropractors, nurses, naturopaths, osteopaths, psychologists, therapists, registered massage therapists, optometrists or other similar medically focused practitioners, for the purpose of consultation, diagnosis, and office treatment associated with such profession. A wellness clinic may include administrative offices, waiting rooms, treatment rooms, laboratories, pharmacies and dispensaries directly associated with the clinic, but excludes a hospital.

    - Wetland means lands that are seasonally or permanently covered by shallow water, as well as lands where the water table is close to or at the surface. In either case the presence of abundant water has caused the formation of hydric soils and has favoured the dominance of either hydrophytic plants or water tolerant plants. The four major types of wetlands are swamps, marshes, bogs and fens. Periodically soaked or wet lands being used for agricultural uses, which no longer exhibit wetland characteristics, are not considered to be wetlands for the purposes of this definition.

    - Wholesale Establishment means the use of any lot or building for the storage or distribution of goods from a manufacturer to a business or person other than the end user or consumer.

    - Workshop means the use of any lot or building where manufacturing is performed by tradespeople requiring manual or mechanical skills and may include a machine shop, carpenter’s shop, furniture maker’s shop, locksmith’s shop, gunsmith’s shop,

tinsmith’s shop, commercial welder’s shop or similar uses and excludes uses

otherwise defined herein.

## X

    - Reserved

## Y

    - Yard means any open, uncovered space appurtenant to and located on the same lot as a building. Where a main wall is not parallel to a lot line, the yard is projected from the main wall.

## Z

    - Reserved

# Section 4:	General Provisions

### Accessory Uses and Buildings

#### Accessory Provisions for All Uses and Buildings

    - Accessory uses and buildings are permitted in all Zones, unless otherwise provided by this By-law, subject to the following provisions:

      - The principal use or building must already be established on the same lot as the accessory use or building;

      - Accessory buildings must comply with the Zone provisions that are applicable to the lot, except for minimum height requirements and except as otherwise specified in this Subsection;

(By-Law Number 2022-62; 2024-332)

      - Parking lots, driveways, private garages and parking structures are permitted as an accessory use to a permitted principal use; and

      - Treehouses and mechanical equipment which is installed outdoors, such as generators, air conditioners, heat pumps, transformers, solar panels or other similar equipment, are considered accessory buildings for the purpose of this By-law, except where they are enclosed in a building that is attached to the principal building, then they are considered a component of the principal building and must comply with the provisions of this By-law that apply to the principal building.

#### Additional Accessory Provisions for Residential Uses and Buildings

    - In an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, HAM Zone, RUR Zone, LSR Zone or DR Zone, an accessory building, excluding a marine facility, must comply with the following requirements:

      - The accessory building must be located in a rear yard, exterior yard or interior yard except in an RUR Zone or LSR Zone where an accessory building is permitted in the front yard if it complies with the required front setback applicable to the principal building;

(By-Law Number 2022-62; 2024-332)

      - In the urban area, in addition to the lot coverage of the applicable Zone, the maximum lot coverage for all accessory buildings on one lot is 10% in the aggregate;

(By-Law Number 2022-62; 2024-332)

      - In the rural area, accessory buildings are subject to and included in the lot coverage provision of the applicable Zone, except:

        - Where a lot existed as of the date of passing of this By-law that is smaller than the minimum lot area of the applicable Zone, in addition to the lot coverage of the applicable Zone, the maximum lot coverage for all accessory buildings on the lot is an additional 5% in the aggregate.

(By-Law Number 2022-62; 2024-332)

      - The maximum height of an accessory building is 4.6 metres except in an RUR Zone where the maximum height of an accessory building is 6.0 metres;

(By-Law Number 2022-62; 2024-332)

      - The accessory building may encroach into the required setback on a lot provided that such accessory building must be setback a minimum of 1.2 metres from all lot lines;

      - The accessory building must comply with the minimum required separation distance from a waterbody in accordance with Clause 4.23.1;

      - Despite Subclause 5. and Clause 4.23.1., on a lot that existed as of the date of passing of this By-law, a maximum of one accessory building no greater than 10 square metres in area is permitted within the required 30 metre separation distance, provided that it maintains a minimum separation distance of 7.5 metres from the high water mark; and,

(By-Law Number 2022-62; 2024-332)

      - Despite Subclause 5., where an accessory building is less than 2.4 metres in height and 15 square metres in gross area (outside perimeter roof dimensions), the minimum setback from all lot lines is 0.6 metres.

(By-Law Number 2022-62; 2024-332)

#### Sleeping Accommodations and General Living Areas in Accessory Buildings

    - In the urban area, accessory buildings:

      - Are not permitted to be used as any type of sleeping accommodation, except for accessory houses and tiny houses as permitted by this By-law; and

(By-Law Number 2022-62; 2024-332)

      - Are permitted to be used as general living areas, including but not limited to an amenity area, study, den, living room, recreational space, studio, home office or home occupation, subject to the following provisions:

        - the building must be accessory to a principal residential use;

        - the minimum gross floor area of the first storey of the building, excluding a mezzanine area, is greater than 10 square metres; and

        - the building must comply with all other provisions that are applicable to an accessory building on the lot.

      - For clarity, an accessory building used as a general living area in accordance with Paragraph 2. is not considered a bedroom.

    - In the rural area, on any lot in a RU, RUR, LSR or HAM Zone, one bunkhouse is permitted as an accessory building, subject to the following provisions:

      - The maximum height of a bunkhouse is the lesser of 4.6 metres or 1 storey;

      - Despite Paragraph 1. and the definition of storey, a mezzanine level is permitted to a maximum floor area equal to 40% of the gross floor area of the first storey and is not considered a storey;

      - The minimum gross floor area of the first storey of the bunkhouse, excluding the mezzanine area permitted by Paragraph 2., is greater than 10 square metres;

      - The maximum gross floor area of the first storey of the bunkhouse, excluding the mezzanine area permitted by Paragraph 2., is 37 square metres; and

      - The bunkhouse must comply with all other provisions that are applicable to an

accessory building on the lot.

    - In the rural area, accessory buildings are permitted to be used as general living areas, including but not limited to an amenity area, study, den, living room, recreational space, studio, home office or home occupation and must comply with the following provisions:

      - The building must be accessory to a principal residential use;

      - The minimum gross floor area of the first storey of the building, excluding a mezzanine area, is greater than 10 square metres; and

      - The building must comply with all other provisions that are applicable to an

accessory building on the lot.

#### Additional Accessory Provisions for Buildings in the WM1 and WM2 Zones

    - In a WM1 or WM2 Zone, an accessory building must be located in a rear yard or

interior yard and must comply with the following requirements:

      - The maximum lot coverage for all accessory buildings on one lot is 10% in the aggregate;

      - The maximum height is 4.6 metres;

      - The accessory building may encroach into the required rear setback provided that such accessory building is setback a minimum of 1.2 metres from the rear lot line; and

      - The accessory building must not be located closer to a street line than the

streetwall of the principal building.

### Swimming Pools

    - Despite anything to the contrary in this By-law, an outdoor swimming pool, including all of its associated and unenclosed mechanical equipment, must be developed in accordance with the following provisions:

      - The minimum setback is 1.5 metres from a swimming pool to any lot line;

      - A swimming pool must not be located in a front yard or within any form of drainage management system such as a swale or ditch;

      - The maximum height of a swimming pool is 2.0 metres, excluding related equipment which must not exceed a maximum height of 4.5 metres. For the purpose of this Subsection, height of a swimming pool is the difference between the highest point of the swimming pool and the average grade level around the perimeter of the swimming pool;

      - A swimming pool must comply with the minimum separation distance from a waterbody in accordance with Subsection 4.23.; and

      - For the purpose of this Subsection, decks and buildings accessory to a swimming pool are not considered related equipment and must comply with all other provisions of this By-law.

    - Despite 4.2.1.2., where the main pedestrian entrance of a building on a corner lot is oriented towards the exterior lot line, a swimming pool may be located in the front yard.

### Amenity Area

    - A minimum of 18.5 square metres of amenity area must be provided for each dwelling unit on a lot with four or more dwelling units in a stacked townhouse, apartment building or mixed use building. Where a stacked townhouse is configured in a manner that each dwelling unit is located on its own individual lot, this requirement does not apply.

(By-Law Number 2022-62; 2024-332; 2024-333)

    - Despite Clause 4.3.1., in the PA1, PA2, PAA3 or on lands subject to and developed in accordance with the Express Transit Area Overlay framework, a minimum of 10.0 square metres of amenity area must be provided for each dwelling unit on a lot.

(By-Law Number 2022-62; 2024-332)

    - Amenity areas, or any part thereof, must be designed and located so that the length does not exceed four times the width.

    - Amenity areas, if provided as communal space, must be aggregated into one area or grouped into areas of not less than 54.0 square metres.

    - Where communal amenity areas are provided exterior to a building, such amenity areas must be accessed by a barrier-free path of travel that is a minimum 1.5 metres in width connecting the amenity area to a street line or a building.

### Lot Servicing

    - In the urban area, a lot or building must not be used or developed unless there is adequate capacity in the sanitary sewer, stormwater sewer or stormwater management facility, and adequate potable water from a public water supply system, to the satisfaction of the City.

    - Where a lot in the urban area is developed as of the date of passing of this By-law on private services or partial services, such private services or partial services may be deemed to fulfill the servicing requirements of Clause 4.4.1. where there is adequate capacity and/or water supply to the satisfaction of the City. The replacement of such existing private services or partial services is permitted.

    - In the rural area, a lot or building must not be used or developed unless there are: adequate public, private, or communal sewage disposal facilities; adequate storm drainage facilities; and adequate potable water from a public, private, or communal water supply, to the satisfaction of the City.

    - Where a lot in the rural area is developed as of the date of passing of this By-law on partial services, such partial services may be deemed to fulfill the servicing requirements of Clause 4.4.3. where there is adequate capacity and/or water supply to the satisfaction of the City. The replacement of such partial services is permitted.

    - Despite the Housing Constraint Area Overlay or any other provision of this By-law that establishes a Holding Overlay in accordance with Clause 2.6.2. in the urban area connected to sanitary sewer, stormwater sewer, or a public water supply system capacity, where a lot is developed in a manner that does not require a new lateral or increase the size of an existing lateral connection to a municipally owned water, sanitary or stormwater system, the applicable Holding Overlay condition does not apply.

(By-Law Number 2022-62; 2024-332)

### Angular Planes and Build-to-Planes

#### Angular Planes

    - In addition to maximum height, in certain Zones the calculation of an angular plane is required in determining maximum height. Where the calculation of an angular plane is required, the following provisions apply:

      - No part of a building is permitted to project above the angular plane unless otherwise specified in this By-law;

      - The maximum height provision for the specific Zone continues to apply in addition to the angular plane provisions; and

      - The angular plane commences at a specified height measured from the average existing grade of the build-to-plane and equals the length of the street line.

#### Build-to-Plane

    - Where the calculation of a build-to-plane is required, the following provisions apply:

      - The height of the build-to-plane must be the lesser of the maximum height for the underlying Zone, or the height specified for commencement of an angular plane;

      - The length of the build-to-plane equals the length of the street line; and

      - A minimum of 80% of the main wall of any building constructed along the street line must be developed at the build-to-plane. The balance of the main wall of the building must not encroach within the specified setback distance for the build-to-plane.

Diagram 4.5.2. – Angular Plane and Build-to-Plane

### Sight Triangles

    - Despite anything to the contrary in this By-law, the following are prohibited within a

#### sight triangle:

      - A building;

      - Any equipment, feature or landscaping, including a wall, tree, hedge, bush or other similar feature, which exceeds 1.0 metre in height above the elevation of the intersection of the two centre lines of the adjacent streets;

      - A driveway, parking lot, commercial parking lot or parking space; and

      - A grade level which exceeds the elevation of the intersection of the two centre lines of the adjacent streets by more than 0.5 metres.

    - Sight triangles are required on a corner lot and must comply with the dimensions identified Table 4.6.2., in accordance with the street type identified in Schedule 4 of this By-law. Table 4.6.2 provides the required length along the front lot line in metres and the required length along the exterior lot line in metres with an “x” in between (for example, “4.5 x 4.5” is 4.5 metres along the front lot line and 4.5 metres along the exterior lot line).

#### Table 4.6.2. – Sight Triangle Dimensions

|  |  | Type of Subject Street Line |  |  |
| --- | --- | --- | --- | --- |
|  |  | Local Road | Collector Road | Arterial Road |
| Type of Intersecting Street Line | Local Road | 4.5 x 4.5 | 9.0 x 9.0 | 9.0 x 9.0 |
|  | Collector Road | 9.0 x 9.0 | 9.0 x 9.0 | 15.0 x 15.0 |
|  | Arterial Road | 9.0 x 9.0 | 15.0 x 15.0 | 15.0 x 15.0 |

    - The area within a sight triangle is determined by measuring from the point of intersection of the front lot line and the exterior lot line the required length along each street line as per Table 4.6.2. A third line connects the two points on the two street lines to complete the sight triangle.

    - Despite Clause 4.6.2., where a development is subject to a Site Plan Control application under Section 41 of the Planning Act or a plan of subdivision application under Section 51 of the Planning Act, such sight triangle may be reduced to the satisfaction of the City through the applicable process.

(By-Law Number 2022-62; 2024-332)

    - Despite Clause 4.6.2., sight triangle provisions do not apply to a building that existed as of the date of passing of this By-law on a lot in the DT1 Zone, DT2 Zone and HCD2 Zone.

    - Despite the definition of sight triangle, where a corner lot includes a front lot line and exterior lot line that do not intersect at one point, the area within the sight triangle is determined by measuring the hypothetical point of intersection of the extension of the front lot line and the extension of the exterior lot line.

Diagram 4.6.6. – Sight Triangle

### Drive-Through Provisions

    - The minimum separation distance from a drive-through to a lot within an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, HAM Zone, RUR Zone, LSR Zone or DR Zone is 15.0 metres.

    - Where a lot is used for a drive-through and the interior lot line or rear lot line abuts a residential use or undeveloped land in an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, HAM Zone, RUR Zone, LSR Zone or DR Zone, then:

      - A minimum 3.0 metre wide planting strip must be provided along the portion of the lot line that abuts such use or lot;

      - The minimum height for the row of trees or a continuous hedgerow of evergreens or shrubs located in the planting strip is 1.5 metres; and

      - Driveways, walkways and similar features are permitted to cut across a

planting strip perpendicularly.

    - Where a lot is used for a drive-through and has a street line located on the opposite side of the street from a residential use or undeveloped land in an Urban Residential Zone, Urban Multi-Unit Residential Zone, HAM Zone, HCD1 Zone, HCD3 Zone, RUR Zone, LSR Zone or DR Zone, then:

      - A minimum 3.0 metre wide planting strip must be provided along the portion of the street line that is opposite to such use or lot; and

      - Driveways, walkways and similar features are permitted to cut across a

planting strip perpendicularly.

### Frontage on a Public Street

    - A lot or building must not be used or developed unless the lot has a lot line

which is also a street line.

    - Despite Clause 4.8.1.:

      - A lot is permitted to be used or developed as a public park where such lot abuts a right-of-way that is used by motor vehicles with deeded access to the City; and

      - A lot in existence on the date of passing of this By-law in the rural area that does not have a lot line which is also a street line, is permitted to be used for agricultural uses, rural uses or forestry uses if the applicable Zone permits such use. No buildings are permitted on such lot.

### Generally Permitted Uses

#### Uses Permitted in all Zones

    - Despite anything to the contrary in this By-law, except where greater restrictions are provided in the Overlay Provisions in Section 5 and waterbody separation distance provisions of Subsection 4.23., any lot or building may be used or developed in any Zone, except for an EPA Zone, for any of the uses indicated in Subclauses 1 through 10 below. All buildings developed in conjunction with these uses must comply with all provisions of this By-law that apply to the lot, except for minimum lot area or minimum lot frontage requirements or where a particular use is specifically exempt from a provision:

      - Municipal infrastructure, stormwater management facilities, electricity transmission facilities, oil and natural gas pipelines;

      - Electricity generation facilities and electricity transmission and distribution infrastructure;

      - Public utility installations required by any public authority or private utility for the provision of gas, oil, water, electricity, sanitary, storm water management, transportation, telephone communication and other similar utilities to the general public;

      - Public uses, conservation areas, conservation uses, flood or erosion control facilities, or other similar uses;

#### Passive recreation;

      - Marine facilities, subject to the provisions of Subsection 6.5.;

      - Community gardens and urban agriculture that are not commercial operations, provided no building or part thereof, including an arbour or other such associated component, is permitted within 1.5 metres of a street;

      - Any use that responds to an emergency, such as an emergency warming shelter or an emergency operations centre or other similar use, to the satisfaction of the City;

      - A public transit facility; and

      - Not-for-profit community food centres.

    - Despite anything to the contrary in this By-law, except where greater restrictions are provided in the Overlay Provisions in Section 5 and waterbody separation distance provisions of Subsection 4.23., any lot or building may be used or developed for essential emergency service uses such as fire halls, police stations, ambulance stations, and similar public response uses in any Zone, except for EPA, AG and MX1 Zones. All buildings developed in conjunction with these uses must comply with all provisions of this By-law that apply to the lot.

    - Despite anything to the contrary in this By-law, parks are permitted as the principal use of any lot or building in any Zone, except for the AG and MX1 Zones. Parks are only permitted as an accessory use or building in the AG and MX1 Zones.

### Complementary Uses

    - Complementary uses are only permitted in Zones where they are specifically permitted by this By-law or in conjunction with a use where the definition specifically identifies permitted complementary uses.

### Prohibited Uses in All Zones

    - Despite anything to the contrary in this By-law, the following uses and activities are prohibited in all Zones, either alone or in conjunction with other uses, except where specifically permitted by this By-law:

      - Any activity or use prohibited by the Environmental Protection Act, R.S.O. 1990,

c. E.19;

      - The incineration or disposal of biomedical wastes, organic or inorganic chemical wastes, or radioactive wastes; and

      - Any uses or activities deemed unlawful under other Federal, Provincial, or Municipal acts, codes, or by-laws.

### Items Exempt from this By-law

    - This By-law is not intended to govern the erection or location of any of the following items, except where they are items that are specifically required to fulfill a provision of this By-law (for example a privacy fence for outdoor storage) or items that are specifically prohibited by this By-law (for example a fence within a sight triangle):

#### Signs;

      - Fences;

      - Retaining walls and boundary walls (except where regulated by Subsection 4.24);

      - Drop awnings;

      - Clothes poles;

      - Flag poles;

      - Garden trellises and open air surfaced areas such as a pergola;

      - Light standards (except where regulated by Section 4.24);

      - Seasonal decorations;

      - Public sidewalks, curbs and bollards;

      - Planters, landscaping features, birdbaths, rain barrels and vegetation;

      - Mailboxes, newspaper, lending libraries or similar dispensing or receiving boxes;

      - Public art, statues and sculptures;

      - Furniture;

      - Play equipment, including but not limited to trampolines;

      - Outdoor skating surfaces;

      - Public transit installations, infrastructure, equipment and other similar public transit features;

      - Active transportation installations, infrastructure, equipment and other similar active transportation features, including but not limited to bike racks, bike lockers, and canopies or other features that provide weather protection for bike spaces. For clarity, this exemption includes bike spaces and other active transportation installations that are required by this By-law;

      - All structures, equipment or other similar features permitted by the City to be located within the City’s street, including outdoor patios; and

      - All equipment, amenities or structures provided in a public park.

(By-Law Number 2022-62; 2024-332)

### Building Components Exempt from Specific Provisions

    - Buildings connected to a public use and buildings located in public parks are exempt from the provisions of this By-law which regulate the size and location of such buildings, including but not limited to setbacks, separation distances, height, building depth, lot coverage and landscaped open space.

    - Any building component that is located wholly below existing grade, including

basements, parking structures, geothermal devices, renewable energy devices, , or

other similar building components, are exempt from the provisions of this By-law which regulate setbacks from lot lines or maximum building depth, but must comply with all other applicable provisions of this By-law.

(By-Law Number 2022-62; 2024-332)

    - Private sewage systems are exempt from the provisions of this By-law which regulate the size and location of a building including but not limited to setbacks, separation distances, height and building depth.

(By-Law Number 2022-62; 2024-332)

    - Despite Clause 4.13.2. and 4.13.3., building components that are located wholly below existing grade, private sewage systems, must comply with all setback provisions from waterbodies, natural heritage features, floodplains and other similar environmental protection areas, except:

      - Where an existing private sewage system is updated or replaced with a new private sewage system, the new private sewage system must not be closer to the waterbody, natural heritage feature, floodplain or other environmental protection area than the existing private sewage system.

(By-Law Number 2022-62; 2024-332)

    - For the purpose of this Subsection, “existing grade” means the existing grade of the portion of the lot immediately surrounding and/or on top of such component, prior to the development of such building or building component.

### Temporary Uses

#### Temporary Retail and Seasonal Sales

    - Nothing in this By-law prevents an area outside of a building within a CN, WM1, WM2, CA, CD, CR, CG, CW, DT1, DT2, HCD2, HB, RC and HAM Zone, which is used in conjunction with a business located on the same lot, to be used for the display or retail sales of seasonal produce or new merchandise, provided such area:

      - Is seasonal in nature and does not include a permanent retailing area; and

      - Is not located within a fire lane, a parking lot or a loading space required to fulfill the provisions of this By-law, or a driveway or passageway which provides an access route for vehicular traffic across the lot or to a street which abuts said lot.

#### Temporary Construction Buildings and Equipment

    - Despite anything to the contrary in this By-law, the following temporary buildings or equipment are permitted in any Zone, except an EPA Zone or lands that are subject to the Floodplain Overlay as per Subsection 5.1. or the waterbody separation distance of Subsection 4.23., during a period of construction:

      - A temporary building or equipment essential to construction or a special event in progress on a lot;

      - A temporary office for the sale of residential lots or residential units;

      - In the rural area, a mobile home, tiny house or a recreational vehicle as temporary accommodation for a period not to exceed two years while a permanent dwelling unit is being developed on the same lot; and

(By-Law Number 2022-62; 2024-332)

      - Any temporary sheds, scaffolds or other buildings incidental to building construction on the lot for so long as the same is necessary for work in progress which has neither been finished nor abandoned.

    - The setback provisions of the underlying Zone and the parking provisions do not apply to the temporary buildings or equipment permitted by Subclauses 4.14.2.1.,

4.14.2.3. and 4.14.2.4. All other provisions of this By-law apply, including Clause 4.23.1.

    - The temporary buildings or equipment permitted by Clause 4.14.2. must be located on the lot only until such time as the construction has been completed or abandoned.

### Occasional Uses

    - The following Zones and uses are permitted to have occasional uses as indicated:

      - In a place of worship, elementary school or secondary school or on any lot in a DT1, DT2, HCD2 and OS2 Zone, auction sales, bake sales, craft sales, plant and produce sales, seasonal sales, bingo, blood donor clinics, carwashes, benefit dances, festivals, shows, concerts, banquets, or other similar special events are permitted as an occasional use; and

      - Garage sales are permitted on any lot as an occasional use, with a maximum number of two garage sales being conducted on any one lot in any calendar year. A single garage sale consists of a maximum of two consecutive days.

#### Trade Shows and Specific Day Retail Sales

    - Trade shows and specific day retail sales are permitted as an occasional use in Institutional Zones, the HAM Zone and on any lot where the principal use is a recreation facility, place of worship, community centre, club, hotel or banquet hall.

#### Public Markets

    - Public markets are permitted as an occasional use on any lot in a Mixed Use Zone, Commercial Zone, Institutional Zone or the OS1, OS2, HCD2, RC, HAM Zones. A public market may be located in a parking lot such that it temporarily prevents the use of a portion of the parking spaces, drive aisles or driveways of another use on the same lot, provided that the public market does not obstruct access to a fire route.

### Planting Strip Provisions for Non-Residential Uses

    - Where any lot is used for a non-residential use and the interior lot line or rear lot line abuts a residential use or undeveloped land in an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, RUR Zone, LSR Zone or DR Zone, then:

      - A minimum 3.0 metre wide planting strip must be provided along the portion of the lot line that abuts such use or lot;

      - The minimum height for the row of trees or a continuous hedgerow of evergreens or shrubs located in the planting strip is 1.5 metres; and

      - Driveways, walkways and similar features are permitted to cut across a

planting strip perpendicularly.

    - Where a lot in an RM1, RM2, MX1, M1, M2, M3, M4, M5, TA, TR or TU Zone has a street line located on the opposite side of the street from a residential use or undeveloped land in an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, HAM Zone, RUR Zone, LSR Zone or DR Zone, then:

      - A minimum 3.0 metre wide planting strip must be provided along the portion of the street line that is opposite to such use or lot; and

      - The minimum height for a row of trees or a continuous hedgerow of evergreens or shrubs located in the planting strip is 1.5 metres; and

      - Driveways, walkways and similar features are permitted to cut across a

planting strip perpendicularly.

(By-Law Number 2022-62; 2024-332)

    - Despite Clauses 4.16.1. and 4.16.2., the height of vegetation planted in a required

planting strip must comply with Section 4.6., where applicable.

### Minimum Distance Separation

#### New Non-Agricultural Uses

    - The expansion or establishment of any new use that is not an agricultural use must comply with the requirements of the applicable minimum distance separation calculation (MDS I) from an existing livestock facility.

    - Despite Clause 4.17.1., minimum distance separation calculation (MDS I) does not apply to applications for consent under Section 53 of the Planning Act for a residence surplus to a farming operation, where the dwelling unit and nearby livestock facility or anaerobic digester are located on a separate lot prior to the consent application, as the potential odour conflict already exists.

#### New or Expanded Livestock Facilities

    - New or expanded livestock facilities must comply with the applicable minimum distance separation calculation (MDS II).

    - If a cemetery is closed or receives low levels of visitation, or is not connected to a place of worship, such cemetery will be treated as a Type A land use for the purpose of calculating minimum distance separation (MDS II) requirements in Clause 4.17.3.

#### Existing Lots of Record

    - The minimum distance separation requirements do not apply in the case of a new

use that is not an agricultural use under this Subsection where a lot with an area of

1.0 hectare or less, existed as of the date of passing of this By-law, and for which the

proposed use is listed as a permitted use in the Zone in which the existing lot is located.

#### Higher Standard to Govern

    - Where more than one minimum distance separation and setback provision apply, the more restrictive provision prevails.

#### Backyard Hen Coops and Pigeon Lofts

    - Accessory buildings constructed in accordance with the backyard hen coop and pigeon loft provisions in Subsections 6.12. and 6.13 are exempt from all provisions in Subsection 4.17.

### Projections Above Maximum Height

    - The height provisions of this By-law do not apply to:

      - Barn, corn crib, grain elevator, farm implement shed, feed or bedding storage, silo or other similar building when used in conjunction with an agricultural use;

      - A public bridge or other similar public infrastructure;

      - Chimney and parapets;

      - Hydroelectric transmission tower;

      - Individual farm windmill;

      - Municipal water tower;

      - Non-commercial radio, television or telecommunications equipment; and

      - Belfry, clock tower, cupola, ornamental domes, spires or steeples on a place of worship or a building containing a public use.

    - Despite the height provisions of this By-law, the following building components are permitted to project a maximum of 5.0 metres above the maximum permitted height, with a maximum area of 30% of the roof area on which they are located, in the aggregate, and a minimum setback from the edge of the roof equal to the vertical height of such building component:

      - Mechanical and service equipment penthouse, elevator or stairway penthouses;

      - Enclosed building components providing tenants with access to rooftop

amenity areas; and

      - Skylights or other similar rooftop components.

(By-Law Number 2022-62; 2024-332)

    - Despite Clause 4.18.2., the following building components are permitted within the required setback from the edge of a roof line:

      - An enclosure dedicated only to stairs that are located at the edge of a

building; and

      - A safety railing for rooftop areas accessible to residents of the building.

    - Despite the height provisions of this By-law, the following building components may exceed the maximum height provision by a maximum of 3.5 metres:

      - A static renewable energy device such as a solar panel or other similar devices that capture or conserve energy without visible motion or emissions;

      - Architectural components supporting green roofs, a roof-top garden, or other similar rooftop sustainability elements; and

      - Exterior residential amenity areas, rooftop terraces and all associated components such as railings, guards and furniture.

### Projections into Required Setbacks

#### Architectural Features in the DT1, DT2, HCD2 and HB Zones

    - In the DT1, DT2, HCD2 and HB Zones, setback provisions do not apply to fire escapes, canopies and awnings above doors and windows, bay windows, belt courses, sills, cornices, eaves and chimneys, which are subject to the following provisions:

#### Fire Escapes

      - The maximum horizontal projection of fire escapes towards the interior lot line and exterior lot line is 1.2 metres from the foundation wall;

      - The minimum interior setback and exterior setback for fire escapes is 0.8 metres; and

      - The maximum horizontal projection of fire escapes towards the rear lot line is

1.5 metres.

#### Bay Windows

      - The maximum horizontal projection of a bay window toward the front lot line, exterior lot line and rear lot line is 1.2 metres from the foundation wall;

      - The minimum required front setback, exterior setback and rear setback for a

bay window is 0.8 metres;

      - The maximum horizontal projection of a bay window toward an interior lot line is 0.8 metres from the foundation wall; and

      - The minimum interior setback of a bay window is 0.8 metres.

#### Belt Courses, Sills, Cornices and Eaves

      - The maximum horizontal projection of belt courses, sills, cornices, and eaves toward the interior lot line or exterior lot line is 0.5 metres from the foundation wall; and

      - The minimum front setback and rear setback of belt courses, sills, cornices

and eaves is 0.8 metres.

#### Chimneys

      - The maximum horizontal projection of a chimney and its foundation, if any, toward any lot line is 0.3 metres from a foundation wall of the adjacent exterior wall of the principal building.

#### Architectural Features in Other Zones

    - In all Zones except the DT1, DT2, HCD2 and HB Zones, despite anything to the contrary in this By-law, sills, belt courses, chimneys, fireplace projections, cornices, eaves, gutters, parapets, pilasters or similar ornamental architectural features may project into any required setback a maximum distance of 0.5 metres, provided such feature is setback a minimum of 0.5 metres to any lot line.

#### Exterior Stairs in Other Zones

    - In all Zones except the DT1, DT2, HCD2 and HB Zones, despite anything to the contrary in this by-law, exterior stairs including their associated guards and any landing areas that are required by the Building Code Act, 1992 are permitted to project into any required setback, provided such exterior stairs are setback a minimum of 0.5 metres to any lot line.

(By-Law Number 2022-62; 2024-332)

### Decks, Porches and Balconies

#### Balconies

    - Despite the setback provisions of this By-law, balconies that project from the main wall of an apartment building or mixed use building must comply with the following provisions:

      - The maximum horizontal projection from the main wall is 2.0 metres;

      - A maximum of 45% of the horizontal length of each face of the main wall of each storey may be occupied by balconies; and

      - The minimum setback from a lot line is 1.0 metre.

(By-Law Number 2022-62; 2024-332)

    - Despite the setback provisions of this By-law, balconies that project from the main wall of a house, semi-detached house, townhouse or stacked townhouse must comply with the following provisions:

      - The maximum horizontal projection from the main wall is 2.0 metres; and

      - The minimum front setback, rear setback, interior setback and exterior setback, must comply with the provisions that apply to decks greater than 1.2 metres in height in Table 4.20.4.

(By-Law Number 2022-62; 2024-332)

    - Despite Clauses 4.20.1. and 4.20.2., where a balcony is wholly enclosed and is covered with a roof, it must comply with the provisions that apply to the principal building.

#### Decks and Porches

    - In an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, HAM Zone, RUR Zone, LSR Zone or DR Zone, a deck or porch are not subject to the required setbacks that apply to the principal building. A deck or porch including its associated guards and exterior stairs, must:

      - Not be located within any form of drainage management system such as a

swale or ditch;

      - Comply with the provisions of Subsection 4.23. regulating separation distances from waterbodies; and

      - Comply with the provisions of Table 4.20.4.

#### Table 4.20.4. – Decks and Porches

| Zoning Provision | Height no greater than 0.6 metres | Height greater than 0.6 metres up to 1.2 metres | Height greater than 1.2 metres |
| --- | --- | --- | --- |
| 1. Maximum surface area of floor level(s) excluding the area of the exterior stairs | 10% of the lot area | 10% of the lot area | 10% of the lot area, of which a maximum of 30.0 square metres is permitted at a height of 1.2 metres or greater |
| 2. Minimum front setback | The lesser of the Zone provision or 3.5 metres | The lesser of the Zone provision or 3.5 metres | Comply with Zone provision |
| 3. Minimum interior setback | (a) semi-detached house and townhouse: 0.6 metres, except along a common party wall where it may be 0.0 metres if there is a common privacy fence a minimum of 1.5 metres tall | (a) semi-detached house and townhouse: Comply with Zone provision, except along a common party wall where it may be 0.0 metres if there is a common privacy fence a minimum of 1.5 metres tall | (a) semi-detached house and townhouse: Comply with Zone provision, except along a common party wall where it may be 0.0 metres if there is a common privacy fence a minimum of 1.5 metres tall |

| Zoning Provision | Height no greater than 0.6 metres | Height greater than 0.6 metres up to 1.2 metres | Height greater than 1.2 metres |
| --- | --- | --- | --- |
|  | (b) all other uses: 0.6 metres | (b) all other uses: Comply with Zone provision | (b) all other uses: Comply with Zone provision |
| 4. Minimum exterior setback | The lesser of the Zone provision or 3.5 metres | The lesser of the Zone provision or 3.5 metres | Comply with Zone provision |
| 5. Minimum rear setback | 2.0 metres | 2.0 metres | 4.0 metres |

(By-Law Number 2022-62; 2024-332)

    - In all Zones other than those regulated by Clause 4.20.4., decks and porches must comply with the provisions that apply to the principal building.

    - For the purpose of Clause 4.20.4., the maximum surface area of the floor level is calculated based on the cumulative floor levels of all decks and porches located on a lot.

    - For the purpose of Clause 4.20.4., decks and porches that have a perimeter foundation are excluded from the calculation of maximum surface area.

    - For the purpose of Clause 4.20.4., the height of a deck and porch means the vertical distance measured from finished grade to the walking surface, excluding the height of any safety railing, guard, privacy screen or other similar feature. Finished grade is the average of the four most distant points representing the outermost corners of the deck and porch.

#### Terraces

    - The provisions of Clauses 4.20.1. to 4.20.8. do not apply to a terrace which is located on the roof of a building above a storey. Rooftop terraces are permitted on the roof of a building and must comply with the Projections Above Maximum Height provisions of Subsection 4.18.

### Barrier-Free Buildings

    - In all Zones except the DT1, DT2, HCD2 and HB Zones, despite any setback that is more restrictive in this By-law, unenclosed building components necessary to ensure that a building and its facilities can be approached, entered, and used by

persons with disabilities in accordance with the Building Code Act, 1992, may project into any required front setback, interior setback or exterior setback provided that the building component is no closer than 0.3 metres from any lot line and is not located within any form of drainage management system such as a swale or ditch. The area of such barrier-free building component is excluded from the calculation of lot coverage.

(By-Law Number 2022-62; 2024-332)

### Railway and Rail Yard Separation Distances

    - Where a lot is adjacent to a principal main line railway right-of-way and such lot is

developed with a permitted use or building:

      - The minimum required setback from the lot line adjacent to such right-of- way to a building containing a sensitive use is:

        - 30 metres, where a minimum 2.5 metre high landscaped berm is provided along the lot line adjacent to the right-of-way; or

        - 120 metres.

      - A minimum 1.83 metre high chain link security fence must be provided along the lot line adjacent to the right-of-way for the entire length of the lot line, to be installed and maintained at the owner’s expense.

    - Where a lot is adjacent to a secondary main line railway right-of-way and such lot

is developed with a permitted use or building:

      - The minimum required setback from the lot line adjacent to such right-of- way to a building containing a sensitive use is:

        - 30 metres, where a minimum 2.0 metre high landscaped berm is provided along the lot line adjacent to the right-of-way; or

        - 120 metres.

      - A minimum 1.83 metre high chain link security fence must be provided along the lot line adjacent to the right-of-way for the entire length of the lot line, to be installed and maintained at the owner’s expense.

    - Where a lot is adjacent to a principal branch line, secondary branch line or spur line railway right-of-way and such lot is developed with a permitted use or building:

      - The minimum required setback from the lot line adjacent to such right-of- way to a building containing a sensitive use is:

        - 15 metres, where a minimum 2.0 metre high landscaped berm is provided along the lot line adjacent to the right-of-way; or

        - 120 metres.

      - A minimum 1.83 metre high chain link security fence must be provided along the lot line adjacent to the right-of-way for the entire length of the lot line, to be installed and maintained at the owner’s expense.

    - Where a lot is not adjacent to such railway right-of-way, but is located in an area where a building with a sensitive use can be constructed within 120 metres of a railway right-of-way, Subclauses 4.22.1.1., 4.22.2.1. and 4.22.3.1. apply when such lot is developed with a permitted use or building.

    - The minimum separation distance between a sensitive use and a rail yard is 300 metres.

### Waterbody Separation Distances

    - The minimum separation distance from the high water mark of a waterbody to any use or building, including any part of a private sewage system, is 30.0 metres, except for:

      - Vegetated buffer to enhance water quality, minimize soil erosion, provide plant and animal habitat, establish connectivity and wildlife corridors, and contribute to the overall health of shoreline ecosystems;

      - Vegetated buffer which screens views of development or creates natural spaces for passive recreation;

      - Parks or conservation areas that provide controlled access to the waterbody;

      - Public trail systems and interpretive signage;

#### Marine facilities;

      - Marinas, where they are permitted in the Zone applicable to the lot, excluding detached accessory buildings with accessory uses;

      - Agricultural uses existing as of the date of passing of this By-law;

      - Shoreline stabilization works; and

      - Utilities infrastructure including stormwater outfall, water treatment and pumping facilities and combined sewer overflow management facilities.

    - It is intended that additional uses or buildings may be authorized through a minor variance in accordance with the policies of the Official Plan and subject to, and in accordance with, the provisions of the Planning Act.

### Pipeline Separation Distances

    - The minimum setback from a lot line adjacent to land that contains a pipeline is:

      - 30.0 metres for an elementary school and secondary school;

      - 7.0 metres for any other type of principal building;

      - 7.0 metres for driveways, drive aisles, parking spaces, parking lots, retaining walls, light standards and utility poles; and

      - 3.0 metres for any accessory building, swimming pool, deck, patio and

#### porch.

    - The minimum separation distance from a registered right-of-way for a pipeline is:

      - 30.0 metres for an elementary school and secondary school;

      - 7.0 metres for any other type of principal building;

      - 7.0 metres for driveways, drive aisles, parking spaces, parking lots, retaining walls, light standards and utility poles; and

      - 3.0 metres for any accessory building, swimming pool, deck, patio and

porch.

### Waste and Recycling

    - Waste and recycling must not be stored on any lot in any Zone except within the principal building or an accessory building on such lot or in a container located in the interior yard or rear yard of such lot.

    - A waste or recycling storage area, including any loading or unloading area, which is visible from an adjoining site in an Urban Residential Zone, Urban Multi-Residential

Zone, HCD1 Zone, HCD3 Zone, CN, WM1, WM2, CA, CD, CR, CG, CW, HB, OS1 or

OS2 Zone, a public open space, a waterbody or a street must have a visual screen at a minimum height that is equal to any bins or dumpsters stored within the waste or recycling storage area.

(By-Law Number 2022-62; 2024-332)

    - A commercial waste or recycling dumpster or bin is permitted in an interior yard, exterior yard or rear yard, and must be setback a minimum of 1.2 metres from a lot line.

(By-Law Number 2022-62; 2024-332)

    - For residential buildings with five to ten dwelling units, a dedicated waste and recycling storage area must be provided inside a building or within the interior yard or rear yard, with a minimum area of at least 5 square metres.

(By-Law Number 2022-62; 2024-332)

### Mineral Aggregate Operation Separation Distance for All Residential Uses

    - A minimum separation distance of 300.0 metres is required between a residential use and a lot with a gravel pit in an MX1 Zone.

    - A minimum separation distance of 500.0 metres is required between a residential use and a lot with a mineral aggregate operation in an MX1 Zone.

    - Where a residential use, gravel pit or mineral aggregate operation existed as of the date of passing of this By-law and does not comply with Clauses 4.26.1. or 4.26.2., such use is deemed to conform with this By-law.

### Walkways for All Residential Uses

    - All dwelling units, including those located in an accessory house, detached building with an additional rural unit, house, semi-detached house, townhouse, stacked townhouse, apartment building or mixed use building must be accessed by a walkway that complies with the following provisions:

      - The minimum width of the walkway is 1.1 metres wide;

      - In the urban area, the walkway must be provided from a street line to the main exterior entrance, or to stairs leading to the main exterior entrance, of every dwelling unit on a lot. Where dwelling units share a common main

exterior entrance, the walkway must be provided to the common main exterior entrance;

      - In the rural area, the walkway must be provided from the driveway containing the parking space for the dwelling unit to the exterior entrance, or to stairs leading to an exterior entrance, of every dwelling unit on a lot;

      - The walkway must be separately delineated and measured distinctly from a required driveway and parking space through a change in surface materials or line painting or other similar mechanism. A vehicle must not park on top of any part of the walkway; and

      - The walkway must be unobstructed up to a minimum height of 2.1 metres above grade with the exception of the following features that are permitted to project as localized protrusions into the walkway:

        - Hydro or gas meters;

        - Window sills, chimneys, fireplace projections, cornices, gutters, pilasters or similar architectural features.

(By-Law Number 2022-62; 2024-332)

    - Clause 4.27.1. does not prevent the establishment of a gate across a walkway to access an exterior entrance through a fenced yard.

    - When a building with a dwelling unit has legal non-complying status in accordance with Subsection 1.8. due to an insufficient walkway, this Subsection is not interpreted to require that the deficiency be made up prior to the renovation of the existing non-complying building. However further development which has the effect of adding one or more bedrooms or dwelling units is not permitted unless Clause 4.27.1. is complied with.

    - Where a house contains a dwelling unit as of the date of passing of this By-law, which wasn’t a permitted use in the former zoning by-laws when such unit was developed, and such dwelling unit does not comply with 4.27.1., the dwelling unit is deemed to be legal non-complying insofar as the walkway requirements of

4.27.1. only, subject to and in accordance with Clause 4.27.3.

(By-Law Number 2022-62; 2024-332)

    - Despite Subclause 4.27.1.4., where a lot was developed with a residential building and driveway as of the date of passing of this By-law, where the lot is developed in a manner that maintains, renovates and/or adds floor area to the existing

residential building and has the effect of adding additional permitted bedrooms and/or dwelling units to the lot (either in the principal building or as an accessory house):

      - The walkway and driveway are permitted to overlap if:

        - The portion of the driveway where the walkway overlaps does not contain any parking spaces; and

        - The 1.1 metre wide walkway must be clearly delineated and distinct from the driveway through a change in surface materials or line painting or other similar mechanism; and

        - A sign must be provided that clearly indicates that vehicles must use caution and yield to pedestrians using the walkway. It must also state that parking is prohibited on top of the walkway; and

      - Portions of a walkway are permitted to be less than 1.1 metres wide where the

walkway is provided in an existing yard that is less than 1.1 metres in width if:

        - the walkway is designed in a manner that provides the greatest possible

walkway width based on the dimensions of the existing yard; and

        - the walkway is a minimum of 0.8 metres wide at the narrowest point; and

        - all portions of the walkway outside of the existing yard are a minimum of 1.1 metres wide.

(By-Law Number 2022-62; 2024-332)

### Maximum Number of Bedrooms

    - A maximum of 8 bedrooms are permitted per lot, in the aggregate, on:

      - Any lot in the Urban Residential Zones, DR Zone and the HCD1 Zone; and

      - A lot in the Urban Multi-Residential Zone or the HCD3 Zone where there are 2 or less principal dwelling units.

    - Despite Clause 4.28.1., where the ”Fourth Residential Unit Holding Area” established in Clause 5.4.5. is removed from a lot, a maximum of 12 bedrooms are permitted per lot, in the aggregate, with the exception of lots located within the area bounded by Sir John A. Macdonald Boulevard to the West, Bath Road and Concession Street to the North, Division Street to the East, and Johnson Street to the South and the

area bounded by Sir John A. Macdonald Boulevard to the West, Johnson Street to the North and East until Ontario Street, the North side of Ontario Street until Gore and the West portion of Gore until Lake Ontario and the HCD1 and HCD3 zones, where the maximum number of bedrooms is 10 bedrooms per lot in the aggregate for all lots developed with 4 dwelling units in a house, semi-detached house, townhouse and/or accessory house.

(By-Law Number 2022-62; 2024-333)

### Accessory Houses in the Urban Area

    - In an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, IN2 Zone or DR Zone, an accessory house is permitted as an accessory use to a house, semi-detached house, townhouse, or a place of worship.

    - Accessory houses are exempt from the provisions of Subsection 4.1. but must comply with all other applicable provisions of this By-law including, but not limited to, the walkway provisions of Subsection 4.27., the bedroom provisions of Subsection 4.28., the parking space and bike space provisions of Section 7., and the housing constraint areas in Subsection 5.4.

    - Accessory houses are exempt from provisions that:

      - establish the maximum density as a measure of dwelling units per net hectare; and

      - establish the minimum lot area per dwelling unit on a lot.

#### Accessory House Provisions

    - Where permitted, an accessory house must comply with the following requirements:

      - Accessory houses are only permitted on a lot with three or less dwelling units in the principal building.

      - A maximum of one dwelling unit is permitted in an accessory house.

      - An accessory house must be located within a rear yard or interior yard and must comply with the following provisions:

        - Minimum rear setback of 1.2 metres;

        - Minimum interior setback of 1.2 metres;

        - Minimum front setback and exterior setback of the applicable Zone;

        - Maximum lot coverage of all accessory buildings on a lot is 10% in the aggregate;

        - Maximum height of 4.6 metres; and

        - Maximum height of 1 storey.

      - The rear yard or interior yard must be screened with a privacy fence with a minimum height of 1.8 metres as follows:

        - When the accessory house is located in a rear yard, the privacy fence must be established along all interior lot lines and rear lot lines adjacent to the rear yard;

        - When the accessory house is located in an interior yard, the privacy fence must be established along the interior lot line closest to the accessory house extending from the intersection of the interior lot line with the rear lot line to the intersection of the interior lot line with the required front setback; or

        - When the accessory house is located in both a rear yard and an interior yard, the privacy fence must be established in accordance with Subclauses (a) and (b).

      - Where a lot is created through a plan of subdivision under the Planning Act or a description under the Condominium Act, 1998 following the date of passing of this By-law, an accessory house must comply with the provisions of Subclauses 4.29.4.5. and 4.29.4.6., except:

        - Accessory houses that are accessed by a private street or public laneway adjacent to the rear lot line must comply with the following provisions:

          - the minimum interior setback is 0.0 metres;

          - the maximum height is the lesser of 7.5 metres or 2 storeys;

          - a privacy fence with a minimum height of 1.8 metres must be established along all interior lot lines adjacent to the rear yard and interior yard. No privacy fence is required along the rear lot line; and

          - despite Clause 4.27.1.4., the walkway requirements of Subsection

4.27. may be satisfied through the provision of an unobstructed 6.0 metre wide private street or public laneway connected to a walkway on the lot.

#### Legal Non-Compliance

      - Where a principal building is legal non-complying in accordance with Subsection 1.8., an accessory house is permitted to be developed on the lot if it complies with all applicable provisions of this Subsection and all other applicable provisions of this By-law.

      - In an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, RUR Zone, RU Zone, LSR Zone or DR Zone a private garage that existed on the date of passing of this By-law is permitted to be converted into an accessory house without any zoning relief required related to the location and size of the private garage on the lot as long as it complies with all other provisions of this Subsection that are not related to the location and size of the private garage and all other applicable provisions of this By-law.

(By-Law Number 2022-62; 2024-332)

### Additional Rural Units in the Rural Area

    - In the AG, RU, RUR, LSR and HAM Zones, additional rural units are permitted only as an accessory use to a house, semi-detached house, or place of worship, if such building is listed as a permitted use in the applicable Zone and in accordance with this Subsection.

    - Additional rural units are exempt from the provisions of Subsection 4.1. but must comply with all other applicable provisions of this By-law including, but not limited to, the walkway provisions of Subsection 4.27., the parking space and bike space provisions of Section 7. and the housing constraint areas in Subsection 5.4.

    - Additional rural units are exempt from provisions that:

      - establish the maximum density as a measure of dwelling units per net hectare; and

      - establish the minimum lot area per dwelling unit on a lot.

(By-Law Number 2022-62; 2024-332)

#### General Provisions for Attached and Detached Additional Rural Units

    - Where permitted, additional rural units must comply with the following provisions:

      - Additional rural units are not permitted on a lot containing two or more

#### principal dwelling units.

      - Additional rural units must be connected to private services to the satisfaction of the City.

      - A maximum of two additional rural units are permitted per lot including a maximum of one second rural unit and a maximum of one third rural unit.

      - Where two additional rural units are located on one lot:

        - A maximum of one detached additional rural unit is permitted; and

        - A maximum of one additional rural unit may be attached to or located within the principal building.

#### Attached Additional Rural Unit Provisions

      - An additional rural unit that is attached to the principal building or located within the principal building must comply with all provisions of Clause 4.30.4. and must comply with all provisions applicable to the principal building.

#### Detached Additional Rural Unit Provisions

      - In addition to meeting all provisions of Clause 4.30.4., an additional rural unit in a detached accessory building must be located within a rear yard or interior yard and the accessory building must comply with the following provisions:

        - Minimum rear setback of 1.2 metres;

        - Minimum interior setback of 1.2 metres;

        - Minimum front setback and exterior setback of the applicable Zone;

        - Maximum lot coverage of all accessory buildings on a lot is 10% in the aggregate;

        - Maximum height of 4.6 metres; and

        - Maximum height of 1 storey.

#### Legal Non-Compliance

      - Where a principal building is legal non-complying in accordance with Subsection 1.8., an additional rural unit is permitted within the existing principal building if the development of the additional rural unit complies with all other standards this By-law including, but not limited to, the walkway provisions of Subsection 4.27., the parking space and bike space provisions of Section 7. and the housing constraint areas in Subsection 5.4.

      - Where a principal building is legal non-complying in accordance with Subsection 1.8., an additional rural unit is permitted to be attached to the principal building if any new development associated with the additional rural unit complies with this By-law.

      - Where a principal building is legal non-complying in accordance with Subsection 1.8., an additional rural unit is permitted in a detached accessory building if the detached accessory building complies with this By-law.

(By-Law Number 2022-62; 2024-332)

### Highway 401 Separation Distance

    - Where a lot is adjacent to the Highway 401 right-of-way and such lot is developed with a permitted use or building, the minimum required separation distance from a use or building to the Highway 401 right-of-way is 14.0 metres.

    - Where a lot is not adjacent to the Highway 401 right-of-way, but is located in an area where a use or building can be developed within 14.0 metres of the Highway 401 right-of-way, Clause 4.31.1. applies.

### Separation Distances between Sensitive Uses and Industrial Facilities

    - Despite anything to the contrary in this By-law, the establishment of a new sensitive use must comply with the following minimum separation distances from a Class 1, Class 2 or Class 3 industrial facility, as applicable, located within an Employment Zone. Despite clause 3.19.7., this separation distance must be measured as per the province’s D-series Environmental Land Use Compatibility Guidelines, or any successor thereof:

      - Minimum separation distance from a Class 1 industrial facility located within an Employment Zone: 20 metres;

      - Minimum separation distance from a Class 2 industrial facility located within an Employment Zone: 70 metres; and

      - Minimum separation distance from a Class 3 industrial facility located within an Employment Zone: 300 metres

    - Despite clause 4.32.1, any sensitive use within the required separation distance existing as of the date of passing of By-law Number 2025-118, is considered to comply with this provision and may be expanded as long as it does not further increase the extent or degree of non-compliance with clause 4.32.1.

(By-Law Number 2022-62; 2024-332; 2025-118)

# Section 5:	Overlay Provisions

### Floodplain Overlay

    - No use or building is permitted within the area identified as “Floodplain” or “Wave Uprush” on Schedule A for any purpose other than the following uses, where the use is permitted by the underlying Zone:

      - Agricultural use, excluding a building;

      - Conservation use, excluding a building;

      - Forestry use, excluding a building;

      - Marine facility;

      - Marina;

      - Park;

      - Public use; and

      - Utility installations that are subject to the requirements of the Environmental Assessment Act, R.S.O. 1990, c. E.18.

    - Despite Clause 5.1.1., where a lot is located within the area identified as the “Wave Uprush” area on Schedule A, uses existing as of the date of passing of this By-law are permitted and Clauses 1.8.2., 1.8.3. and 1.8.4. apply to the expansion of legal non-complying buildings and accessory buildings in connection with these permitted uses.

    - The Floodplain Overlay on Schedule A may be adjusted as a technical revision in accordance with Clause 1.10.2., where such adjustment lessens the extent of the floodplain in a manner that is supported by a technical assessment to the satisfaction of the Cataraqui Region Conservation Authority and the Director.

### Source Water Protection Overlay

#### Cana Wellhead Protection Area

    - The area identified as “Cana Wellhead Protection Area” on Schedule B is located in a Wellhead Protection Area identified by the Cataraqui Source Protection Plan. The lands within the “Cana Wellhead Protection Area” are subject to all applicable provisions of this By-law, with the exception that any activity, use or undertaking that is prohibited or regulated under Section 59 of the Clean Water Act is not permitted except in accordance with the Clean Water Act.

#### Intake Protection Zone

    - The areas identified as “Intake Protection Zone” on Schedule B are located in an Intake Protection Zone identified by the Cataraqui Source Protection Plan. The lands within the “Intake Protection Zone” are subject to all applicable provisions of this By-law, with the exception that any activity, use or undertaking that is prohibited or regulated under Section 59 of the Clean Water Act is not permitted except in accordance with the Clean Water Act.

### Airport Noise Exposure Overlay

    - Despite anything to the contrary in this By-law, sensitive uses are not permitted within the area above 30 NEF as shown on Schedule C.

### Housing Constraint Area Overlays

    - Schedule D1, Urban Constraint Areas and Second Rural Unit Holding Overlay, Schedule D2, Third Rural Unit Holding Overlay and Schedule D3, Fourth Urban Residential Unit Holding Overlay, have the effect of restricting the development or use of a lot or building in accordance with the provisions of this Subsection.

(By-Law Number 2022-62; 2024-332; 2024-333)

    - Additional rural units, accessory houses and the construction of new dwelling units beyond what legally existed on the date of passing of this By-law;

      - On lands identified as “Rural Constraint Area – Loughborough Lake (At- Capacity Lake)” on Schedule D1;

      - Within a basement or basement storey on lands identified as “Urban Constraint Area – Sewer Surcharging (Combined Storm and Sewer)” on Schedule D1. For clarity, this provision only restricts the entirety of a dwelling unit from being located within a basement or basement storey – habitable floor area of a dwelling unit is permitted in a basement or basement storey when a habitable room of such dwelling unit is located at or above the first storey;

      - On lands identified as “Urban Constraint Area – Sewer Capacity” on Schedule

D1; or

      - Within a basement or basement storey on lands identified as “Urban Constraint Area – Sewer Surcharging” on Schedule D1. For clarity, this provision only restricts the entirety of a dwelling unit from being located within a basement or basement storey – habitable floor area of a dwelling unit is permitted in a basement or basement storey when a habitable room of such dwelling unit is located at or above the first storey.

(By-Law Number 2022-62; 2024-332)

    - In accordance with Clause 2.6.2., a Holding Overlay has been established in the area identified as “Water Supply/Water Quality” on Schedules D1 and D2. The “Second Rural Unit– Water Supply/Water Quality” on Schedule D1 only applies to the development of a second rural unit and the “Third Rural Unit– Water

Supply/Water Quality” on Schedule D2 only applies to the development of a third rural unit. Prior to the removal of any lot from a Holding Overlay and the issuance of a building permit for an additional rural unit, the following conditions must be satisfied:

      - The following conditions apply to an additional rural unit that is attached to the principal building and connects to the private services of the principal building:

        - A letter of opinion to the satisfaction of the City’s Environment Director (or designate) from an independent, qualified professional must be submitted stating that the private water supply is sufficient to support the additional rural unit in combination with the normal operation of the principal dwelling on the lot. The qualified professional must hold a valid licence to practice in Ontario as either an engineer (P.Eng.) or geoscientist (P.Geo). The letter must be signed by the qualified professional and must demonstrate how the supply well will support the increased demand required by the additional rural unit while ensuring that neighbouring wells are not adversely impacted. In addition, the qualified professional must include a statement that any water quality treatment systems in place at the time of review are sufficient in terms of design, maintenance and condition to safely service the proposed additional rural unit in combination with the existing principal dwelling unit; and

        - Approval of the private sewage system must be obtained from the City

or applicable approval authority.

      - The following conditions apply to a detached additional rural unit or an additional rural unit that is attached to the principal building and is not connecting to existing private services:

        - A Hydrogeological Study is to be completed to the satisfaction of the City’s Environment Director (or designate) from an independent qualified professional (P.Eng.) or geoscientist (P.Geo). to determine that the groundwater quality and quantity is sufficient for the additional rural unit and will not adversely impact the water supply of adjacent lots and the principal dwelling unit. The Hydrogeological Study must be completed in accordance with the City’s Standard for Hydrogeological Assessments. Adjustments to the requirements of a full hydrogeological study to demonstrate the suitability of private water supply may be considered by the City’s Environment Director (or designate). The Hydrogeological Study must also assess sewage system impact and demonstrate that:

          - The area of development is not hydrogeologically sensitive; and

          - The private sewage system is isolated from the receiving aquifer, or the impact of the principal dwelling unit plus the additional rural unit is less than 10 milligrams per litre nitrate-nitrogen at the property boundary.

        - Approval of the private sewage system must be obtained from the City

or applicable approval authority.

(By-Law Number 2022-62; 2024-332)

    - In accordance with Clause 2.6.2., a Holding Overlay has been established in the area identified as “Servicing Capacity (Cana Subdivision)” on Schedules D1 and D2. The “Second Rural Unit– Servicing Capacity (Cana Subdivision)” on Schedule D1 only applies to the development of a second rural unit and the “Third Rural Unit– Servicing Capacity (Cana Subdivision)” on Schedule D2 only applies to the development of a third rural unit. Prior to the removal of any lot from a Holding Overlay and the issuance of a building permit for an additional rural unit, the following conditions must be satisfied:

      - A letter of opinion has been provided from a qualified professional confirming that water and/or wastewater capacity issues will not be experienced on the lot as a result of the establishment of an additional rural unit, to the satisfaction of Utilities Kingston.

(By-Law Number 2022-62; 2024-332)

    - In accordance with Clause 2.6.2., a Holding Overlay has been established in the area identified as "Fourth Residential Unit Holding Area" on Schedule D3 and only applies to the development of a fourth dwelling unit in a house, semi-detached house, townhouse or accessory house on one lot in chronological order of development of the lot. Prior to the removal of any lot from the Holding Overlay and the issuance of a building permit for the fourth dwelling unit in a house, semi- detached house, townhouse or accessory house on one lot, the following conditions must be satisfied:

      - The City is satisfied that there is adequate servicing capacity (i.e., water, wastewater, natural gas, and electrical) for the proposed development;

      - All necessary studies, as determined by the City, have been completed and accepted by the City. Required studied may include but are not limited to studies related to servicing capacity, transportation impact, parking, soil, nosid, natural heritage features, archaeological, environmental constraints or a Record of Site Conditions.

Note: There are limitations in the number of plumbing fixtures that are permitted under the Ontario Building Code that are dependent on the size of the water service to a property. Many areas of the City have a water service size that will limit the number of plumbing fixtures. Applicants are encouraged to contact the City early in the design process to determine the number of plumbing fixtures that will be permitted.

(By-Law Number 2022-62; 2024-333)

### Exception Overlay

    - The Exception Overlay on Schedule E has the effect of restricting the development or use of a lot or building in accordance with the provisions of the corresponding Legacy Exception in Section 20 or Exception in Section 21. Unless specifically amended by the Exception, all provisions of this By-law continue to apply.

    - Despite Clauses 1.5.1. and 1.5.2., the applicable former zoning by-laws apply to assist in the interpretation of any Legacy Exception provision in Section 20 originally enacted under the applicable former zoning by-law. Where a provision of this By- law is more restrictive than the applicable former zoning by-law in respect of the interpretation of any Legacy Exception, the provision of the former zoning by-law that was in force as of the date of passing of this By-law prevails and may be relied upon for the issuance of a building permit where the building permit is consistent with the content of the rezoning application that resulted in the approval of such Legacy Exception.

    - The Legacy Exceptions in Section 20 are drafted in the language of the applicable former zoning by-law, which may not be consistent with the language used in the balance of this By-law. Terminology contained in Section 20 is specific to the particular Legacy Exception and does not apply to the balance of this By-law. Uses referenced in Section 20 and not referenced elsewhere in this By-law are not to be construed as distinct from those uses referred to in Sections 1 through 19 or Sections 21 through 24.

    - Where a Legacy Exception in Section 20 or Exception in Section 21 refer to “the site specific by-law”, the intent is to refence the implementing zoning by-law that established the Legacy Exception under the applicable former zoning by-law or Exception under this By-Law, as applicable.

    - Despite Clause 5.5.2., where a Legacy Exception only permits a select range of building types that are considered to be a house, semi-detached house or townhouse by this By-law, such Legacy Exception is interpreted to allow a house, semi-detached house or townhouse subject to the maximum number of dwelling units identified in the applicable Zone, of which a maximum of one dwelling unit is permitted in an accessory house or as a detached additional rural unit (as applicable), and subject to compliance with all provisions of this By-law including all Housing Constraint Area Overlays in Subsection 5.4.

(By-Law Number 2022-62; 2024-332)

### Holding Overlay

    - In accordance with Clause 2.6.2., a Holding Overlay has been established as Schedule F which has the effect of restricting the development of a lot in accordance with the corresponding Holding Conditions in Section 22.

    - A lot that is subject to a Holding Overlay may only be developed for the following purposes until such time as the Holding Overlay is removed:

      - Uses that legally existed on the date of the passing of this By-law;

      - A model home in accordance with Clause 6.6.1.

    - Despite Clause 5.6.2., where the corresponding Holding Condition in Section 22 states that it only applies to specific development, all other permitted development may proceed prior to the removal of the Holding Overlay.

    - The Holding Overlay may be partially removed from any lot, either through revisions to the extent of lands covered by the Holding Overlay or by revisions to the language of the Holding Conditions in Section 22.

    - A building that legally existed as of the date of passing of this By-law on a lot that is subject to a Holding Overlay may be renovated or altered if such renovation or alteration does not:

      - Increase the height, size or volume of the building;

      - Have the effect of adding one or more bedrooms or dwelling units; or

      - Change the use.

### Non-Residential Conversion Overlay

    - All existing buildings used, either in part or as a whole, for non-residential uses on a lot identified as “Non-Residential Conversion Area” on Schedule G of this By-law are permitted to be converted to a mixed use building or an apartment building through the construction of one or more dwelling units.

    - The conversion of existing buildings permitted by Clause 5.7.1. does not require any zoning relief related to the location and size of the existing building and is exempt from Subsection 4.3. requiring amenity area and Subsection 4.16. requiring planting strips, but must comply with all other applicable provisions of this By-law.

    - In accordance with Clause 2.6.2, a Holding Overlay has been established in the area identified as “Non-Residential Conversion Area – Holding Overlay” on Schedule G. Prior to the removal of any lot from a Holding Overlay and the issuance of a building permit for a conversion in accordance with Clauses 5.7.1. and 5.7.2., the following conditions must be satisfied:

- The City is satisfied that there is adequate servicing capacity (i.e. water, wastewater, natural gas, and electrical) for the proposed development;

- All necessary studies, as determined by the City through the pre- consultation process, have been completed and accepted by the City. Required studies may include but are not limited to studies related to servicing capacity, transportation impact, parking, soil, noise, natural heritage features, archeological assessments, heritage impact assessments, environmental constraints or a Record of Site Condition; and

- All agreements required by the City through the pre-consultation process, including site plan control, have been executed and registered on title, as appropriate.

### Express Transit Area Overlay

    - All lots with a street line along a street identified as an Express Transit route subject to an identified height (in storeys and metres) on Schedule H of this By-law are intended to be developed with an apartment building or mixed use building through a minor variance application in accordance with Section 45 of the Planning Act and the Express Transit Area policies of the Official Plan.

    - Where an apartment building or mixed use building is permitted through a minor variance in accordance with Clause 5.8.1., the provisions of Sections 8 to 21 of this By-law do not apply, and instead the performance standards in Table 5.8.2. and Clauses 5.8.3. and 5.8.4. apply. All other applicable provisions of this By-law must be complied with. It is the intent of this By-law to allow for minor variances to the performance standards where necessary due to the unique configuration, layout or location of a lot in accordance with the policies of the Official Plan.

#### Table 5.8.2. – Express Transit Area Performance Standards

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |

| Zoning Provision | all permitted uses |
| --- | --- |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres and storeys) | The maximum height is the lesser of the number of storeys or height in metres identified along the street line of the lot on Schedule H of this By-law |
| 4. Minimum and maximum front setback (metres) | To be determined through a minor variance application |
| 5. Minimum rear setback (metres) | To be determined through a minor variance application |
| 6. Minimum exterior setback (metres) | To be determined through a minor variance application |
| 7. Minimum interior setback (metres) | To be determined through a minor variance application |
| 8. Minimum stepbacks | (a) for lots where the maximum height is greater than 6 storeys, the minimum stepbacks required along the front lot line and/or exterior lot line: (i) storeys 7 and above: minimum 3.0 metres from the exterior wall of the 6th storey (b) for all other lots: N/A |
| 9. Minimum landscaped open space | 10% |
| 10. Maximum lot coverage | To be determined through minor variance application |
| 11. Minimum floor to floor height of first storey | 4.5 metres |

    - Despite the definitions of front lot line and rear lot line, for the purpose of this Subsection, the front lot line is the street line along a street identified as “Express Transit”. If a lot has more than one street line along a street identified as “Express Transit”, then the shorter lot line is considered the front lot line.

    - In addition to the provisions of Table 5.8.2., apartment buildings or mixed use buildings permitted through a minor variance must comply with the following provisions:

      - The minimum separation distance between a building and a lot located in a UR Zone is:

        - For storeys 1 to 4: 10 metres;

        - For storeys 5 to 6: 15 metres; and

        - For storeys 7 and above: 20 metres.

      - The main pedestrian entrance to the building must be located on the main wall facing the front lot line.

      - A minimum of 50% of the front yard and exterior yard must be soft landscaping that allows water to permeate into the ground (vegetation such as trees, shrubs, hedges, ornamental plantings, grass, groundcover or other similar plantings).

      - Loading spaces, parking spaces and parking lots are not permitted in a

#### front yard or exterior yard.

      - Parking structures are not permitted to occupy any part of a main wall facing a street line.

      - At the second storey and above, the maximum horizontal length of one continuous plane of a main wall of a building facing the front lot line and exterior lot line is 30 metres before articulation is provided. Articulation must be provided in the form of relief for a minimum horizontal length of 6 metres along the main wall at a minimum depth of 2 metres from the face of the main wall.

      - For buildings greater than 10 storeys in height, the maximum floor plate of all storeys above the 6th storey is 800 square metres. Floor plate includes all areas enclosed within exterior walls, including hallways, elevators, stairs, mechanical shafts, and all similar components.

      - Where a lot is adjacent to a lot in a UR Zone, within the required rear setback, a minimum 2.0 metre wide planting strip must be provided along the full length of the rear lot line and must comply with Subclause 4.16.1.2.

### Servicing Allocation Policy Overlay

    - Where a Servicing Allocation Policy is in effect, in accordance with Clause 2.6.2, a Holding Overlay is established in the area identified as “Servicing Allocation Policy Area” on Schedule J.

    - No new development is permitted in the Servicing Allocation Policy Area until such time that the Holding Overlay is removed for specified forms of development through a partial hold removal in accordance with Clause 2.6.4.

    - In order to remove the Servicing Allocation Policy Area Holding Overlay, the following conditions must be satisfied:

      - The City has confirmed servicing allocation for the specified form of development in accordance with the Servicing Allocation Policy.

    - Despite Clause 5.9.2., where the Servicing Allocation Policy specifies forms of development that are not subject to the Policy, such forms of development are not subject to the Servicing Allocation Policy Area Holding Overlay.

    - Where the City has established an expiry on a servicing allocation for a specified form of development as per Subclause 5.9.3.1. and that allocation expires, amendments to the Servicing Allocation Policy Overlay to delete the partial hold removal are considered a technical revision in accordance with Clause 1.10.2.”;

(By-Law Number 2022-62; 2024-332)

# Section 6:	Specific Use Provisions

### Tiny Houses, Mobile Homes and innovative Housing Technologies

    - Tiny houses, mobile homes, modular homes and other similar innovative housing technologies that result in the construction of a dwelling unit are considered a dwelling unit in a residential building. Where permitted, they must comply with all provisions applicable to:

      - the principal residential building in the urban area or rural area, as applicable; or

      - detached additional rural unit if it is located in the rural area; or

      - accessory houses if it is located in the urban area.

(By-Law Number 2022-62; 2024-332)

### Gasoline Pump

    - Where a gasoline pump is accessory to a service station, the gasoline pump may be located in the front yard or exterior yard, provided that:

      - The minimum setback from the gasoline pump to any lot line is 4.5 metres; and

      - Where the lot is a corner lot, a gasoline pump must maintain a minimum

separation distance of 3.0 metres from a sight triangle.

### Home Offices and Home Occupations

    - Home offices are permitted in all dwelling units and may be located in an

#### accessory building to the principal dwelling unit.

    - A home occupation is permitted in all dwelling units and may be located in an accessory building to the principal dwelling unit. In addition to the provisions that apply to the principal dwelling unit and accessory buildings, a home occupation is subject to the following provisions:

      - Where the home occupation is located within the principal dwelling unit, the gross floor area of the home occupation must be less than the gross floor area dedicated to the principal dwelling unit. For the purpose of this

calculation, where gross floor area is shared between the principal dwelling unit and the home occupation, it is included in the gross floor area for both the principal dwelling unit and the home occupation;

      - A maximum of one employee, other than a resident of the dwelling unit, is permitted to physically work in the home occupation;

      - Outdoor storage of materials, equipment, or containers, exterior parking or storage of commercial or industrial vehicles or equipment, animal enclosures or other outdoor manufacturing or processing activities are prohibited on any lot in conjunction with the home occupation, except as permitted by Clause

7.6.1.; and

      - The following activities are prohibited as a home occupation:

        - sale and servicing of motor vehicles, recreational vehicles or any motorized equipment, except on a lot in an AG or RU Zone; and

        - cannabis production for any use other than personal use.

(By-Law Number 2022-62; 2024-332)

### Kennels

    - Where a kennel is permitted by this By-law, the kennel may be located in the same

building as the principal dwelling unit, subject to the provisions of Clause 6.4.2.

    - All components associated with a kennel, including buildings and all outdoor animal runs, are considered buildings for zoning purposes, and must comply with all applicable provisions of this By-law, as well as the following provisions:

#### Location of Animal Runs

      - Outdoor animal runs associated with a kennel must be located in the rear yard or interior yard.

#### Minimum Separation

      - A minimum separation distance of 120.0 metres is required between a kennel

and all dwelling units, except for a dwelling unit located on the same lot.

    - Despite Subclause 6.4.2.2., when a kennel is established under the provisions of this By-law, the subsequent construction of a new dwelling unit on a separate lot does not render such kennel non-complying.

    - All kennels existing as of the date of passing of this By-law which would be made non-complying uses under Clause 6.4.2. are deemed to comply with this By-law.

### Marine Facilities

    - A marine facility is permitted as an accessory use on any lot with a lot line that adjoins a waterbody. Marine facilities are exempt from all other provisions of this By-law, including the waterbody separation distance required by Clause 4.23.1., and:

      - Must have a minimum setback of 1.2 metres from the interior lot line and/or exterior lot lines, measured at the point where the marine facility intersects with the lot line that adjoins the waterbody;

      - Must not encroach on an adjacent lot as determined by measuring the hypothetical extension of the lot lines into the waterbody;

      - Where a marine facility includes a building with exterior walls and a roof, including a boathouse, such building must:

        - Include a direct connection to the waterbody, either through an open boat slip internal to the building or a marine railway;

        - Not exceed a height of 6.0 metres, with height being measured from the high water mark of the waterbody to the highest point of the building; and

        - Not exceed 1 storey.

    - Despite Clause 6.5.1., a marine facility is not permitted within a wetland that has been identified by the Province of Ontario as a Provincially Significant Wetland.

### Model Homes

    - Where a Model Home Agreement or a Subdivision Agreement permitting the construction of model homes has been executed by the owner and the City, more than one house, semi-detached house or townhouse may be constructed on a lot prior to registration of the plan of subdivision under the Planning Act or a description under the Condominium Act, 1998, subject to the following provisions:

      - The use must be permitted in the underlying Zone in which the house, semi-detached house or townhouse is to be located;

      - The model home must not be occupied as a dwelling unit prior to the date of the registration of the subdivision plan or condominium plan;

      - The maximum number of model homes within one plan of subdivision under the Planning Act or a description under the Condominium Act, 1998 proposed for registration is equal to 10% of the total number of lots intended for a house, semi-detached house or townhouse purposes within the plan or description for registration, to a maximum of 10 model homes;

(By-Law Number 2022-62; 2024-332)

      - The model home must comply with all other provisions of this By-law, as though constructed on the lot within the registered plan of subdivision under the Planning Act or the description under the Condominium Act, 1998; and

      - The model home must comply with all applicable terms and conditions of the Model Home Agreement or Subdivision Agreement, as applicable.

### Outdoor Patios

    - An outdoor patio is permitted as an accessory use on a lot containing a permitted commercial or hospitality use, or on the roof of a building on such lot, and must comply with the following provisions:

      - Where the lot has a lot line that is adjacent to a lot in an Urban Residential Zone, HCD1 Zone or HCD3 Zone:

        - A minimum separation distance of 3.0 metres is required between an

outdoor patio and such lot line; and

        - A continuous, solid privacy fence with a minimum height of 1.8 metres must be provided along the portion of such lot line adjacent to the outdoor patio;

      - Outdoor patios are permitted to occupy parking spaces, excluding required

#### accessible spaces, car-share spaces or visitor spaces; and

      - Despite Section 7, no additional loading spaces or parking spaces are required for an outdoor patio.

### Co-Living Units

    - Co-living units are permitted in an apartment building or a mixed use building.

    - Where this By-law sets a standard based on the number of dwelling units for apartment buildings or mixed use buildings, co-living units must comply with all provisions that apply to dwelling units at a ratio of 4 private bedrooms and/or living spaces to 1 dwelling unit.

### Transportation Terminals

    - In addition to the uses permitted in the applicable Zone, the following uses are permitted within a transportation terminal:

      - Financial institution;

      - Personal service shop;

      - Restaurant;

      - Retail store; and

#### Repair shop.

    - The uses permitted in Clause 6.9.1. must comply with the following provisions:

      - Each individual occupancy must not exceed 50.0 square metres in gross floor area; and

      - The maximum cumulative gross floor area occupied by the permitted uses in Clause 6.9.1 is 100.0 square metres.

### Propane Transfer Facilities

    - Where a permitted use or building includes a propane transfer facility that is regulated by the Technical Standards and Safety Act, 2000, S.O. 2000, c. 16, such use or building must comply with all applicable provisions of this By-law and:

      - The propane transfer facility must be setback a minimum of 50 metres from any lot line, or a greater distance if required by a regulating Provincial body.

### Propane and Natural Gas Handling Facilities

    - Facilities relating to the handling and transfer of propane and natural gas which are not regulated by the Technical Standards and Safety Act, including tanks and associated compressors, pumps and other similar facilities must:

      - Not be located in any required setback; or

      - Maintain a minimum separation distance of 30.0 metres to any lot where a

dwelling unit is a permitted use.

### Backyard Hen Coops

    - Backyard hen coops and backyard hen runs are permitted as accessory buildings on any lot where the permitted principal use of such lot is a dwelling unit or agricultural use and must comply with all provisions of Subsection 4.1. except as specifically regulated by this Subsection.

    - Backyard hen coops and backyard hen runs must comply with the following minimum setbacks:

      - Rear lot line: 1.2 metres;

      - Interior lot line: 1.2 metres; and

      - Exterior lot line: 1.2 metres.

    - Backyard hen coops and backyard hen runs are not permitted in a front yard, interior yard or exterior yard.

    - Backyard hen coops and backyard hen runs must be located a minimum

#### separation distance of:

      - 15.0 metres from any lot that contains an elementary school or secondary school;

      - 7.5 metres from any lot that contains a place of worship or business; and

      - 3.0 metres from all windows and doors of residential buildings that are located on adjacent lots.

    - Where a conflict exists between the terms of Subsection 6.12. and the terms of the Animal Control By-law, the more restrictive provision prevails.

### Pigeon Lofts

    - Pigeon lofts are permitted as accessory buildings on any lot where the permitted principal use of such lot is a dwelling unit and where such lot has a minimum lot area of 1,350 square metres or where the permitted principal use of such lot is an agricultural use, and must comply with all provisions of Subsection 4.1. except as specifically regulated by this Subsection.

    - Pigeon lofts must comply with the following minimum setbacks:

      - Rear lot line: 6.0 metres;

      - Front lot line: 6.0 metres;

      - Interior lot line: 6.0 metres; and

      - Exterior lot line: 6.0 metres.

    - Pigeon lofts are only permitted in a rear yard.

    - The maximum height of a pigeon loft is 5.0 metres.

    - The minimum floor area of a pigeon loft is 1.0 square metre of floor area for every 10 pigeons.

    - The maximum floor area of a pigeon loft is 10 square metres.

    - Pigeon lofts must be located a minimum separation distance of 15.0 metres from

residential buildings that are located on a different lot.

    - Where a conflict exists between the terms of Subsection 6.13. and the terms of the Animal Control By-law, the more restrictive provision prevails.

### Place of Worship

    - Where a place of worship is permitted, the following complementary uses are permitted where they only occupy floor area that is developed to support the principal use of the lot:

      - Educational uses;

      - Programs of community, cultural or social benefit;

      - Community assembly areas;

      - Catering kitchens, provided they use existing kitchen facilities that support the

place of worship;

      - Offices;

      - Day care centres;

      - Cemetery;

      - Non-residential uses that are permitted in the CN Zone as per Table 15.1.2.;

      - Other uses that are similar in nature to Paragraphs 1. through 6.

    - Dwelling units are permitted as an accessory use in a place of worship, to a maximum of four dwelling units in the urban area and three dwelling units in the rural area, inclusive of dwelling units in an accessory house or detached additional rural unit. Such dwelling units must comply with Subsections 4.29. or 4.30., or the Zone standards that apply to the principal building.

(By-Law Number 2022-62; 2024-332; 2024-333)

### Schools

    - Where an elementary school or secondary school is permitted, portable classrooms are permitted and must comply with all setbacks that apply to the lot as though they are a principal building. For clarity, provisions requiring additional bike spaces, accessible spaces, loading spaces and any other additional facilities do not apply to portable classrooms.

    - Where a building is occupied by both an elementary school and a secondary school, or a combination thereof, the provisions that apply to secondary schools apply.

    - Where an elementary school or secondary school existed as of the date of passing of this by-law, such use is considered to be a permitted use of the lot so long so long as the building continues to be used as an elementary school or secondary school, as the case may be.

#### Adaptive Reuse of Elementary Schools, Secondary Schools and Community Facilities

    - Despite the permitted uses, in any Zone other than an Employment Zone where a building was originally designed and constructed for the purpose of an elementary school, secondary school, library, community centre or museum, such building

is permitted to be used as an elementary school, secondary school, library, community centre or museum.

(By-Law Number 2022-62; 2024-332)

### Stadiums

    - Stadiums are not permitted as an accessory use.

    - Despite Clause 6.16.1., where a stadium legally existed as an accessory use prior to the passage of this By-law, the stadium is considered to be a permitted use on such lot so long as the stadium continues to be used as a stadium.

### Food Trucks

    - Food trucks that stand more or less permanently in one place on a lot are considered buildings in accordance with this By-law and must comply with all provisions of this By-law that apply to buildings, in addition to the provisions of Subclauses 6.17.3.1. to 6.17.3.7.

    - This By-law does not apply to food trucks that are not considered buildings

pursuant to Clause 6.17.1., except as set out in Clause 6.17.3.

    - Food trucks must comply with the following provisions:

      - Requirements established in the Business Licensing By-law;

      - Food trucks are only permitted:

        - on a lot where the applicable Zone permits a restaurant;

        - in a public park;

        - as an accessory use on a lot where the applicable Zone permits commercial, employment and institutional uses as principal uses;

        - as an accessory use to a principal agricultural use on a lot that is zoned AG or RU, to a maximum of one food truck per lot;

      - Food trucks are permitted to occupy a maximum of 2 parking spaces on a

lot, excluding accessible spaces, car-share spaces or visitor spaces;

      - Food trucks must be setback a minimum of 3.0 metres from all lot lines; and

      - Food trucks must comply with sight triangle provisions in Subsection 4.6.

### Seasonal Cabins

    - In the rural area, where a lot is permitted to be used as a house by the applicable Zone, a cabin is a permitted principal use and must comply with all provisions that apply to a house.

(By-Law Number 2022-62; 2024-332)

### Residential Uses

    - An emergency shelter, group home, supportive housing, transitional housing, community home and other similar uses are considered a dwelling unit where the nature of such use does not qualify as a special needs facility. Where such use is considered a dwelling unit, it is permitted on any lot where the Zone permits a dwelling unit, and must be in a permitted building according to the applicable Zone.

### Existing Mineral Aggregate Operations

    - Where a mineral aggregate operation legally existed and was licensed by the Province prior to the passage of this By-law, the mineral aggregate operation is considered to be a permitted use on such lot and must comply with the provisions of the MX1 Zone, so long as the mineral aggregate operation continues to be used. For the purposes of Subsection 4.26., such mineral aggregate operation is considered to be on a lot in an MX1 Zone.

### Back-to-Back Townhouses

    - Despite the required rear setback of the applicable Zone, where multiple lots are

developed with back-to-back townhouses, the minimum rear setback is 0 metres.

(By-Law Number 2022-62; 2024-332)

# Section 7:	Parking, Loading and Bike Parking Provisions

### Parking Space Ratios

    - The minimum number of parking spaces, maximum number of parking spaces and minimum number of car-share spaces set out in Table 7.1.1. must be complied with and located on the same lot as the corresponding use or building.

    - This By-law places all lands subject to this By-law in one or more Parking Areas shown on Schedule 2. In accordance with Clause 7.1.1., required parking spaces are calculated by the ratio required for the applicable Parking Area. For the purpose of this By-law, the following nomenclature may be used interchangeably:

      - Parking Area 1 or PA1;

      - Parking Area 2 or PA2;

      - Parking Area 3 or PA3;

      - Parking Area 4 or PA4; and

      - Parking Area 5 or PA5.

    - Despite Clause 7.1.2., where any portion of a lot in PA5 is within 600 metres walking distance of the “Kingston Transit Route” identified on Schedule 2, measured to the boundary of the street line, the Parking Area that applies to that portion of the

“Kingston Transit Route” may apply to such lot.

    - Where more than one Parking Area applies to one lot, the required parking spaces are calculated based on the Parking Area that requires the fewest number of parking spaces.

    - Where a lot contains more than one use, the required number of parking spaces is the sum of all parking spaces required for each use.

    - When the computation of the required number of parking spaces for each use on a lot results in a number containing a fraction, that minimum number of parking spaces required for each use must be increased to the next highest whole number only if the fraction is equal to or greater than 0.5.

    - Despite Clause 7.1.6., when the computation of the required number of accessible spaces results in a number containing a fraction, the minimum number of accessible spaces is increased to the next highest whole number.

    - Despite Clause 2.1.9., where two or more provisions of this By-law conflict as it relates to the calculation of the minimum number of standard parking spaces, the lesser requirement prevails. For example, where an Exception in Section 20 or Section 21 requires a greater number of parking spaces than Section 7, the provisions of Section 7 prevail. For clarity, this applies only to standard parking spaces and does not include car-share spaces, visitor spaces, accessible spaces, bike spaces or loading spaces.

    - Where the number of parking spaces is identified as a ratio per “person”, the number is calculated based on the maximum number of persons which can be accommodated within the building according to the Building Code Act, 1992, unless otherwise specified in this By-law.

    - Where a maximum number of parking spaces is identified in Table 7.1.1., the maximum applies to the number of standard parking spaces and excludes accessible spaces, car-share spaces and visitor spaces.

    - Despite the definition of gross floor area, for the purpose of calculating the required number of parking spaces, floor area that is dedicated solely to the warehousing or storage of goods or products is excluded from the gross floor area calculation.

#### Off-Site Parking Spaces

    - Despite Clause 7.1.1., where a building that exists as of the date of passing of this By-law is converted resulting in an increase in the number of dwelling units, required parking spaces may be provided off-site, within 60.0 metres of the lot.

(By-Law Number 2022-62; 2024-332)

#### Affordable Units and Heritage Buildings

    - Despite Clause 7.1.1.:

      - For affordable units, no parking spaces or car-share spaces are required, except for accessible spaces and visitor spaces. Where affordable units are mixed with other dwelling units on a lot, this provision only applies to the units that are affordable units.

      - For a lot that contains a designated heritage building, no parking spaces, visitor spaces or car-share spaces are required. Where parking spaces are provided, accessible spaces must be provided based on the ratio required by Subsection 7.2. before other parking spaces may be provided.

#### Reduction in Minimum Number of Required Parking Spaces

    - Despite Clause 7.1.1.:

      - For apartment buildings, dwelling units in a mixed use building and stacked townhouses, the minimum number of parking spaces required may be reduced:

        - In accordance with the Cash-in-Lieu of Parking By-law.

(By-Law Number 2022-62; 2024-332)

#### Permission to Provide More Parking Spaces than the Maximum

    - Despite Clause 7.1.1.:

      - In PA1 and PA2, the maximum number of parking spaces permitted by this By-law may be increased to a maximum of 1.25 parking spaces per dwelling unit if all parking spaces provided above the maximum ratio are electric vehicle ready.

      - In PA3, PA4 and PA5, the maximum number of parking spaces permitted by this By-law may be increased to a maximum of 1.5 parking spaces per dwelling unit if all parking spaces provided above the maximum ratio are electric vehicle ready.

(By-Law Number 2022-62; 2024-332)

#### Electric Vehicle Supply Equipment Permitted

    - Parking spaces required or provided for any use may be used as part of electric vehicle supply equipment, whether or not a fee is charged for the use of such electric vehicle supply equipment.

#### Car-share Spaces Permitted

    - Car-share spaces are permitted in any parking lot, commercial parking lot or a

parking structure in any Zone.

#### Temporary Use of Car-share Spaces

    - Where car-share spaces are required on a lot, such car-share spaces may be temporarily used as visitor spaces if no car-share vehicle services are in operation on the lot.

#### Addition to Existing Building – Parking Spaces

    - When a lot or building has insufficient parking on the date of passing of this By-law to conform to the provisions of this By-law, this By-law is not to be interpreted as requiring that the deficiency be made up prior to the construction of any addition provided, however, any additional parking spaces required by this By-law for such addition must be provided in accordance with all provisions respecting parking spaces.

(By-Law Number 2022-62; 2024-332)

#### Table 7.1.1. – Required Number of Parking, Visitor and Car-Share Spaces

|  | use | Number of Parking Spaces (minimum, unless otherwise specified) |
| --- | --- | --- |
| 1. | - apartment building - dwelling unit in mixed use building - stacked townhouse | - Required number of parking spaces: PA1, PA2: minimum 0.4 to a maximum of 1.0 per dwelling unit PA3: minimum 0.6 to a maximum of 1.0 per dwelling unit PA4: minimum 0.8 to a maximum of 1.0 per dwelling unit PA5: minimum and maximum of 1.0 per dwelling unit - In addition to (i), where a lot contains 30 or more dwelling units, a minimum of: - 1 car-share space must be provided, with 1 additional car- share space provided for every 50 dwelling units thereafter; and - 1 short-term delivery parking space, the location and signage of which must be provided to the satisfaction of the City through the site plan review process (iii) In addition to (i) and (ii), the minimum number of visitor spaces: PA1, PA2: 0.03 per dwelling unit PA3, PA4, PA5: 0.06 per dwelling unit |
| 2. | In the urban area: - house - townhouse - semi-detached house - accessory house | PA1, PA2, PA3, PA4: 1.0 parking spaces per lot PA5: 1.0 per dwelling unit for the first two dwelling units on a lot |

|  | use | Number of Parking Spaces (minimum, unless otherwise specified) |
| --- | --- | --- |
| 3. | In the rural area: (a) dwelling unit | 1.0 per dwelling unit |
| 4. | home occupation | PA1: 0 parking spaces PA2, PA3, PA4, PA5: 0 parking spaces, except where an employee who does not reside in the dwelling unit, or a customer physically attend the lot, then 1 parking space |
| 5. | All other uses | — |

(By-Law Number 2022-62; 2024-332)

### Accessible Parking Provisions

    - Accessible spaces must comply with all applicable provisions of Section 7, except as specifically required by the provisions of Subsection 7.2.

#### Effective Parking Space Ratios to Calculate Required Accessible Spaces

    - The number of parking spaces used as the basis of calculating the minimum number of accessible spaces in Clause 7.2.3. is the greater of:

      - The number of parking spaces provided on the lot; or

      - The calculated number of parking spaces based on the effective ratio

established for the corresponding use in Table 7.2.2.

#### Number of Required Accessible Spaces

    - The minimum number of accessible spaces required by the following Subclauses must be provided on the same lot as the use or building:

      - Where the number of parking spaces in Clause 7.2.2. is between 1 and 12

parking spaces, one parking space must be a Type A accessible space;

      - Where the number of parking spaces in Clause 7.2.2. is between 13 and 100 parking spaces, 4% of the number of parking spaces in Clause 7.2.2. must be accessible spaces;

      - Where the number of parking spaces in Clause 7.2.2. is between 101 and 200 parking spaces, one parking space plus 3% of the number of parking spaces in Clause 7.2.2. must be accessible spaces;

      - Where the number of parking spaces in Clause 7.2.2. is between 201 and 1,000 parking spaces, two parking spaces plus 2% of the number of parking spaces in Clause 7.2.2. must be accessible spaces; and

      - Where the number of parking spaces in Clause 7.2.2. is more than 1,000 parking spaces, 11 parking spaces plus 1% of number of parking spaces in Clause 7.2.2. must be accessible spaces.

    - Despite Clause 7.2.3., where a lot contains a house, semi-detached house, additional residential unit, tiny house or freehold townhouse where parking spaces are accessed directly from a driveway, no accessible spaces are required.

(By-Law Number 2022-62; 2024-332)

    - Despite Clauses 7.2.2. and 7.2.3., where a lot contains a use with parking spaces that are reserved for exclusive use, such as law enforcement vehicles, emergency services vehicles, impounded vehicles, transportation fleets or other similar vehicles, such exclusive use parking spaces are not included in the calculation of the parking spaces for the purpose of calculating accessible spaces.

#### Table 7.2.2. – Effective Ratios to Calculate Required Accessible Spaces

|  | Use | Effective Ratio |
| --- | --- | --- |
| 1. | residential use | 1.0 per dwelling unit |
|  | automobile body shop, automobile repair shop, | PA1: 0 spaces PA2, PA3: 2 per 100 square metres of gross floor area PA4, PA5: 4 per 100 square metres of gross floor area |
|  | automobile sales establishment, banquet hall, animal care, |  |
|  | animal shelter, building supply store, carwash, club, |  |
| 2. | creativity centre, department store, financial institution, fitness centre, garden centre, service station, grocery |  |
|  | store, laundry store, office, personal service shop, |  |
|  | recreational vehicle sales establishment, repair shop, |  |
|  | restaurant, retail store, training facility, wellness clinic |  |

|  | Use | Effective Ratio |
| --- | --- | --- |
| 3. | agricultural sales establishment, call centre, catering service, contractor’s yard, correctional institution, equipment rental, factory outlet, feedmill, ferry terminal, heavy equipment or truck repair shop, heavy industrial use, hospital, industrial repair shop, laboratory, library, light industrial use, museum, production studio, research establishment, salvage yard, self-service storage facility, special needs facility, towing compound, transportation depot, transportation terminal, warehouse, waste disposal area, wastewater treatment facility, water supply plant, wholesale establishment, workshop | 1 per 100 square metres of gross floor area |
| 4. | community centre, entertainment establishment, funeral establishment, recreation facility, place of worship, stadium | 0.25 per person |
| 5. | agricultural use, airport facilities, cemetery, community garden, correctional college, food truck, forestry use, gravel pit, military installation, mineral aggregate operation, park, transformer station | — |
| 6. | day care centre, elementary school, post-secondary institution, secondary school | 2.0 per classroom |
| 7. | campground | 1 per campsite |
| 8. | golf course | 2 per hole |
| 9. | hotel | 1 per guest room |
| 10. | kennel | 1 per animal run |
| 11. | marina | 1 per boat slip |

(By-Law Number 2022-62; 2024-332)

#### Type A and Type B Accessible Space Calculations

    - The calculation of the required number of Type A and Type B accessible spaces

must comply with the following provisions:

      - Where an even number of accessible spaces is required by Clause 7.2.3., an equal number of Type A and Type B accessible spaces must be provided;

      - Where an odd number of accessible spaces is required by Clause 7.2.3., an equal number of Type A and Type B accessible spaces must be provided, but

the additional accessible space, the odd-numbered space, may be either a Type A or a Type B accessible space;

      - Despite Subclause 2., where only one accessible space is required, it must be a Type A accessible space.

#### Accessible Parking Design Standards

    - Accessible spaces must comply with the following minimum widths:

      - The minimum width of a Type A accessible space is 3.4 metres; and

      - The minimum width of a Type B accessible space is 2.7 metres.

    - Accessible spaces must comply with the following minimum vertical clearances:

      - The minimum vertical clearance of an accessible space located in a parking structure is 2.1 metres; and

      - The minimum vertical clearance of all other accessible spaces is 2.9 metres.

    - An accessible aisle must be provided for each accessible space and may be shared by a maximum of two accessible spaces. Accessible aisles must:

- Have a minimum width of 1.5 metres;

- Extend the full length of the accessible space; and

- Be marked with high tonal contrast diagonal lines where the surface is asphalt, concrete or some other hard surface.

    - All accessible spaces must:

      - Have an accessible permit parking sign that satisfies the requirements of the

Highway Traffic Act;

      - Have a maximum gradient of 5%; and

      - Type A accessible spaces must have signage that identifies the space as “van accessible”.

### Bike Parking Provisions

#### Bike Space Ratios

    - The minimum number of long-term bike spaces and short-term bike spaces required by the ratios set out in Table 7.3.1. or Table 7.3.2. or Table 7.3.3 must be provided on the same lot as the corresponding use or building. Despite the generalized Zone-based ratios provided in Table 7.3.2. for non-residential uses, the use-based ratios in Table 7.3.3. apply, except in the case of a multi-tenanted building with more than one principal use, then the generalized Zone-based ratios of Table 7.3.2. apply.

    - Where a lot contains more than one use, the required number of bike spaces is the sum of all bike spaces required for each use. Where multiple tenants separately occupy spaces or units in one building, the required number of bike spaces is calculated based on the total number of bike spaces that are required for the entire building and may be provided as one central bike facility, or individually across the separate units in a manner that generally proportions the bike facilities based on the gross floor area of each unit. Where the ultimate use of each unit is unknown at the time of construction, the permitted use which generates the highest number of bike spaces must be used to calculate the required number of bike spaces.

    - Where a lot contains more than one building, the bike space requirements are calculated and provided for each building as though they are on their own individual lot.

    - When the computation of required bike spaces results in a number containing a fraction, the minimum number of bike spaces required is increased to the next highest whole number if the fraction is equal to or greater than 0.5, unless otherwise required by this By-law.

    - Despite the definition of gross floor area, for the purpose of calculating the required number of bike spaces, floor area that is dedicated solely to the storage of goods or products is excluded from the gross floor area calculation.

    - Where possible, the requirements of Clauses 7.3.7. through 7.3.13. may be combined in a manner that allows one bike space to satisfy more than one requirement.

(By-Law Number 2022-62; 2024-332)

#### Long-term Bike Space Requirements

    - The long-term bike spaces required by Clause 7.3.1.:

      - Must be provided in a secure, weather-proof enclosure with controlled access where a bicycle may be parked and secured for the long-term in a stable position with at least one point of contact with the frame of the bicycle;

      - Are not permitted in a dwelling unit or on the balcony of a dwelling unit;

      - Must be provided in a location that has access directly to a street line by way of a continuous pathway consisting of:

        - A hallway, aisle, sidewalk or walkway;

        - An elevator that permits bikes to the satisfaction of the City;

        - A ramp, which may be provided as a wheel ramp along the side of a set of stairs if such ramp is a minimum of 0.15 metres wide and does not cut into the stair tread; and/or

        - A drive aisle or driveway.

      - A maximum of 75% of the required long-term bike spaces are permitted to be provided as vertical bike spaces.

(By-Law Number 2022-62; 2024-332)

    - Despite Clause 7.3.7.:

      - Where a long-term bike space is required for a house, semi-detached house, townhouse and accessory house, it is exempt from Clauses 7.3.7., 7.3.10.,

7.3.11. and 7.3.12. and must:

        - be provided in a secure, enclosed location such as within an accessory building, or within a vestibule or other interior floor area that doesn’t form part of a dwelling unit; and

        - be provided with a continuous pathway that ensures clear access from the street to the long-term bike space.

      - Where a building that existed on the date of passing of this By-law is converted in a manner that results in the introduction of dwelling units or an increase in the number of dwelling units, it is exempt from Clauses 7.3.7.2. and 7.3.7.4.

(By-Law Number 2022-62; 2024-332)

#### Short-term Bike Space Requirements

    - The short-term bike spaces required by Clause 7.3.1.:

      - Must be provided as horizontal bike spaces;

      - Must be provided at grade or on the first storey in a location that is accessible to the general public by way of a continuous pathway consisting of:

        - A hallway, aisle, sidewalk or walkway;

        - An elevator that permits bikes to the satisfaction of the City;

        - A ramp, which may be provided as a wheel ramp along the side of a set of stairs if such ramp is a minimum of 0.15 metres wide and does not cut into the stair tread; and/or

        - A drive aisle or driveway; and

      - Must be provided in an easily accessible and well lit location no more than 15.0 metres walking distance from a pedestrian entrance to the building. Where multiple tenants separately occupy spaces or units in one building, and the bike spaces are provided as one central bike facility as per Clause 7.3.2., this requirement does not apply.

(By-Law Number 2022-62; 2024-332)

#### Bike Space and Bike Aisle Dimensions

    - Horizontal bike spaces must:

      - Have minimum dimensions of 0.6 metres wide by 1.8 metres horizontal length, with a minimum vertical clearance of 1.9 metres;

      - Be provided with a bike rack where a lock is capable of securing the bicycle frame and front wheel; and

      - Be accessed by an aisle with a minimum width of 1.2 metres where 4 or more

bike spaces are provided in a common bike storage area.

(By-Law Number 2022-62; 2024-332)

#### Vertical bike spaces must:

      - Have minimum dimensions of 0.6 metres wide by 1.8 metres vertical length, where the bike, when secured on the storage rack is provided with a minimum horizontal clearance from the wall of 1.2 metres;

      - Be equipped with a storage rack that supports the bike without having the bike suspended by its wheels, and where a lock is capable of securing the bike frame and front wheel to the rack; and

      - Be accessed by an aisle with a minimum width of 1.2 metres where 4 or more

bike spaces are provided in a common bike storage area.

(By-Law Number 2022-62; 2024-332)

#### Stacked bike spaces must:

      - Have minimum dimensions of 0.6 metres wide by 1.8 metres horizontal length, with a minimum vertical clearance of 1.2 metres;

      - Be equipped with a storage rack where a lock is capable of securing the bike frame and front wheel to the rack;

      - Be equipped with a lift mechanism providing floor level access to both bike spaces; and

      - Be accessed by an aisle with a minimum width of 1.2 metres where 4 or more

bike spaces are provided in a common bike storage area.

(By-Law Number 2022-62; 2024-332)

#### Enhanced Bike Parking Facilities for Multi-Unit Residential

    - For apartment buildings, dwelling units in mixed use buildings, and stacked townhouses, enhanced bike parking facilities must be provided in accordance with the following:

      - A minimum of 5% of the long-term bike spaces must be provided as larger horizontal bike spaces with minimum dimensions of 1.0 metre wide by 2.6 metres horizontal length, with a minimum vertical clearance of 1.9 metres and must be provided with access to one standard electrical outlet;

      - A minimum of 5% of the long-term bike spaces must be provided in secure bike lockers that are provided with individual, secure enclosures where a private lock can be affixed and must include a standard electrical outlet;

      - A minimum of 10% of the long-term bike spaces provided in a shared bike room must be provided with access to one standard electrical outlet per long- term bike space;

      - Where more than 25 long-term bike spaces are required, a bike maintenance area must be provided that is a sufficient size to accommodate repairs and maintenance of bikes, and must include a bike pump, bike repair stand and a bench; and

      - A minimum of 50% of the short-term bike spaces must be weather protected.

(By-Law Number 2022-62; 2024-332)

#### End-of-Trip Bike Facilities for Non-Residential Uses

    - Where a lot or building is required to provide long-term bike spaces for any non- residential use, end-of-trip bike facilities must be provided at the following ratios:

      - Where 5 to 60 long-term bike spaces are required, 1 end-of-trip bike facility;

      - Where 61 to 120 long-term bike spaces are required, 2 end-of-trip bike facilities;

      - Where 121 to 180 long-term bike spaces are required, 3 end-of-trip bike facilities; and

      - Where more than 180 long-term bike spaces are required, 4 end-of-trip bike facilities.

    - Where end-of-trip bike facilities are required by Clause 7.3.14., clothing lockers must be provided at a minimum ratio of 1 clothing locker for every 1 long-term bike space required. Such clothing lockers must:

      - Have a minimum height of 0.9 metres, a minimum width of 0.3 metres and a minimum depth of 0.45 metres; and

      - Be provided with a secure enclosure where a private lock can be affixed.

#### Addition to Existing Building – Bike Spaces and End-of-Trip Facilities

    - Where a lot or building has insufficient bike spaces, enhanced bike parking facilities or end-of-trip bike facilities on the date of passing of this By-law to conform with the provisions of this By-law, this By-law is not to be interpreted as

requiring that the deficiency be made up prior to the construction of any addition or provided, however, that any additional bike spaces or enhanced bike parking facilities or end-of-trip bike facilities required by this By-Law for such addition must be provided in accordance with all provisions of Subsection 7.3.

(By-Law Number 2022-62; 2024-332)

#### Table 7.3.1. – Required Number of Long-Term and Short-Term Bike Spaces

|  | Use | Minimum Number of Long-Term Bike Spaces | Minimum Number of Short-Term Bike Spaces |
| --- | --- | --- | --- |
| 1. 1 | Residential - apartment building - dwelling unit in mixed use building - stacked townhouse | 0.9 per dwelling unit | 0.1 per dwelling unit |
| 2. | - house - semi-detached house - townhouse - accessory house | - in the urban area: 1.0 per dwelling unit - in the rural area: — | — |
| 3. | home occupation | — | — |

(By-Law Number 2022-62; 2024-332)

#### Table 7.3.2. – Required Number of Long-Term and Short-Term Bike Spaces for Non- Residential Uses by Zone

|  | Zone or Use | Minimum Number of Long-Term Bike Spaces | Minimum Number of Short- Term Bike Spaces |
| --- | --- | --- | --- |
| 1. | Rural Zones | — | — |
| 2. | Rural Industrial Zones | — | — |
| 3. | Non-residential uses in Mixed Use Zones, Urban Residential Zones, Urban Multi-Residential Zones and Heritage Zones | 0.1 per 100 square metres of gross floor area | 0.2 per 100 square metres of gross floor area |
| 4. | Commercial Zones | 0.1 per 100 square metres of gross floor area | 0.2 per 100 square metres of gross floor area |

|  | Zone or Use | Minimum Number of Long-Term Bike Spaces | Minimum Number of Short- Term Bike Spaces |
| --- | --- | --- | --- |
| 5. | Employment Zones | 0.1 per 100 square metres of gross floor area | — |
| 6. | Transportation and Utilities Zones | — | — |

(By-Law Number 2022-62; 2024-332)

#### Table 7.3.3. – Required Number of Long-Term and Short-Term Bike Spaces for Non-

Residential Uses by Use

|  | Zone or Use | Minimum Number of Long- Term Bike Spaces | Minimum Number of Short- Term Bike Spaces |
| --- | --- | --- | --- |
| 1. | airport facilities, automobile body shop, automobile repair shop, automobile sales establishment, campground, carwash, cemetery, community garden, correctional college, correctional institution, feedmill, ferry terminal, food truck, forestry use, funeral establishment, golf course, hospital, hotel, kennel, marina, military installation, park, recreational vehicle sales establishment, service station | — | — |
| 2. | banquet hall, library, museum | — | 0.2 per 100 square metres of gross floor area |
| 3. | community centre, entertainment establishment, place of worship, recreation facility, stadium | — | 0.1 per person |
| 4. | day care centre | — | 0.5 per classroom |
| 5. | elementary school, secondary school | 1 per classroom | 1.5 per classroom |
| 6. | post-secondary institution | — | 3 per classroom |

(By-Law Number 2022-62; 2024-332)

### Driveway, Drive Aisle & Parking Space Requirements

    - The minimum length, width and vehicle projections of parking spaces and minimum width of driveways and drive aisles set out in Table 7.4.1. must be complied with for all parking spaces, accessible spaces and car-share spaces, except as specifically required by Subsection 7.2.

    - The minimum vertical clearance for all parking spaces, driveways and drive aisles, except for accessible spaces, is 2.1 metres.

    - The minimum angle of intersection between a driveway and a street line is 60 degrees from the street line. Where the street line is curved, the angle of intersection is measured from the tangent of the curved street line.

    - In the urban area, all parking lots, drive aisles, driveways and parking spaces must be maintained with a stable surface which is treated so as to prevent the raising of dust or loose particles.

    - Where parking spaces are accessed by a drive aisle, and such drive aisle is a dead- end, a 1.5 metre deep turnaround area must be provided at the dead-end for the full width of the drive aisle, with a 1.0 metre radius between parking spaces and the turnaround area.

    - For the purpose of this By-law, the width of a driveway or drive aisle is measured perpendicular to the direction in which a motor vehicle drives.

    - For the purpose of this By-law, vehicle projection for angled parking spaces is measured perpendicular to the drive aisle.

#### Table 7.4.1. – Minimum Parking Space and Drive Aisle Dimensions

| Orientation of Parking Space Relative to Driveway or Drive Aisle | Minimum Parking Space Width | Minimum Parking Space Length or Vehicle Projection (measured perpendicular to drive aisle) | Minimum Driveway or Drive Aisle Width |
| --- | --- | --- | --- |
| In line with and accessed directly from driveway | 2.6 metres | 6.0 metre length | 2.6 metre driveway |
| In line with and accessed directly from driveway as tandem parking spaces | 2.6 metres | 12.0 metres length | 2.6 metre driveway |

| Perpendicular to drive aisle | 2.6 metres | 5.5 metre length | 6.7 metre drive aisle for the portion of the drive aisle used to access a parking space, and 6.0 metres for any internal circulation road that is not directly used to access a parking space |
| --- | --- | --- | --- |
| Parallel to drive aisle | 2.6 metres | 6.7 metre length | 6.0 metre drive aisle |
|  |  |  | for two-way traffic and 3.0 metre drive aisle for one-way traffic |
| 45 degree angle from one- way drive aisle | 2.75 metres | 5.4 metre vehicle projection | 4.2 metre drive aisle |
| 50 degree angle from one- way drive aisle | 2.75 metres | 5.6 metre vehicle projection | 4.4 metre drive aisle |
| 55 degree angle from one- way drive aisle | 2.75 metres | 5.7 metre vehicle projection | 4.5 metre drive aisle |
| 60 degree angle from one- way drive aisle | 2.75 metres | 5.8 metre vehicle projection | 4.8 metre drive aisle |
| 65 degree angle from one- way drive aisle | 2.75 metres | 5.8 metre vehicle projection | 5.0 metre drive aisle |
| 70 degree angle from one- way drive aisle | 2.75 metres | 5.9 metre vehicle projection | 5.4 metre drive aisle |
| 75 degree angle from one- way drive aisle | 2.75 metres | 5.8 metre vehicle projection | 5.7 metre drive aisle |

(By-Law Number 2022-62; 2024-332)

#### Additional Driveway & Parking Provisions for Ground Oriented Residential in Urban Area

    - In the urban area, on a lot with a house, semi-detached house, accessory house

and or townhouse, the following provisions must be complied with:

      - Parking spaces must be located in a permitted private garage, driveway or

#### parking lot;

      - Parking spaces are permitted in tandem configuration;

      - Parking lots are only permitted in a rear yard or interior yard;

      - The maximum area of a parking lot is 43 square metres, excluding drive aisles;

      - The maximum width of a driveway at the point of intersection with the front lot line or exterior lot line is the lesser of:

        - 6.0 metres; or

        - 40% of the length of the applicable street line, provided that the minimum width of the driveway is 2.6 metres;

      - Beyond the point of intersection with the lot line, the maximum cumulative width of all driveways on a lot is 6.0 metres, except:

        - If a lot complies with and will continue to comply with the required landscaped open space and all other provisions of this By-law including all requirements of Clause 7.4.8., the driveway may be widened to accommodate one additional parking space to a maximum driveway width of 9.0 metres, if:

          - The increased driveway width is limited to the extent required to accommodate one additional parking space at a maximum dimension of 2.6 metres wide by 6 metres long plus any driveway area required to maneuver a vehicle into the additional parking space; and

        - Where the driveway leads to a private garage, the maximum width beyond the required front setback or exterior setback is the width of the private garage on the subject lot;

      - Despite Subclause 6., where the driveway leads to a private garage, the maximum width is the greater of 6.0 metres or the width of the private garage on the subject lot;

      - The minimum required setback from a private garage to the street line

where the driveway gains access to a street is 6.0 metres; and

      - The maximum number of driveways is 1 per lot, except as follows:

        - On a corner lot, a maximum of 2 driveways are permitted with a maximum of 1 driveway per street line and must be measured in accordance with the street line where the driveway gains access; and

        - Where 2 principal units in a semi-detached house are located on one

lot, a maximum of 2 driveways are permitted.

(By-Law Number 2022-62; 2024-332)

#### Additional Driveway & Parking Provisions for Multi-Unit Residential in the Urban Area

    - In the urban area, on a lot with a, stacked townhouse, apartment building, or a dwelling unit in a mixed use building, the following provisions must be complied with:

      - Where the side of a parking space is obstructed by any part of a fixed object such as a wall, column, bollard, fence or pipe within 0.3 metres of the side of the parking space, measured at right angles, and more than 1.0 metre from the front or rear of the parking space, the minimum width of the parking space must be increased by 0.3 metres for each side that is obstructed;

      - The maximum width of a driveway within the required front setback or

exterior setback is 6.7 metres; and

      - Parking spaces must be located in a permitted private garage, parking structure, driveway or parking lot in the rear yard or interior yard, except as follows:

        - Visitor spaces may be located in the front yard or exterior yard provided the visitor space is not closer than 3.0 metres to any lot line and not closer than 7.5 metres to any street line; and

      - Despite Clause 7.4.1., a maximum of 10% of parking spaces provided on a lot, excluding accessible spaces, visitor spaces and car-share spaces, are permitted to be parking spaces for small cars, with a minimum length of 4.8 metres and a minimum width of 2.4 metres, with signage that identifies the

space as “small car parking space”.

(By-Law Number 2022-62; 2024-332)

#### Additional Driveway & Parking Provisions for All Other Uses in the Urban Area

    - In the urban area, on a lot with all uses other than those specified in Clauses 7.4.8. and 7.4.9., the following provisions must be complied with:

      - Where the side of a parking space is obstructed by any part of a fixed object such as a wall, column, bollard, fence or pipe within 0.3 metres of the side of the parking space, measured at right angles, and more than 1.0 metre from the front or rear of the parking space, the minimum width of the parking space must be increased by 0.3 metres for each side that is obstructed;

      - The maximum width of a driveway within the required front setback or

exterior setback is 9.0 metres;

      - Parking spaces must be located in a permitted parking structure, driveway

or parking lot; and

      - Parking spaces and parking lots are permitted in the front yard, exterior yard, interior yard or rear yard, provided they comply with sight triangle provisions.

#### Additional Driveway & Parking Provisions for All Uses in the Rural Area

    - In the rural area, on a lot with any permitted use, the following provisions must be complied with:

      - The maximum width of a driveway within the required front setback or

exterior setback is 9.0 metres;

      - Parking spaces must be located in a permitted private garage, driveway or

parking lot; and

      - Parking spaces and parking lots are permitted in the front yard, exterior yard, interior yard or rear yard, provided they comply with sight triangle provisions.

(By-Law Number 2022-62; 2024-332)

#### Legal Non-Complying Driveways, Private Garages and Parking Spaces

    - Despite Clauses 7.4.8., 7.4.9., 7.4.10., 7.4.11. and 13.4.2.2., an existing driveway, private garage or parking space that does not meet the provisions of this By-law, but which legally existed on the date of passing of this By-law, is considered to be

legal non-complying. Clauses 1.8.2., 1.8.3. and 1.8.4 apply to legal non-complying

driveways, private garages or parking spaces.

### Loading Space Provisions

#### Loading Space Ratios

    - The minimum number of loading spaces required by the ratios set out in Table

7.5.1. must be provided on the same lot as the corresponding use or building.

    - Where a lot contains more than one use, the required number of loading spaces is the sum of all loading spaces required for each use.

    - Where a lot contains more than one building, the required number of loading spaces is calculated and provided for each building as though it was on an individual lot.

    - When the computation of required loading spaces results in a number containing a fraction, the minimum number of loading spaces required is increased to the next highest whole number if the fraction is equal to or greater than 0.5, unless otherwise required by this By-law.

#### Table 7.5.1. – Number of Required Loading Spaces

| Use | Gross Floor Area or Number of Dwelling Units | Number of Loading Spaces Required |
| --- | --- | --- |
| Employment Uses | 0 to 300.0 square metres | 1 |
|  | More than 300.0 to 2,500.0 square metres | 2 |
|  | More than 2,500.0 to 7,500.0 square metres | 3 |
|  | More than 7,500.0 square metres | 3 plus 1 for each additional 9,300.0 square metres beyond 7,500.0 square metres |
| Commercial Uses | 0 – 300.0 square metres | 0 |
|  | More than 300.0 to 2,500.0 square metres | 1 |
|  | More than 2,500.0 to 7,500.0 square metres | 2 |

| Use | Gross Floor Area or Number of Dwelling Units | Number of Loading Spaces Required |
| --- | --- | --- |
|  | More than 7,500.0 square metres | 2 plus 1 for each additional 9,300.0 square metres beyond 7,500.0 square metres |
| Residential Uses | 0 to 50 dwelling units | 0 |
|  | 51 to 399 dwelling units | 1 |
|  | 400 or more dwelling units | 2 |

(By-Law Number 2022-62; 2024-332)

#### Loading Space Dimensions

    - The minimum dimensions of a loading space are 3.5 metres wide by 9.0 metres long, with a minimum vertical clearance of 4.2 metres.

#### Surface Treatment of Loading Spaces

    - All loading spaces must be maintained with a stable surface which is treated so as to prevent the raising of dust or loose particles.

#### Location of Loading Spaces

    - A loading space must abut the use or building that requires the loading space.

    - Access to loading spaces must be provided by means of one or more unobstructed aisles which must:

      - Have a minimum unobstructed width of 3.5 metres and a minimum vertical clearance of 4.2 metres;

      - Provide sufficient space to permit the manoeuvring of vehicles on the lot so as not to obstruct, or otherwise cause a traffic hazard on, an adjacent street; and

      - Lead directly from the loading space to a street.

#### Addition to Existing Building – Loading Spaces

    - Where a lot or building has insufficient loading spaces on the date of passing of this By-law to conform with the provisions of this By-law, this By-law is not to be interpreted as requiring that the deficiency be made up prior to the construction of any addition, provided, however, any additional loading spaces required by this By-

Law for such addition must be provided in accordance with all provisions respecting

#### loading spaces.

(By-Law Number 2022-62; 2024-332)

### Outdoor Storage of Vehicles in Urban Residential Zones

#### Commercial Motor Vehicles

    - One commercial motor vehicle per dwelling unit is permitted on a lot in an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, HAM Zone, RUR Zone, LSR Zone or DR Zone, to a maximum of 3 commercial motor vehicles per lot. For clarity, such commercial motor vehicle may be connected to a home occupation.

    - The maximum rated capacity of a commercial motor vehicle is 2.0 tonnes.

#### Storage of Recreational Vehicles, Watercraft, and Trailers in Residential Areas

    - In an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, HAM Zone, RUR Zone, LSR Zone or DR Zone, no boat, personal watercraft, all- terrain vehicle, motor home, travel trailer, tent trailer, camper, snowmobile or other recreational vehicle, or a utility trailer may be stored or parked on a lot, except for:

      - One boat which must not exceed 8.2 metres in length;

      - One motor home which must not exceed 8.2 metres in length;

      - Not more than two personal watercraft, all-terrain vehicles, snowmobiles or other recreational vehicles, or any combination thereof;

      - One travel trailer, tent trailer or camper, which must not exceed 8.2 metres in length, exclusive of hitch or tongue; and

      - One utility trailer, which must not exceed 8.2 metres in length, exclusive of hitch or tongue.

    - The storage of a boat, personal watercraft, all-terrain vehicle, motor home, travel trailer, tent trailer, camper, snowmobile or other recreational vehicle, or a utility trailer permitted by Clause 7.6.3. must be in an interior yard or rear yard, and must not be:

      - closer than 1.0 metre from any lot line; and

      - in any required parking space or sight triangle.

#### Temporary Parking of Recreational Vehicles, Watercraft, and Trailers in Driveways in Residential Zones

    - Despite Clauses 7.6.3. and 7.6.4., in an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, HAM Zone, RUR Zone, LSR Zone or DR Zone, the temporary parking of the following vehicles, watercraft and trailers is permitted in a portion of a driveway in a front yard, or in an exterior yard on a lot between April 1 and October 31 of each year:

      - One boat which must not exceed 8.2 metres in length;

      - A maximum combined total of one motor home or travel trailer or tent trailer or camper, which must not exceed 8.2 metres in length, exclusive of hitch or tongue;

      - No more than two personal watercraft;

      - One utility trailer, which must not exceed 8.2 metres in length, exclusive of hitch or tongue; and

      - No more than two all-terrain vehicles or a similar recreational vehicle.

    - Despite Clauses 7.6.3. and 7.6.4., in an Urban Residential Zone, Urban Multi-Unit Residential Zone, HCD1 Zone, HCD3 Zone, HAM Zone, RUR Zone, LSR Zone or DR Zone, the temporary parking of the following vehicles and trailers is permitted in a portion of a driveway in a front yard, or in an exterior yard on a lot between November 1 of each year and March 31 of the following year:

      - No more than two snowmobiles and associated trailers.

    - The temporary parking of the vehicles, watercraft and trailers permitted by Clauses

7.6.5. and 7.6.6. must not be:

- Closer than 1.0 metre to any lot line;

- Located in any required parking space or sight triangle; and

- Located in a parking lot on a lot with a mixed use building or an apartment building.

# Section 8:	Rural Zones

### All Rural Zones

    - For the purposes of this By-law, Rural Zones include: Prime Agricultural Area Zone (AG), General Rural Area Zone (RU), Rural Residential Zone (RUR), Limited Service Rural Residential Zone (LSR), Rural Commercial Zone (RC) and Hamlet Zone (HAM).

    - Uses permitted in Rural Zones are limited to the uses identified in Table 8.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 8.1.2., the following provisions apply:

      - Is permitted only as an accessory use to a principal use on the lot.

      - New houses are permitted only as accessory uses where an agricultural use is the principal use of the lot. houses existing as of the date of passing of this By-law are permitted to be principal uses.

      - Despite the definition of recreation facility, such use may take place outside and is not required to operate wholly within an enclosed building.

      - Is permitted only as a complementary use to a principal agricultural use on the lot.

(By-Law Number 2022-62; 2024-332)

#### Table 8.1.2. - Permitted Uses in the Rural Zones

| Use | AG | RU | RUR | LSR | RC | HAM |
| --- | --- | --- | --- | --- | --- | --- |
| Residential dwelling unit in a mixed use building | — | — | — | — | ●1 | ● |
| house | ●2 | ● | ● | ● | ●1 | ● |
| semi-detached house | — | — | — | — | — | ● |
| Other agricultural sales establishment | — | ● | — | — | ● | ● |
| agricultural use | ● | ● | — | — | — | — |
| agri-tourism | ●4 | ●4 | — | — | — | — |

| Use | AG | RU | RUR | LSR | RC | HAM |
| --- | --- | --- | --- | --- | --- | --- |
| animal care | — | — | — | — | ● | ● |
| automobile sales establishment | — | — | — | — | — | ● |
| banquet hall | — | — | — | — | ● | ● |
| building supply store | — | — | — | — | — | ● |
| campground | — | — | — | — | ● | — |
| cemetery | — | ● | — | — | — | ● |
| community centre | — | ● | ● | — | ● | ● |
| creativity centre | — | — | — | — | — | ● |
| club | — | — | — | — | ● | ● |
| day care centre | — | — | — | — | — | ● |
| elementary school | — | ● | ● | — | — | ● |
| feedmill | — | ● | — | — | ● | ● |
| fitness centre | — | — | — | — | ● | ● |
| forestry use | ● | ● | — | — | — | — |
| garden centre | — | — | — | — | ● | ● |
| service station | — | — | — | — | — | ● |
| golf course | — | — | — | — | ● | — |
| grocery store | — | — | — | — | — | ● |
| hotel | — | — | — | — | ● | ● |
| kennel | ● | ● | — | — | — |  |
| laundry store | — | — | — | — | — | ● |
| library | — | ● | ● | — | ● | ● |
| marina | — | — | — | — | ● | — |
| museum | — | ● | ● | — | ● | ● |
| office | — | — | — | — | ●1 | ● |
| outdoor storage | — | — | — | — | ●1 | ●1 |
| personal service shop | — | — | — | — | — | ● |
| place of worship | — | ● | ● | — | ● | ● |

| Use | AG | RU | RUR | LSR | RC | HAM |
| --- | --- | --- | --- | --- | --- | --- |
| recreation facility | — | — | — | — | ●3 | ● |
| recreational vehicle sales establishment | — | — | — | — | — | ● |
| repair shop | — | — | — | — | — | ● |
| restaurant | — | — | — | — | — | ● |
| retail store | — | — | — | — | ●1 | ● |
| rural use | ●1 | ● | ●1 | ●1 | ●1 | ●1 |
| secondary school | — | — | — | — | — | ● |
| special needs facility | — | — | — | — | — | ● |
| training facility | — | — | — | — | ● | — |
| wellness clinic | — | — | — | — | — | ● |
| wholesale establishment | — | — | — | — | ●1 | ●1 |

(By-Law Number 2022-62; 2024-332)

### Prime Agricultural Area Zone (AG)

    - The use of any lot or building in the AG Zone must comply with the provisions of Table 8.2.1.

#### Table 8.2.1. – AG Provisions

| Zoning Provision | existing house | all other permitted uses |
| --- | --- | --- |
| 1. Minimum lot area (hectares) | 1.0 | 40.0 |
| 2. Minimum lot frontage (metres) | 90.0 | 90.0 |
| 3. Maximum height (metres) | — | — |
| 4. Minimum front setback (metres) | 7.6 | 7.6 |
| 5. Minimum rear setback (metres) | 7.6 | 7.6 |
| 6. Minimum exterior setback (metres) | 7.6 | 7.6 |
| 7. Minimum interior setback (metres) | 9.0 | 12.0 |
| 8. Maximum lot coverage | 10% | 35% |
| 9. Maximum number of principal dwelling units per lot | 1.0 | — |

(By-Law Number 2022-62; 2024-332)

#### Additional Provisions for Lots Zoned AG

    - In addition to the provisions of Table 8.2.1. uses in the AG Zone must comply with the following provisions:

      - When a lot with an agricultural use is enlarged by acquiring another lot with an agricultural use in the RU or AG Zones resulting in a surplus house, a severance application for the surplus house must comply with all zoning provisions of the RUR Zone in Table 8.4.1. The surplus house is deemed to be a permitted use on the severed lot;

(By-Law Number 2022-62; 2024-332

      - It is intended that agriculture-related uses or on-farm diversified uses that are not permitted as-of-right in the AG Zone may be authorized through a minor variance in accordance with the policies of the Official Plan and subject to, and in accordance with, the provisions of the Planning Act; and

#### Housing for Seasonal Farm Labour

      - Despite anything to the contrary in this By-law, housing for seasonal farm labour is permitted, subject to the following provisions:

        - A maximum of one building is permitted for seasonal farm labour;

        - Housing for seasonal farm labour may be in the form of a cabin or a

house; and

        - Housing for seasonal farm labour must comply with the setback, height

and lot coverage provisions applicable to houses.

(By-Law Number 2022-62; 2024-332)

### General Rural Area Zone (RU)

    - The use of any lot or building in the RU Zone must comply with the provisions of Table 8.3.1.

#### Table 8.3.1. – RU Provisions

| Zoning Provision | house | all other permitted uses |
| --- | --- | --- |
| 1. Minimum lot area (hectares) | 1.0 | - agricultural use: 40.0 - other uses: 4.0 |
| 2. Minimum lot frontage (metres) | 60.0 | 60.0 |
| 3. Maximum height (metres) | — | — |
| 4. Minimum front setback (metres) | 7.6 | 7.6 |
| 5. Minimum rear setback (metres) | 7.6 | 7.6 |
| 6. Minimum exterior setback (metres) | 7.6 | 7.6 |
| 7. Minimum interior setback (metres) | 9.0 | 12.0 |
| 8. Maximum lot coverage | 10% | 35% |
| 9. Maximum number of principal dwelling units per lot | 1.0 | — |

(By-Law Number 2022-62; 2024-332)

#### Additional Provisions for Lots Zoned RU

    - In addition to the provisions of Table 8.3.1. uses in the RU Zone must comply with the following provisions:

      - Where a rural use is the only principal use and no other principal use is established on a lot, a building dedicated to the rural use, including the storage of associated vehicles and/or equipment (excluding a self-storage facility), is permitted and must comply with the provisions applicable to “all other permitted uses” in Table 8.3.1., along with the following provisions:

        - the maximum gross floor area is 100 square metres;

        - the maximum height is 1 storey; and

        - a maximum of 1 rural use building is permitted per lot.

      - Despite Paragraph 1., where a rural use is established on a lot that has any other permitted principal use, a building dedicated to the rural use must comply with the provisions applicable to “all other permitted uses” in Table 8.3.1.

      - When a lot with an agricultural use is enlarged by acquiring another lot with an agricultural use in the RU or AG Zones resulting in a surplus house, a severance application for the surplus house must comply with all zoning provisions of the RUR Zone in Table 8.4.1. The surplus house is deemed to be a permitted use on the severed lot;

(By-Law Number 2022-62; 2024-332)

      - Where a lot legally existed as of the date of passing of this By-law, is less than

1.0 hectare in area and was developed with a house as of the date of passing of this By-law, the standards of the RUR Zone in Table 8.4.1. apply to the lot; and

      - It is intended that agriculture-related uses or on-farm diversified uses that are not permitted as-of-right in the RU Zone may be authorized through a minor variance in accordance with the policies of the Official Plan and subject to, and in accordance with, the provisions of the Planning Act; and

(By-Law Number 2022-62; 2024-332)

#### Housing for Seasonal Farm Labour

      - Despite anything to the contrary in this By-law, housing for seasonal farm labour is permitted, subject to the following provisions:

        - A maximum of 1 building is permitted for seasonal farm labour;

        - Housing for seasonal farm labour may be in the form of a cabin or a

house; and

        - Housing for seasonal farm labour must comply with the setback, height

and lot coverage provisions applicable to houses.

(By-Law Number 2022-62; 2024-332)

### Rural Residential Zone (RUR)

    - The use of any lot or building in the RUR Zone must comply with the provisions of Table 8.4.1.

#### Table 8.4.1. – RUR Provisions

| Zoning Provision | House | all other permitted uses |
| --- | --- | --- |
| 1. Minimum lot area | 1.0 hectare | - Private Services – 4,000.0 square metres - Partial Services – 1,393.6 square metres |
| 2. Minimum lot frontage (metres) | - Private Services – 36.5 - Partial Services – 30.5 | - Private Services – 36.5 - Partial Services – 30.5 |
| 3. Maximum height (metres) | 10.7 | 10.7 |
| 4. Minimum front setback (metres) | 6.1 | 6.1 |
| 5. Minimum rear setback (metres) | 7.5 | 7.5 |
| 6. Minimum exterior setback (metres) | 6.1 | 6.1 |
| 7. Minimum interior setback (metres) | 1.2 | 4.5 |
| 8. Maximum lot coverage | 10% | — |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum number of principal buildings per lot | 1.0 | 1.0 |
| 11. Maximum number of principal dwelling units per lot | 1.0 | — |

(By-Law Number 2022-62; 2024-332)

#### Additional Provisions for Lots Zoned RUR

    - In addition to the provisions of Table 8.4.1., uses in the RUR Zone must comply with the following provisions:

      - Despite the minimum lot area and lot frontage requirements in Table 8.4.1., where a lot was created through a plan of subdivision under the Planning Act as of the date of passing of this By-law and the lot area and/or lot frontage

are less than what is required by Table 8.4.1., the existing lot area and/or lot frontage are deemed to comply with this By-law.

### Limited Service Rural Residential Zone (LSR)

    - The use of any lot or building in the LSR Zone must comply with the provisions of Table 8.5.1.

Table 8.5.1. – LSR Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | As existing on the date of passing of this By-law |
| 2. Minimum lot frontage (metres) | As existing on the date of passing of this By-law |
| 3. Maximum height (metres) | 9.0 |
| 4. Minimum front setback (metres) | 7.5 |
| 5. Minimum rear setback (metres) | 7.5 |
| 6. Minimum exterior setback (metres) | 7.5 |
| 7. Minimum interior setback (metres) | 4.5 |
| 8. Maximum lot coverage | 15% |
| 9. Maximum number of principal buildings per lot | 1.0 |
| 10. Minimum landscaped open space | 30% |
| 11. Minimum setback from a right- of-way (metres) | 7.5 |
| 12. Maximum number of principal dwelling units per lot | 1.0 |

### Rural Commercial Zone (RC)

    - The use of any lot or building in the RC Zone must comply with the provisions of Table 8.6.1.

#### Table 8.6.1. – RC Provisions

| Zoning Provision | House | all other permitted uses |
| --- | --- | --- |
| 1. Minimum lot area (hectares) | 1.0 | 1.0 |
| 2. Minimum lot frontage (metres) | 30.0 | - Private Services: 45.0 - Partial Services: 30.0 |
| 3. Maximum height (metres) | 12.0 | 12.0 |
| 4. Minimum front setback (metres) | 3.0 | 3.0 |
| 5. Minimum rear setback (metres) | 7.5 | 7.5 |
| 6. Minimum exterior setback (metres) | 3.0 | 3.0 |
| 7. Minimum interior setback (metres) | 0.0 | 0.01 |
| 8. Minimum landscaped open space | 10% | 10% |
| 9. Maximum number of principal dwelling units per lot | 1.0 | 1.0 |

(By-Law Number 2022-62; 2024-332)

#### Additional Provisions for Lots Zoned RC

    - In addition to the provisions of Table 8.6.1., uses in the RC Zone must comply with the following provisions:

      - Where an interior lot line abuts a Zone other than a Rural Commercial Zone or Rural Institutional Zone, the minimum interior setback is 6.0 metres;

      - Outdoor storage is only permitted in the rear yard, except for outdoor storage associated with a marina, which is permitted in any yard; and

      - Despite Subclause 2., in the case of an automobile sales establishment, outdoor storage is permitted provided that it is located a minimum of 1.0 metre from a street line.

### Hamlet Zone (HAM)

    - The use of any lot or building in the HAM Zone must comply with the provisions of Table 8.7.1.

#### Table 8.7.1. – HAM Provisions

| Zoning Provision | Residential uses | dwelling unit in a mixed use building | Non- Residential Uses |
| --- | --- | --- | --- |
| 1. Minimum lot area (hectares) | 1.0 | 1.0 | 1.0 |
| 2. Minimum lot frontage (metres) | 30.0 | 30.0 | 30.0 |
| 3. Maximum height (metres) | 10.7 | 10.7 | 10.7 |
| 4. Minimum front setback (metres) | 6.0 | 3.0 | 3.0 |
| 5. Minimum rear setback (metres) | 7.5 | 7.5 | 7.5 |
| 6. Minimum exterior setback (metres) | 6.0 | 3.0 | 3.0 |
| 7. Minimum interior setback (metres) | 1.2 | 6.01 | 6.01 |
| 8. Minimum landscaped open space | 30% | 30% | 30% |
| 9. Maximum number of principal buildings per lot | 1.0 | — | — |
| 10. Maximum number of principal dwelling units per lot | 2.0 | 2.0 | — |

(By-Law Number 2022-62; 2024-332)

#### Additional Provisions for Lots Zoned HAM

    - In addition to the provisions of Table 8.7.1., uses in the HAM Zone must comply with the following provisions:

      - Where the interior lot line is adjacent to a lot that is not developed with a house, or semi-detached house, there is no minimum interior setback requirement;

      - Where an existing building was originally designed and constructed for a non- residential use, such building is permitted to be converted to a house, or semi-detached house;

      - Outdoor storage is only permitted as an accessory use to a non-residential use where such lot is not adjacent to a lot that is developed with a house, or semi-detached house, subject to the following provisions:

        - outdoor storage is only permitted in the rear yard; and

        - outdoor storage must comply with the setback provisions applicable to the principal building.

      - Despite Subclause 8.7.2.2., when outdoor storage is established under the provisions of this By-law, the subsequent construction of a new house, or semi-detached house, on an adjacent lot does not render such outdoor storage non-complying; and

      - Despite the minimum lot area and lot frontage requirements in Table 8.7.1., where a lot was created through a plan of subdivision under the Planning Act as of the date of passing of this By-law and the lot area and/or lot frontage are less than what is required by Table 8.7.1., the existing lot area and/or lot frontage are deemed to comply with this By-law.

(By-Law Number 2022-62; 2024-332)

# Section 9:	Rural Industrial Zones

### All Rural Industrial Zones

    - For the purposes of this By-law, Rural Industrial Zones include Rural Industrial Zone (RM1), Rural Heavy Industrial Zone (RM2) and Mineral Resource and Extraction Zone (MX1).

    - Uses permitted in Rural Industrial Zones are limited to the uses identified in Table 9.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 9.1.2., the following corresponding provisions apply:

      - Is permitted only as an accessory use to a principal use on a lot.

Table 9.1.2. - Permitted Uses in the Rural Industrial Zones

| Use | RM1 | RM2 | MX1 |
| --- | --- | --- | --- |
| agricultural use | — | — | ● |
| automobile body shop | ● | — | — |
| automobile repair shop | ● | — | — |
| building supply store | ● | — | — |
| contractor’s yard | ● | — | — |
| feedmill | ● | — | — |
| forestry use | — | — | ● |
| gravel pit | — | — | ● |
| heavy equipment or truck repair shop | ● | ● | — |
| heavy industrial uses | — | ● | — |
| industrial repair shop | ● | — | — |
| light industrial use | ● | ● | — |
| mineral aggregate operation | — | — | ● |
| outdoor storage | ●1 | ● | ● |

| Use | RM1 | RM2 | MX1 |
| --- | --- | --- | --- |
| repair shop | ● | ● | — |
| salvage yard | — | ● | — |
| transformer station | — | ● | — |
| transportation depot | ● | ● | — |
| transportation terminal | ● | — | — |
| warehouse | ● | ● | — |
| water supply plant | — | ● | — |
| workshop | ● | — | — |

### Rural Industrial Zone (RM1)

    - The use of any lot or building in the RM1 Zone must comply with the provisions of Table 9.2.1.

#### Table 9.2.1. – RM1 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (hectares) | 1.0 |
| 2. Minimum lot frontage (metres) | 30.0 |
| 3. Maximum height (metres) | 20.0 |
| 4. Minimum front setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 24.0 - all other lots: 15.0 |
| 5. Minimum rear setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 15.0 - all other lots: 7.51 |
| 6. Minimum exterior setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 24.0 - all other lots: 15.0 |
| 7. Minimum interior setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 6.1 - all other lots: 3.01 |
| 8. Minimum landscaped open space | 10% |
| 9. Maximum lot coverage | 35% |

Additional Provisions for Lots Zoned RM1

    - In addition to the provisions of Table 9.2.1., uses in the RM1 Zone must comply with the following provisions:

      - Outdoor storage must:

        - Comply with setback and lot coverage provisions of this Zone as if the

outdoor storage were a building;

        - Not be located in a front yard or exterior yard; and

        - Be provided with a visual screen with a minimum height of 1.8 metres and in such a manner that the outdoor storage is not visible from a street or a lot that is located in any Zone other than a Rural Industrial Zone.

(By-Law Number 2022-62; 2024-332)

### Rural Heavy Industrial Zone (RM2)

    - The use of any lot or building in the RM2 Zone must comply with the provisions of Table 9.3.1.

#### Table 9.3.1. – RM2 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (hectares) | 1.0 |
| 2. Minimum lot frontage (metres) | 30.0 |
| 3. Maximum height (metres) | — |
| 4. Minimum front setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 24.0 - all other lots: 15.0 |
| 5. Minimum rear setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 15.0 - all other lots: 7.51 |
| 6. Minimum exterior setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 24.0 - all other lots: 15.0 |
| 7. Minimum interior setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 12.0 - all other lots: 3.01 |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | — |

Additional Provisions for Lots Zoned RM2

    - In addition to the provisions of Table 9.3.1., uses in the RM2 Zone must comply with the following provisions:

      - No interior setback or rear setback is required along any portion of a lot line

which abuts a railroad right-of way.

### Mineral Resource and Extraction Zone (MX1)

    - The use of any lot or building in the MX1 Zone must comply with the provisions of Table 9.4.1.

#### Table 9.4.1. – MX1 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (hectares) | 1.0 |
| 2. Minimum lot frontage (metres) | 30.0 |
| 3. Maximum height (metres) | — |
| 4. Minimum front setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 30.0 - all other lots: 22.04 |
| 5. Minimum rear setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 30.0 - all other lots: 15.01,2,3,4 |
| 6. Minimum exterior setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 30.0 - all other lots: 22.04 |
| 7. Minimum interior setback (metres) | - where abutting a lot that is located in any Zone other than a Rural Industrial Zone: 30.0 - all other lots: 15.01,2,3,4 |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | — |

Additional Provisions for Lots Zoned MX1

    - In addition to the provisions of Table 9.4.1., uses in the MX1 Zone must comply with the following provisions:

      - No interior setback or rear setback is required along any portion of a lot line

which abuts a railroad right-of way;

      - Where a gravel pit abuts a lot in a Zone that permits a residential use or a DR Zone, minimum interior setback and rear setback of a gravel pit: 60.0 metres;

      - Where a mineral aggregate operation abuts a lot in a Zone that permits a residential use or a DR Zone, the minimum interior setback and rear setback of a mineral aggregate operation is 60.0 metres; and

      - A building, plant or product stockpile must:

        - Have a minimum setback of 30.5 metres from any lot line; and

        - Have a minimum setback of 91.4 metres of any lot line or part thereof which abuts a lot in a Zone that permits a residential use or a DR Zone.

# Section 10: Mixed Use Zones

### All Mixed Use Zones

    - For the purposes of this By-law, Mixed Use Zones include Williamsville Zone 1 (WM1), Williamsville Zone 2 (WM2), Downtown Zone 1 (DT1), Downtown Zone 2 (DT2), Mixed Zone 1 (MU1), Mixed Zone 2 (MU2), and Mixed Zone 3 (MU3).

(By-Law Number 2022-62; 2025-118)

    - Uses permitted in Mixed Use Zones are limited to the uses identified in Table 10.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

#### Table 10.1.2. - Permitted Uses in the Mixed Use Zones

| Use | WM1, WM2 | DT1, DT2 | MU1, MU2, MU3 |
| --- | --- | --- | --- |
| Residential apartment building | ● | ● | ● |
| dwelling unit in a mixed use building | ● | ● | ● |
| stacked townhouse | ● | — | ● |
| townhouse | ● |  | ● |
| Non-residential animal care | — | ● | — |
| automobile sales establishment | — | ● | — |
| banquet hall | ● | ● | ● |
| building supply store | — | ● | — |
| catering service | — | — | ● |
| commercial parking lot | — | ● | — |
| community centre | ● | ● | ● |
| club | ● | ● | ● |
| creativity centre | ● | ● | ● |
| day care centre | ● | ● | ● |
| department store | — | ● | — |

| Use | WM1, WM2 | DT1, DT2 | MU1, MU2, MU3 |
| --- | --- | --- | --- |
| entertainment establishment | ● | ● | ● |
| financial institution | ● | ● | ● |
| fitness centre | ● | ● | ● |
| service station | — | ● | — |
| grocery store | ● | ● | ● |
| funeral establishment | ● | ● | ● |
| hotel | ● | ● | ● |
| laboratory | ● | — | ● |
| laundry store | ● | ● | ● |
| library | ● | ● | ● |
| museum | ● | ● | ● |
| office | ● | ● | ● |
| personal service shop | ● | ● | ● |
| place of worship | ● | ● | ● |
| production studio | — | — | ● |
| public market | ● | ● | ● |
| recreation facility | ● | ● | ● |
| repair shop | — | ● | — |
| research establishment | — | — | ● |
| restaurant | ● | ● | ● |
| retail store | ● | ● | ● |
| special needs facility | ● | — | ● |
| training facility | — | ● | ●1 |
| transportation depot | — | ● | — |
| transportation terminal | — | ● | — |
| wellness clinic | ● | ● | ● |
| workshop | — | — | ●1 |

(By-Law Number 2022-62; 2025-118)

    - Where a permitted use includes a reference number in superscript beside the “●” symbol in Table 10.1.2., the following corresponding provision applies:

1. Is required to operate within an enclosed building.

(By-Law Number 2022-62; 2025-118)

### Williamsville Zone 1 (WM1)

    - Despite the definitions of front lot line and rear lot line, in the WM1 Zone:

      - Where a lot has a street line adjacent to Princess Street, the street line

adjacent to Princess Street is deemed to be the front lot line; and

      - Where a lot has a street line adjacent to Princess Street, all lot lines that separate the lot from a Zone outside the WM1, WM2 or a Commercial Zone are considered a rear lot line.

    - The use of any lot or building in the WM1 Zone must comply with the provisions of Table 10.2.1.

#### Table 10.2.1. – WM1 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | The lesser of: - 20 metres - 6 storeys |
| 4. Minimum and maximum front setback (metres) | - front lot line on Princess Street, Division Street, Concession Street or Bath Road: - first storey: minimum 3.0 metres, maximum 5.0 metres - storeys 2 to 4: minimum 2.0 metres, maximum 5.0 metres (b) front lot line on all other streets: (i) storeys 1 to 4: minimum 2.0 metres, maximum 5.0 metres |
| 5. Minimum rear setback (metres) | 8.0 |
| 6. Minimum exterior setback (metres) | (a) exterior lot line on Division Street, Concession Street or Bath Road: (i) first storey: minimum 3.0 metres, maximum 5.0 metres |

| Zoning Provision | all permitted uses |
| --- | --- |
|  | (ii) storeys 2 to 4: minimum 2.0 metres, maximum 5.0 metres (b) exterior lot line on all other streets: (i) storeys 1 to 4: minimum 2.0 metres, maximum 5.0 metres |
| 7. Minimum interior setback (metres) | - where a lot has front lot line on Princess Street: 0.0 metres - all other lots: 1.2 metres |
| 8. Minimum and maximum stepbacks | (a) Front lot line and/or exterior lot line: (i) storey 5 to 6: minimum 2.0 metres from the exterior wall of the 4th storey |
| 9. Minimum landscaped open space | — |
| 10. Maximum lot coverage | 70% |
| 11. Maximum residential density | 210 dwelling units per net hectare |
| 12. Minimum streetwall height for all buildings | 10.5 metres |
| 13. Minimum floor to floor height of first storey | 4.5 metres |

Additional Provisions for Lots Zoned WM1

    - In addition to the provisions of Table 10.2.1., uses in the WM1 Zone must comply with the following provisions:

#### Front Setback of Streetwall

      - A minimum of 75% of the streetwall of a building must be built to the minimum required front setback for the height of the streetwall.

#### Projecting Balconies

      - Balconies that project from the exterior wall of a building are only permitted above the fourth storey where the exterior wall faces a street line, and may only project to a maximum depth of 1.5 metres from the wall.

      - Balconies that project from the exterior wall of a building are only permitted above the second storey where the exterior wall faces a lot line that is not a street line, and may only project to a maximum depth of 2.0 metres from the wall.

#### Ground Floor Commercial Uses

      - Buildings fronting on Princess Street are required to have ground floor commercial uses on the first storey where any portion of the lot aligns with the area identified as “Required Ground Floor Commercial” on Schedule 3 of this By-Law.

      - Where ground floor commercial uses are required by Subclause 4., the entire streetwall of the first storey, excluding areas devoted to a lobby or other shared entrances/exits for other permitted uses, must be occupied by commercial uses. Portions of the floor area of the first storey that do not have an exterior wall facing a street line may be occupied by uses that service the building such as loading spaces, waste management facilities and rooms, mechanical rooms, bike parking facilities and other similar uses.

#### Loading and Parking

      - Loading spaces, parking spaces and parking lots are not permitted in a

front yard or exterior yard.

### Williamsville Zone 2 (WM2)

    - For the purpose of the WM2 Zone, the following definitions apply:

      - Podium means the base component of any building that is no greater than 20 meters in height (excluding mechanical penthouses) and only includes the first storey through sixth storeys of such building.

      - Tower means any portion of any building that is greater than 20 metres in height, excluding a podium, below grade building components and mechanical penthouses.

    - Despite the definitions of front lot line and rear lot line, in the WM2 Zone:

      - Where a lot has a street line adjacent to Princess Street, the street line

adjacent to Princess Street is deemed to be the front lot line; and

      - Where a lot has a street line adjacent to Princess Street, all lot lines that separate the lot from a Zone outside the WM1, WM2 or a Commercial Zone are considered a rear lot line.

    - The use of any lot or building in the WM2 Zone must comply with the provisions of Table 10.3.1.

#### Table 10.3.1. – WM2 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | The lesser of: - 61.5 metres - 20 storeys |
| 4. Minimum and maximum front setback (metres) | - front lot line on Princess Street, Division Street, Concession Street or Bath Road: - first storey: minimum 3.0 metres, maximum 5.0 metres - storeys 2 to 4: minimum 2.0 metres, maximum 5.0 metres (b) front lot line on all other streets: |

| Zoning Provision | all permitted uses |
| --- | --- |
|  | (i) storeys 1 to 4: minimum 2.0 metres, maximum 5.0 metres |
| 5. Minimum rear setback (metres) | 8.0 |
| 6. Minimum exterior setback (metres) | - exterior lot line on Division Street, Concession Street or Bath Road: - first storey: minimum 3.0 metres, maximum 5.0 metres - storeys 2 to 4: minimum 2.0 metres, maximum 5.0 metres - exterior lot line on all other streets: - storeys 1 to 4: minimum 2.0 metres, maximum 5.0 metres |
| 7. Minimum interior setback (metres) | - where a lot has front lot line on Princess Street: 0.0 metres - all other lots: 1.2 metres |
| 8. Minimum and maximum stepbacks | (a) Front lot line and/or exterior lot line: (i) storey 5 to 6: minimum 2.0 metres from the exterior wall of the 4th storey |
| 9. Minimum landscaped open space | — |
| 10. Maximum lot coverage | 70% |
| 11. Maximum residential density | - where a tower is developed: 480 dwelling units per net hectare - all other lots: 210 dwelling units per net hectare |
| 12. Minimum streetwall height for all buildings | 10.5 metres |
| 13. Minimum floor to floor height of first storey | 4.5 metres |

Additional Provisions for Lots Zoned WM2

    - In addition to the provisions of Table 10.3.1., uses in the WM2 Zone must comply with the following provisions:

#### Front Setback of Streetwall

      - A minimum of 75% of the streetwall of a building must be built to the minimum required front setback for the height of the streetwall.

#### Projecting Balconies

      - Balconies that project from the exterior wall of a building are only permitted above the fourth storey where the exterior wall faces a street line, and may only project to a maximum depth of 1.5 metres from the wall.

      - Balconies that project from the exterior wall of a building are only permitted above the second storey where the exterior wall faces a lot line that is not a street line, and may only project to a maximum depth of 2.0 metres from the wall.

#### Ground Floor Commercial Uses

      - Buildings fronting on Princess Street are required to have ground floor commercial uses on the first storey where any portion of the lot aligns with the area identified as “Required Ground Floor Commercial” on Schedule 3.

      - Where ground floor commercial uses are required by Subclause 4., the entire streetwall of the first storey, excluding areas devoted to a lobby or other shared entrances/exits for other permitted uses, must be occupied by commercial uses. Portions of the floor area of the first storey that do not have an exterior wall facing a street line may be occupied by uses that service the building such as loading spaces, waste management facilities and rooms, mechanical rooms, bike parking facilities and other similar uses.

#### Tower Conditions

      - The maximum floor plate of a tower is 790 square metres. Tower floor plate includes all areas enclosed within exterior walls, including hallways, elevators, stairs, mechanical shafts, and all similar components.

      - The minimum separation distance between a tower and another tower is

25.0 metres.

      - The minimum setback from a tower to a lot line shared with an adjacent property is 12.5 metres.

      - Despite Subclause 8., where an adjacent property has already been developed with a tower, the tower is permitted to be located closer than 12.5 metres to the lot line shared with that adjacent property so long as the 25.0 metre tower separation distance is maintained.

      - The minimum setback from a tower to the exterior wall of the podium is 2.0 metres.

#### Loading and Parking

      - Loading spaces, parking spaces and parking lots are not permitted in a

#### front yard or exterior yard.

Density Exception

      - The maximum residential density provisions of Table 10.3.1. do not apply to the properties municipally known as 429 and 445 Princess Street.

### Downtown Zone 1 (DT1)

    - The use of any lot or building in the DT1 Zone must comply with the provisions of Table 10.4.1.

#### Table 10.4.1. – DT1 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Minimum height | 2 storeys, not less than 8.5 metres |
| 4. Maximum height (metres) | - Height at build-to-plane: 4 storeys, not to exceed 17.0 metres - Height under angular plane: 6 storeys, not to exceed 25.5 metres |
| 5. Minimum front setback (metres) | See subclause 10.4.2.1. |
| 6. Minimum rear setback (metres) | See subclause 10.4.2.2. |
| 7. Minimum exterior setback (metres) | 0.0 |
| 8. Minimum interior setback (metres) | See subclause 10.4.2.2. |
| 9. Minimum landscaped open space | — |
| 10. Minimum lot coverage | 50% |
| 11. Maximum lot coverage | 100% |
| 12. Maximum number of dwelling units per lot | 123 dwelling units per net hectare |
| 13. Applicable angular plane | 39 degrees |
| 14. Height for commencement of angular plane | 17.0 |

Additional Provisions for Lots Zoned DT1

    - In addition to the provisions of Table 10.4.1., uses in the DT1 Zone must comply with the following provisions:

      - The minimum front setback and build-to-plane is the average of the existing front setback of immediately adjacent buildings fronting on the same street, subject to the following provisions:

        - Where immediately adjacent buildings have different front setbacks, the front setback must be the same as the immediately adjacent building closest to the street line; and

        - Where no adjacent buildings exist within the same block, fronting on the same street, the front setback and build-to-plane is at the street line;

      - The required minimum interior setback and rear setback is 0.0 metres, except where the interior lot line or rear lot line abuts a lot in an Urban Residential Zone, HCD1 Zone or HCD3 Zone, the minimum interior setback or rear setback is increased to half the height of the building or 6.0 metres, whichever is greater;

      - Conversion of commercial uses in buildings existing as of the date of passing of this By-law for residential use is permitted, subject to compliance with all provisions of this By-law except provisions regulating minimum setbacks, amenity area and maximum lot coverage do not apply;

      - Commercial parking lot, parking spaces and parking lots are only permitted in the interior yard or rear yard;

      - Despite Clause 4.23.1., the minimum separation distance from the high water mark of a waterbody for any use or building is 10.0 metres;

      - The minimum width of a waterfront walkway is 10.0 metres;

      - All waterfront walkways must be designed so that they are open from the ground to the sky and not covered by any component of any building;

      - For parking structures, the following provisions do not apply:

        - The build-to-plane provisions;

        - The minimum height provisions; and

        - The minimum lot coverage provisions;

      - No loading spaces are required;

      - Drive-throughs are prohibited;

      - Buildings are required to have ground floor commercial uses on the first storey where any portion of the lot aligns with the area identified as “Required Ground Floor Commercial” on Schedule 3.

      - Where ground floor commercial uses are required by Subclause 11., the entire streetwall of the first storey, excluding areas devoted to a lobby or other shared entrances/exits for other permitted uses, must be occupied by commercial uses. Portions of the floor area of the first storey that do not have an exterior wall facing a street line may be occupied by uses that service the building such as loading spaces, waste management facilities and rooms, mechanical rooms, bike parking facilities and other similar uses.

(By-Law Number 2022-62; 2024-332)

### Downtown Zone 2 (DT2)

    - The use of any lot or building in the DT2 Zone must comply with the provisions of Table 10.5.1.

#### Table 10.5.1. – DT2 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Minimum height | 2 storeys, not less than 8.5 metres |
| 4. Maximum height (metres) | 4 storeys, not to exceed 17.0 metres |
| 5. Minimum front setback (metres) | See subclause 10.5.2.1. |
| 6. Minimum rear setback (metres) | See subclause 10.5.2.2. |
| 7. Minimum exterior setback (metres) | 0.0 |
| 8. Minimum interior setback (metres) | See subclause 10.5.2.2. |
| 9. Minimum landscaped open space | — |
| 10. Minimum lot coverage | 50% |
| 11. Maximum lot coverage | 100% |
| 12. Maximum number of dwelling units per lot | 123 dwelling units per net hectare |

Additional Provisions for Lots Zoned DT2

    - In addition to the provisions of Table 10.5.1., uses in the DT2 Zone must comply with the following provisions:

      - The minimum front setback and build-to-plane is the average of the existing front setback of immediately adjacent buildings fronting on the same street, subject to the following provisions:

        - Where immediately adjacent buildings have different front setbacks, the front setback must be the same as the immediately adjacent building closest to the street line; and

        - Where no adjacent buildings exist within the same block, fronting on the same street, the front setback and build-to-plane is at the street line;

      - The required minimum interior setback and rear setback is 0.0 metres, except where the interior lot line or rear lot line abuts a lot in an Urban Residential Zone, HCD1 Zone or HCD3 Zone, the minimum interior setback or rear setback is increased to half the height of the building or 6.0 metres, whichever is greater;

      - Conversion of commercial buildings existing as of the date of passing of this By-law for residential use is permitted, subject to compliance with all provisions of this By-law except provisions regulating minimum setbacks, amenity area and maximum lot coverage do not apply;

      - Commercial parking lot, parking spaces and parking lots are only permitted in the interior yard or rear yard;

      - Despite Clause 4.23.1., the minimum separation distance from the high water mark of a waterbody for any use or building is 10.0 metres;

      - The minimum width of a waterfront walkway is 10.0 metres;

      - All waterfront walkways must be designed so that they are open from the ground to the sky and not covered by any component of any building;

      - For parking structures, the following provisions do not apply:

        - The build-to-plane provisions;

        - The minimum height provisions; and

        - The minimum lot coverage provisions;

      - No loading spaces are required;

      - Drive-throughs are prohibited;

      - Buildings are required to have ground floor commercial uses on the first storey where any portion of the lot aligns with the area identified as “Required Ground Floor Commercial” on Schedule 3.

      - Where ground floor commercial uses are required by Subclause 11., the entire streetwall of the first storey, excluding areas devoted to a lobby or other shared entrances/exits for other permitted uses, must be occupied by

commercial uses. Portions of the floor area of the first storey that do not have an exterior wall facing a street line may be occupied by uses that service the building such as loading spaces, waste management facilities and rooms, mechanical rooms, bike parking facilities and other similar uses.

### Mixed Zone 1 (MU1)

    - The use of any lot or building in the MU1 Zone must comply with the provisions of Table 10.6.1.

#### Table 10.6.1. – MU1 Provisions are added as follows:

| Zoning Provision | All permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Minimum streetwall height (metres) | 10.5 |
| 4. Minimum floor to floor height of first storey (metres) | 4.5 |
| 5. Maximum height | The lesser of: - 20 metres - 6 storeys |
| 6. Minimum front setback (metres) | 3.0 |
| 7. Minimum rear setback (metres) | 10.0 |
| 8. Minimum exterior setback (metres) | 3.0 |
| 9. Minimum interior setback (metres) | 3.0 |
| 10. Minimum stepbacks (metres) | (a) Front lot line and exterior lot line: i. storey 5 to 6: minimum 3.0 metres from the exterior wall of the 4th storey |
| 11. Minimum landscaped open space | 15% |
| 12. Minimum lot coverage | 55% |
| 13. Maximum residential density | 175 dwelling units per net hectare |

(By-Law Number 2022-62; 2025-118)

#### Additional Provisions for Lots Zoned MU1

    - In addition to the provisions of Table 10.6.1., uses in the MU1 Zone must comply with the following provisions:

#### Front Setback of Streetwall

      - A minimum of 75% of the streetwall of a building must be built to the minimum required front setback for the height of the streetwall.

#### Ground Floor Commercial Uses

      - Buildings are required to have ground floor commercial uses on the first storey where any portion of the lot aligns with the area identified as “Required Ground Floor Commercial” on Schedule 3 of this By-law.

      - Where ground floor commercial uses are required by Subclause 2., the entire streetwall of the first storey, excluding areas devoted to a lobby or other shared entrances/exits for other permitted uses, must be occupied by commercial uses. Portions of the floor area of the first storey that do not have an exterior wall facing a street line may be occupied by uses that service the building such as loading spaces, waste management facilities and rooms, mechanical rooms, bike parking facilities and other similar uses.

#### Main Pedestrian Entrance

      - The main pedestrian entrance to the building must be located on the main wall facing the front lot line.

#### Projecting Balconies

      - Balconies are only permitted above the second storey, to a maximum projection of 1.5 metres from the main wall.

#### Loading and Parking

      - Parking structures are not permitted to occupy any part of a main wall facing a street line.

      - Loading spaces, parking spaces and parking lots are not permitted in a

#### front yard or exterior yard.

Planting Strip

      - Where a lot is adjacent to a lot in a UR Zone, a Commercial Zone or an Employment Zone, within the required rear setback, a minimum 2.0 metre wide planting strip must be provided along the full length of the rear lot line and must comply with Subclause 4.16.1.2.

#### Legally Existing Uses

      - Residential uses that legally existed prior to the passage of this By-law are deemed to be permitted uses and must comply with the provisions of the UR5 Zone.

(By-Law Number 2022-62; 2025-118)

#### Mixed Zone 2 (MU2)

    - For the purpose of the MU2 Zone, the following definitions apply:

      - Podium means the base component of any building that is no greater than 20 meters in height (excluding mechanical penthouses) and only includes the first storey through sixth storeys of such building.

      - Tower means any portion of any building that is greater than 20 metres in height, excluding a podium, below grade building components and mechanical penthouses.

    - The use of any lot or building in the MU2 Zone must comply with the provisions of Table 10.7.1.

#### Table 10.7.1. – MU2 Provisions

| Zoning Provision | All permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Minimum streetwall height (metres) | 10.5 |
| 4. Minimum floor to floor height of first storey (metres) | 4.5 |
| 5. Maximum height | The lesser of: - 50 metres - 15 storeys |
| 6. Minimum front setback (metres) | 3.0 |
| 7. Minimum rear setback (metres) | 10.0 |
| 8. Minimum exterior setback (metres) | 3.0 |
| 9. Minimum interior setback (metres) | 3.0 |

| 10. Minimum stepbacks (metres) | (a) Front lot line and exterior lot line: i. storey 5 to 6: minimum 3.0 metres from the exterior wall of the 4th storey |
| --- | --- |
| 11. Minimum landscaped open space | 15% |
| 12. Maximum lot coverage | 55% |
| 13. Maximum residential density | 300 dwelling units per net hectare |

Additional Provisions for Lots Zoned MU2

    - In addition to the provisions of Table 10.7.1., uses in the MU2 Zone must comply with the following provisions:

#### Front Setback of Streetwall

      - A minimum of 75% of the streetwall of a building must be built to the minimum required front setback for the height of the streetwall.

#### Ground Floor Commercial Uses

      - Buildings are required to have ground floor commercial uses on the first storey where any portion of the lot aligns with the area identified as “Required Ground Floor Commercial” on Schedule 3 of this By-law.

      - Where ground floor commercial uses are required by Subclause 2., the entire streetwall of the first storey, excluding areas devoted to a lobby or other shared entrances/exits for other permitted uses, must be occupied by commercial uses. Portions of the floor area of the first storey that do not have an exterior wall facing a street line may be occupied by uses that service the building such as loading spaces, waste management facilities and rooms, mechanical rooms, bike parking facilities and other similar uses.

#### Main Pedestrian Entrance

      - The main pedestrian entrance to the building must be located on the main wall facing the front lot line.

#### Projecting Balconies

      - Balconies are only permitted above the second storey, to a maximum projection of 1.5 metres from the main wall.

#### Loading and Parking

      - Parking structures are not permitted to occupy any part of a main wall

facing a street line.

      - Loading spaces, parking spaces and parking lots are not permitted in a

front yard or exterior yard.

#### Tower Conditions

      - The maximum floor plate of a tower is 800 square metres. Tower floor plate includes all areas enclosed within exterior walls, including hallways, elevators, stairs, mechanical shafts, and all similar components.

      - The minimum separation distance between a tower and another tower is

25.0 metres.

      - The minimum setback from a tower to a lot line shared with an adjacent property is 12.5 metres.

      - Despite Subclause 10., where an adjacent property has already been developed with a tower, the tower is permitted to be located closer than 12.5 metres to the lot line shared with that adjacent property so long as the 25.0 metre tower separation distance is maintained.

      - The minimum setback from a tower to the exterior wall of the podium is 2.0 metres.

#### Planting Strip

      - Where a lot is adjacent to a lot in a UR Zone, a Commercial Zone or an Employment Zone, within the required rear setback, a minimum 2.0 metre wide planting strip must be provided along the full length of the rear lot line and must comply with Subclause 4.16.1.2.

#### Legally Existing Uses

      - Residential uses that legally existed prior to the passage of this By-law are deemed to be permitted uses and must comply with the provisions of the UR5 Zone.

      - Commercial uses that legally existed prior to the passage of this By-law are deemed to be permitted uses and must comply with the provisions of the CN Zone.

(By-Law Number 2022-62; 2025-118)

#### Mixed Zone (MU3)

    - The use of any lot or building in the MU3 Zone must comply with the provisions of Table 10.8.1.

#### Table 10.8.1. – MU3 Provisions

| Zoning Provision | All permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Minimum height (storeys) | 2 |
| 4. Maximum height | The lesser of: - 4 storeys - 12.5 metres |
| 5. Minimum front setback (metres) | 3.0 |
| 6. Minimum rear setback (metres) | 8.0 |
| 7. Minimum exterior setback (metres) | 3.0 |
| 8. Minimum interior setback (metres) | - non-residential buildings: equal to half the height of the building - residential buildings up to 3 storeys: 1.2 metres - each additional storey above 3: 1.2 metres d) Despite (b) and (c), for townhouses, where a common party wall is located along a lot line: 0 metres |
| 9. Minimum landscaped open space | 30% |
| 10. Maximum lot coverage | 45% |
| 11. Maximum residential density | 90 dwelling units per net hectare |

Additional Provisions for Lots Zoned MU3

    - In addition to the provisions of Table 10.8.1., uses in the MU3 Zone must comply with the following provisions:

#### Front Setback of Streetwall

      - A minimum of 75% of the streetwall of a building must be built to the minimum required front setback for the height of the streetwall.

#### Loading and Parking

      - Loading spaces, parking spaces and parking lots are not permitted in a

#### front yard or exterior yard.

Planting Strip

      - Within the required rear setback, a minimum 2.0 metre wide planting strip must be provided along the full length of the rear lot line and must comply with Subclause 4.16.1.2.

#### Legally Existing Uses

      - Residential uses that legally existed prior to the passage of this By-law are deemed to be permitted uses and must comply with the provisions of the UR5 Zone.

(By-Law Number 2022-62; 2025-118)

# Section 11: Urban Residential Zones

### All Urban Residential Zones

    - For the purposes of this By-law, Urban Residential Zones include Urban Residential Zone 1 (UR1), Urban Residential Zone 2 (UR2), Urban Residential Zone 3 (UR3), Urban Residential Zone 4 (UR4), Urban Residential Zone 5 (UR5), Urban Residential Zone 6 (UR6), Urban Residential Zone 7 (UR7), Urban Residential Zone 8 (UR8), and Urban Residential Zone 9 (UR9).

    - Uses permitted in Urban Residential Zones are limited to the uses identified in Table 11.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 11.1.2., the following provisions apply:

      - Is only permitted on a lot that has a front lot line and/or exterior lot line on a Collector Road or Arterial Road in accordance with the street type identified in Schedule 4.

(By-Law Number 2022-62; 2024-332)

#### Table 11.1.2. – Permitted Uses in the Urban Residential Zones

| Use | UR1 | UR2 | UR3 | UR4 | UR5 | UR6 | UR7 | UR8 | UR9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Residential house | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| semi-detached house | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| townhouse | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| Non-Residential community centre | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| elementary school | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| library | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| museum | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| place of worship | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| secondary school | ●1 | ●1 | ●1 | ●1 | ●1 | ●1 | ●1 | ●1 | ●1 |

(By-Law Number 2022-62; 2024-332)

### Urban Residential Zone 1 (UR1)

    - The use of any lot or building in the UR1 Zone must comply with the provisions of Table 11.2.1.

#### Table 11.2.1. – UR1 Provisions

| Zoning Provision | House, semi-detached house and townhouse | non-residential buildings |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | — | — |
| 2. Minimum lot frontage (metres) | - house: 10.0 - semi-detached house: 9.0 per lot - townhouse: 6.0 per lot | 10.0 |
| 3. Maximum height (metres) | - flat roof: 9.0 - all other: 10.7 | - flat roof: 9.0 - all other: 10.7 |
| 4. Minimum front setback (metres) | 6.0 | 6.0 |
| 5. Minimum rear setback (metres) | 6.75 | equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | 6.0 | 6.0 |
| 7. Minimum interior setback (metres) | - 1.2 metres - Despite (a), where a common party wall is located along a lot line: 0 metres | 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |
| 8. Minimum aggregate of interior setbacks | — | — |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum lot coverage | — | — |
| 11. Maximum number of principal buildings per lot | 1.0 | — |

| Zoning Provision | House, semi-detached house and townhouse | non-residential buildings |
| --- | --- | --- |
| 12. Maximum building depth (metres) | — | — |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

### Urban Residential Zone 2 (UR2)

    - The use of any lot or building in the UR2 Zone must comply with the provisions of Table 11.3.1.

#### Table 11.3.1. – UR2 Provisions

| Zoning Provision | house, semi-detached house and townhouse | non-residential buildings |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | — | — |
| 2. Minimum lot frontage (metres) | - house: 12.0 - semi-detached house: 9.0 per lot - townhouse: 6.0 per lot | 12.0 |
| 3. Maximum height (metres) | - flat roof: 9.0 - all other: 10.7 | - flat roof: 9.0 - all other: 10.7 |
| 4. Minimum front setback (metres) | 6.0 | 6.0 |
| 5. Minimum rear setback (metres) | 7.5 | equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | 6.0 | 6.0 |
| 7. Minimum interior setback (metres) | - 1.2 metres - Despite (a), where a common party wall is located along a lot line: 0 metres | 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |
| 8. Minimum aggregate of interior setbacks | — | — |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum lot coverage | — | — |
| 11. Maximum number of principal buildings per lot | 1.0 | — |
| 12. Maximum building depth (metres) | — | — |

| Zoning Provision | house, semi-detached house and townhouse | non-residential buildings |
| --- | --- | --- |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

#### Additional Provisions for Lots Zoned UR2

    - In addition to the provisions of Table 11.3.1., the use of any lot or building in the UR2 Zone must comply with the following provisions:

      - Where a lot was vacant or was created through a plan of subdivision under the Planning Act or a description under the Condominium Act, 1998 following the date of passing of this By-law, development must comply with the following provisions:

        - Despite the minimum front setback listed in Table 11.3.1., the minimum

front setback is 3.0 metres; and

        - Despite the minimum lot frontage listed in Table 11.3.1., the minimum

lot frontage for a corner lot is:

          - 10.3 metres for a house;

          - 8.8 metres per lot for a semi-detached house; and

          - 8.6 metres per lot for a townhouse.

(By-Law Number 2022-62; 2024-332)

### Urban Residential Zone 3 (UR3)

    - The use of any lot or building in the UR3 Zone must comply with the provisions of Table 11.4.1.

#### Table 11.4.1. – UR3 Provisions

| Zoning Provision | house, semi-detached house and townhouse | non-residential buildings |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | — | — |
| 2. Minimum lot frontage (metres) | - house: 9.0 - semi-detached house: 7.5 per lot - townhouse: 6.0 per lot | 9.0 |
| 3. Maximum height (metres) | - flat roof: 9.0 - all other: 10.7 | - flat roof: 9.0 - all other: 10.7 |
| 4. Minimum front setback (metres) | - Where 2 adjacent buildings have a front lot line on the same street: the greater of 3.0 metres or the average front setbacks of adjacent buildings - Where 1 adjacent building has a front lot line on the same street: the greater of 3.0 metres or the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 3.0 metres | 6.0 |
| 5. Minimum rear setback (metres) | 6.0 | equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | 2.4 | 2.4 |
| 7. Minimum interior setback (metres) | - 1.2 metres on one side and 0.6 metres on the other side - Despite (a), where a common party wall is located along a lot line: 0 metres | 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |
| 8. Minimum aggregate of interior setbacks | — | — |

| Zoning Provision | house, semi-detached house and townhouse | non-residential buildings |
| --- | --- | --- |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum lot coverage | — | — |
| 11. Maximum number of principal buildings per lot | 1.0 | — |
| 12. Maximum building depth | — | — |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

#### Additional Provisions for Lots Zoned UR3

    - In addition to the provisions of Table 11.4.1., the use of any lot or building in the UR3 Zone must comply with the following provisions:

      - Where a lot was vacant or was created through a plan of subdivision under the Planning Act or a description under the Condominium Act, 1998 following the date of passing of this By-law, development must comply with the following provisions:

        - Despite the minimum front setback listed in Table 11.4.1., the minimum

front setback is 3.0 metres; and

        - Despite the minimum lot frontage listed in Table 11.4.1., the minimum

lot frontage for a corner lot is:

          - 10.3 metres for a house;

          - 8.8 metres per lot for a semi-detached house; and

          - 8.6 metres per lot for a townhouse.

(By-Law Number 2022-62; 2024-332)

### Urban Residential Zone 4 (UR4)

    - Despite the provisions of Section 3, the following definitions apply to lots in the UR4 Zone:

      - Finished Grade means the average elevation of the undisturbed ground, measured at the two points where the required front setback meets the side lot lines.

      - Height, when used with reference to a building, means the vertical distance between the finished grade and the highest point of the building, exclusive of any accessory or auxiliary roof construction, such as an antenna, a chimney, a ventilation duct, an elevator or service penthouse, or a steeple.

      - Lot Coverage means the percentage of the lot area covered by buildings, excluding the following:

        - unenclosed steps and porches;

#### patios;

        - decks; and

        - balconies, bay windows, canopies and overhanging eaves which are 2.0 metres or more in height above the finished grade.

    - The use of any lot or building in the UR4 Zone must comply with the provisions of Table 11.5.1.

#### Table 11.5.1. – UR4 Provisions

| Zoning Provision | house, semi-detached house and townhouse | non-residential building |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | - house: 557.4 - semi-detached house: 350.0 per lot - townhouse: 270.0 per lot | 557.4 |
| 2. Minimum lot frontage (metres) | - house: 16.7 - semi-detached house: 10.0 per lot - townhouse: 8.0 per lot | 16.7 |
| 3. Maximum height (metres) | - flat roof: 9.0 - all other roof types: 10.7 | - flat roof: 9.0 - all other roof types: 10.7 |

| Zoning Provision | house, semi-detached house and townhouse | non-residential building |
| --- | --- | --- |
| 4. Minimum front setback (metres) | 6.1 | 6.1 |
| 5. Minimum rear setback (metres) | 7.6 | equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | 6.1 | 6.1 |
| 7. Minimum interior setback (metres) | - where there is an attached private garage: 1.22 metres - where there is no attached private garage: 2.44 metres on one side and 1.22 metres on the other side (c) Despite (a) and (b), where a common party wall is located along a lot line: 0 metres from the lot line with the common party wall and 1.22 metres from the other interior lot line | 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |
| 8. Minimum aggregate of interior setbacks | — | — |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum lot coverage | 30% | 30% |
| 11. Maximum number of principal buildings per lot | 1.0 | 1.0 |
| 12. Maximum building depth | — | — |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

### Urban Residential Zone 5 (UR5)

    - The use of any lot or building in the UR5 Zone must comply with the provisions of Table 11.6.1.

#### Table 11.6.1. – UR5 Provisions

| Zoning Provision | house, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | - house: 370.0 - semi-detached house: 330.0 per lot - townhouse: 295.0 per lot | 370.0 |
| 2. Minimum lot frontage (metres) | - house: 10.0 - semi-detached house: 9.0 per lot - townhouse: 8.0 per lot | 10.0 |
| 3. Maximum height | (a) flat roof: lesser of 9.0 metres or 3 storeys (b) all other: lesser of 10.7 metres or 3 storeys | (a) flat roof: lesser of 9.0 metres or 3 storeys (b) all other: lesser of 10.7 metres or 3 storeys |
| 4. Minimum front setback (metres) | - Where 2 adjacent buildings have a front lot line on the same street: the lesser of 4.5 metres or the average front setbacks of adjacent buildings - Where 1 adjacent building has a front lot line on the same street: the lesser of 4.5 metres or the average of 4.5 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 4.5 metres - Despite (a), (b) and (c), where a building existed as of the date of passing of this By- law and the front setback is less than required, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback | - 4.5 - Despite (a), where a building existed as of the date of passing of this By-law and the front setback is less than 4.5 metres, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback |

| Zoning Provision | house, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
| 5. Minimum rear setback (metres) | — | equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | - Where 1 adjacent building has a front lot line on the same street: the lesser of 4.5 metres or the average of 4.5 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 4.5 metres - Despite (a) and (b), where a building existed as of the date of passing of this By- law and the exterior setback is less than required, the minimum exterior setback for the existing building and any further development that enlarges or alters the building is the existing exterior setback | - 4.5 - Despite (a), where a building existed as of the date of passing of this By-law and the exterior setback is less than 4.5 metres, the minimum exterior setback for the existing building and any further development that enlarges or alters the building is the existing exterior setback |
| 7. Minimum interior setback (metres) | - house: 0.6 - semi-detached house and townhouse: where a common party wall is located along a lot line: 0 metres from the lot line with the common party wall and 1.2 metres from the other interior lot line | 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |
| 8. Minimum aggregate of interior setbacks | - house: 3.2 - semi-detached house and townhouse: — | — |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum lot coverage | — | — |
| 11. Maximum number of principal buildings per lot | 1.0 | — |

| Zoning Provision | house, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
| 12. Maximum building depth (metres) | - 18.0 - Despite (a), the rear wall of the principal building must not be closer than 7.5 metres to the rear lot line | — |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

### Urban Residential Zone 6 (UR6)

    - The use of any lot or building in the UR6 Zone must comply with the provisions of Table 11.7.1.

#### Table 11.7.1. – UR6 Provisions

| Zoning Provision | House, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | - house: 465.0 - semi-detached house: 280.0 per lot (c) townhouse: 250.0 per lot | 465.0 |
| 2. Minimum lot frontage (metres) | - house: 15.0 - semi-detached house: 9.0 per lot - townhouse: 8.0 per lot | 15.0 |
| 3. Maximum height | - flat roof: lesser of 9.0 metres or 3 storeys - all other: lesser of 10.7 metres or 3 storeys | - flat roof: lesser of 9.0 metres or 3 storeys - all other: lesser of 10.7 metres or 3 storeys |
| 4. Minimum front setback (metres) | - Where 2 adjacent buildings have a front lot line on the same street: the lesser of 4.5 metres or the average front setbacks of adjacent buildings - Where 1 adjacent building has a front lot line on the same street: the lesser of 4.5 metres or the average of 4.5 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 4.5 metres - Despite (a), (b) and (c), where a building existed as of the date of passing of this By- | - 4.5 - Despite (a), where a building existed as of the date of passing of this By-law and the front setback is less than 4.5 metres, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback |

| Zoning Provision | House, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
|  | law and the front setback is less than required, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback |  |
| 5. Minimum rear setback (metres) | — | equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | - Where 1 adjacent building has a front lot line on the same street: the lesser of 4.5 metres or the average of 4.5 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 4.5 metres - Despite (a) and (b), where a building existed as of the date of passing of this By-law and the exterior setback is less than required, the minimum exterior setback for the existing building and any further development that enlarges or alters the building is the existing exterior setback | a) 4.5 (b) Despite (a), where a building existed as of the date of passing of this By-law and the exterior setback is less than 4.5 metres, the minimum exterior setback for the existing building and any further development that enlarges or alters the building is the existing exterior setback |
| 7. Minimum interior setback (metres) | - house: 0.6 - semi-detached house and townhouse: where a common party wall is located along a lot line: 0 metres from the lot line with the common party | 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |

| Zoning Provision | House, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
|  | wall and 1.2 metres from the other interior lot line |  |
| 8. Minimum aggregate of interior setbacks | - house: 3.0 - semi-detached house and townhouse: — | — |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum lot coverage | — | — |
| 11. Maximum number of principal buildings per lot | 1.0 | — |
| 12. Maximum building depth (metres) | - 18.0 - Despite (a), the rear wall of the principal building must not be closer than 7.5 metres to the rear lot line | — |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

### Urban Residential Zone 7 (UR7)

    - The use of any lot or building in the UR7 Zone must comply with the provisions of Table 11.8.1.

#### Table 11.8.1. – UR7 Provisions

| Zoning Provision | house, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | - house: 555.0 - semi-detached house: 330.0 per lot - townhouse: 295.0 per lot | 555.0 |

| Zoning Provision | house, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
| 2. Minimum lot frontage (metres) | - house: 15.0 - semi-detached house: 9.0 per lot - townhouse: 8.0 per lot | - corner lot: 16.5 - all other lots: 15.0 |
| 3. Maximum height | (a) flat roof: lesser of 9.0 metres or 3 storeys (b) all other: lesser of 10.7 metres or 3 storeys | (a) flat roof: lesser of 9.0 metres or 3 storeys (b) all other: lesser of 10.7 metres or 3 storeys |
| 4. Minimum front setback (metres) | - 7.5 - Despite (a), where a building existed as of the date of passing of this By-law and the front setback is less than 7.5 metres, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback | - 7.5 - Despite (a), where a building existed as of the date of passing of this By-law and the front setback is less than 7.5 metres, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback |
| 5. Minimum rear setback (metres) | — | equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | - 7.5 - Despite (a), where a building existed as of the date of passing of this By-law and the exterior setback is less than 7.5 metres, the minimum exterior setback for the existing building and any further development that enlarges or alters the building is the existing exterior setback | - 7.5 - Despite (a), where a building existed as of the date of passing of this By-law and the exterior setback is less than 7.5 metres, the minimum exterior setback for the existing building and any further development that enlarges or alters the building is the |

| Zoning Provision | house, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
|  |  | existing exterior setback |
| 7. Minimum interior setback (metres) | - house: 1.2 - semi-detached house and townhouse: where a common party wall is located along a lot line: 0 metres from the lot line with the common party wall and 1.2 metres from the other interior lot line | 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |
| 8. Minimum aggregate of interior setbacks | - house: 3.0 - semi-detached house and townhouse: — | — |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum lot coverage | — | — |
| 11. Maximum number of principal buildings per lot | 1.0 | — |
| 12. Maximum building depth (metres) | - 18.0 - Despite (a), the rear wall of the principal building must not be closer than 7.5 metres to the rear lot line | — |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

### Urban Residential Zone 8 (UR8)

    - The use of any lot or building in the UR8 Zone must comply with the provisions of Table 11.9.1.

#### Table 11.9.1. – UR8 Provisions

| Zoning Provision | House, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | - house: 418.0 - semi-detached house: 275.0 per lot - townhouse: 245.0 per lot | 418.0 |
| 2. Minimum lot frontage (metres) | - house: 13.7 - semi-detached house: 9.0 per lot - townhouse: 8.0 per lot | - corner lot: 16.5 - all other lots: 13.7 |
| 3. Maximum height | (a) flat roof: lesser of 9.0 metres or 3 storeys (b) all other: lesser of 10.7 metres or 3 storeys | (a) flat roof: lesser of 9.0 metres or 3 storeys (b) all other: lesser of 10.7 metres or 3 storeys |
| 4. Minimum front setback (metres) | - 6.0 - Despite (a), where a building existed as of the date of passing of this By-law and the front setback is less than 6.0 metres, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback | - 6.0 - Despite (a), where a building existed as of the date of passing of this By-law and the front setback is less than 6.0 metres, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback |
| 5. Minimum rear setback (metres) | — | equal to the height of the rear wall |

| Zoning Provision | House, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
| 6. Minimum exterior setback (metres) | - 6.0 - Despite (a), where a building existed as of the date of passing of this By-law and the exterior setback is less than 6.0 metres, the minimum exterior setback for the existing building and any further development that enlarges or alters the building is the existing exterior setback | - 6.0 - Despite (a), where a building existed as of the date of passing of this By-law and the exterior setback is less than 6.0 metres, the minimum exterior setback for the existing building and any further development that enlarges or alters the building is the existing exterior setback |
| 7. Minimum interior setback (metres) | - house: 1.2 - semi-detached house and townhouse: where a common party wall is located along a lot line: 0 metres from the lot line with the common party wall and 1.2 metres from the other interior lot line | 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |
| 8. Minimum aggregate of interior setbacks | - house: 3.0 - semi-detached house and townhouse: — | — |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum lot coverage | — | — |
| 11. Maximum number of principal buildings per lot | 1.0 | — |
| 12. Maximum building depth (metres) | - 18.0 - Despite (a), the rear wall of the principal building must not be closer than 7.5 metres to the rear lot line | — |

| Zoning Provision | House, semi-detached house and townhouse | non-residential uses |
| --- | --- | --- |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

### Urban Residential Zone 9 (UR9)

    - Despite the provisions of Section 3, the following definitions apply to lots in the UR9 Zone:

      - Linked Dwelling means a house that is linked to another house by common underground masonry footing only.

(By-Law Number 2022-62; 2024-332)

    - The use of any lot or building in the UR9 Zone must comply with the provisions of Table 11.10.2.

#### Table 11.10.2. – UR9 Provisions

| Zoning Provision | house, semi-detached house and townhouse | semi-detached house, linked dwelling |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | - house: 320.0 - semi-detached house: 270.0 per lot - townhouse: 240.0 per lot | 320.0 |
| 2. Minimum lot frontage (metres) | - house: 10.6 - semi-detached house: 9.0 per lot - townhouse: 8.0 per lot | - corner lot: 14.0 - other lot: 10.6 |
| 3. Maximum height | - flat roof: lesser of 9.0 metres or 3 storeys - all other: lesser of 10.7 metres or 3 storeys | - flat roof: lesser of 9.0 metres or 3 storeys - all other: lesser of 10.7 metres or 3 storeys |
| 4. Minimum front setback (metres) | 6.0 | 6.0 |
| 5. Minimum rear setback (metres) | 6.0 | equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | 6.0 | 6.0 |
| 7. Minimum interior setback (metres) | - house with attached private garage: 1.2 - house with no attached private garage: 1.2 metres on one side and 2.4 metres on other side | 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |

| Zoning Provision | house, semi-detached house and townhouse | semi-detached house, linked dwelling |
| --- | --- | --- |
|  | (c) semi-detached house and townhouse: where a common party wall is located along a lot line: 0 metres from the lot line with the common party wall and 1.2 metres from the other interior lot line |  |
| 8. Minimum aggregate of interior setbacks | - house: 3.2 - semi-detached house and townhouse: — | — |
| 9. Minimum landscaped open space | 30% | 30% |
| 10. Maximum lot coverage | — | — |
| 11. Maximum number of principal buildings per lot | 1.0 | — |
| 12. Maximum building depth (metres) | — | — |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

#### Additional Provisions for Lots Zoned UR9

    - In addition to the provisions of Table 11.10.2., the use of any lot or building in the UR9 Zone must comply with the following provisions:

      - The minimum separation distance between the main walls above grade of a

linked dwelling is 1.8 metres; and

      - Despite Clause 7.4.8., driveway widths that legally existed on a lot in the UR9 Zone as of the date of passing of this By-law are deemed to be permitted.

# Section 12: Urban Multi-Residential Zones

### All Urban Multi-Residential Zones

    - For the purposes of this By-law, Urban Multi-Residential Zones include Urban Multi- Residential Zone 1 (URM1), Urban Multi-Residential Zone 2 (URM2), Urban Multi- Residential Zone 3 (URM3), Urban Multi-Residential Zone 4 (URM4), Urban Multi- Residential Zone 5 (URM5), Urban Multi-Residential Zone 6 (URM6), Urban Multi- Residential Zone 7 (URM7), Urban Multi-Residential Zone 8 (URM8), Urban Multi- Residential Zone 9 (URM9), Urban Multi-Residential Zone 10 (URM10) and Urban Multi-Residential Zone 11 (URM11).

(By-Law Number 2022-62; 2025-118)

    - Uses permitted in Urban Multi-Residential Zones are limited to the uses identified in Table 12.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 12.1.2., the following corresponding provisions apply:

      - May only contain non-residential uses that are permitted in the CN Zone as per Table 15.1.2., and the non-residential uses are only permitted on the first storey.

      - Is only permitted on a lot that has a front lot line and/or exterior lot line on a Collector Road or Arterial Road in accordance with the street type identified in Schedule 4.

#### Table 12.1.2. - Permitted Uses in the Urban Multi-Residential Zones

| Use | URM1 | URM2 | URM3 | URM4 | URM5 | URM6 | URM7 | URM8 | URM9 | URM10 | URM11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Residential apartment building | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| dwelling unit in a mixed use building | — | — | — | — | — | — | — | ●1 | — | — | ●1 |
| house | ● | — | ● | ● | ● | — | — | — | — | — | — |
| semi-detached house | ● | — | — | — | — | — | — | — | — | — | — |
| stacked townhouse | ● | ● | — | — | — | — | — | ● | ● | ● | ● |
| townhouse | ● | ● | — | ● | ● | — | — | — | — | — | ● |

| Use | URM1 | URM2 | URM3 | URM4 | URM5 | URM6 | URM7 | URM8 | URM9 | URM10 | URM11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non- residential community centre | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| day care centre | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| elementary school | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| library | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| museum | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| place of worship | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |
| secondary school | ●2 | ●2 | ●2 | ●2 | ●2 | ●2 | ●2 | ●2 | ●2 | ●2 | ●2 |

(By-Law Number 2022-62; 2024-332; 2025-118)

### Urban Multi-Residential Zone 1 (URM1)

    - The use of any lot or building in the URM1 Zone must comply with the provisions of Table 12.2.1.

#### Table 12.2.1. – URM1 Provisions

| Zoning Provision | semi-detached house, townhouse | House | apartment building, stacked townhouse | all other permitted uses |
| --- | --- | --- | --- | --- |
| 1. Minimum lot area (square metres) | 180.0 per dwelling unit | 300.0 | 540.0 | 360.0 |
| 2. Minimum lot frontage (metres) | 7.5 | 10.0 | 18.0 | 12.0 |
| 3. Maximum height (metres) | 10.7 | 10.7 | 10.7 | 10.7 |
| 4. Minimum front setback (metres) | The lesser of: - 6.0 - average of the existing front setbacks of the adjacent buildings, to a minimum of 3.0 metres | The lesser of: - 6.0 - average of the existing front setbacks of the adjacent buildings, to a minimum of 3.0 metres | The lesser of: - 6.0 - average of the existing front setbacks of the adjacent buildings, to a minimum of 3.0 metres | The lesser of: - 6.0 - average of the existing front setbacks of the adjacent buildings, to a minimum of 3.0 metres |
| 5. Minimum rear setback (metres) | The greater of: - 7.5 metres - 25% of the lot depth | The greater of: - 7.5 metres - 25% of the lot depth | The greater of: - 7.5 metres - 25% of the lot depth | The greater of: - 7.5 metres - 25% of the lot depth |
| 6. Minimum exterior setback (metres) | 5.0 | 5.0 | 6.0 | 6.0 |
| 7. Minimum interior setback (metres) | - 1.8 metres - where a common party wall is located | 3.6 | - 6.0 metres - where a common party wall is located | 3.6 |

| Zoning Provision | semi-detached house, townhouse | House | apartment building, stacked townhouse | all other permitted uses |
| --- | --- | --- | --- | --- |
|  | along a lot line: 0 metres |  | along a lot line: 0 metres |  |
| 8. Minimum landscaped open space | 30% | 30% | 30% | 30% |
| 9. Maximum lot coverage | 45% | 45% | 45% | 45% |
| 10. Maximum number of principal buildings per lot | 1.0 | 1.0 | 1.0 | 1.0 |
| 11. Minimum aggregate of interior setbacks | — | 3.6 metres, of which one interior setback must be a minimum of 0.6 metres | — | 3.6 metres, of which one interior setback must be a minimum of 0.6 metres |
| 12. Maximum number of principal dwelling units per lot including accessory houses | 4.0 | 4.0 | — | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

### Urban Multi-Residential Zone 2 (URM2)

    - The use of any lot or building in the URM2 Zone must comply with the provisions of Table 12.3.1.

Table 12.3.1. – URM2 Provisions

| Zoning Provision | townhouse | apartment building, stacked townhouse | all other permitted uses |
| --- | --- | --- | --- |
| 1. Minimum lot area (square metres) | 180.0 per dwelling unit | 540.0 | 360.0 |
| 2. Minimum lot frontage (metres) | 7.5 | 18.0 | 12.0 |
| 3. Maximum height (metres) | 11.0 | 12.5 | 10.7 |
| 4. Minimum front setback (metres) | The lesser of: - 6.0 - average of the existing front setbacks of the adjacent buildings, to a minimum of 3.0 metres | The lesser of: - 6.0 - average of the existing front setbacks of the adjacent buildings, to a minimum of 3.0 metres | The lesser of: - 6.0 - average of the existing front setbacks of the adjacent buildings, to a minimum of 3.0 metres |
| 5. Minimum rear setback (metres) | The greater of: - 7.5 metres - 25% of the lot depth | The greater of: - 7.5 metres - 25% of the lot depth | The greater of: - 7.5 metres - 25% of the lot depth |
| 6. Minimum exterior setback (metres) | 5.0 | 6.0 | 6.0 |
| 7. Minimum interior setback (metres) | - 1.8 metres - where a common party wall is located along a lot line: 0 metres | 6.0 | 3.6 |

| Zoning Provision | townhouse | apartment building, stacked townhouse | all other permitted uses |
| --- | --- | --- | --- |
| 8. Minimum landscaped open space | 30% | 30% | 30% |
| 9. Maximum lot coverage | 45% | 45% | 45% |
| 10. Maximum number of principal buildings per lot | 1.0 | 1.0 | 1.0 |
| 11. Minimum aggregate of interior setbacks | — | — | 3.6 metres, of which one interior setback must be a minimum of 0.6 metres |

### Urban Multi-Residential Zone 3 (URM3)

    - The use of any lot or building in the URM3 Zone must comply with the provisions of Table 12.4.1.

#### Table 12.4.1. – URM3 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | 370.0 |
| 2. Minimum lot frontage (metres) | 10.0 |
| 3. Maximum height | - flat roof: lesser of 9.0 metres or 3 storeys - all other: lesser of 10.7 metres or 3 storeys |
| 4. Minimum front setback (metres) | - Where 2 adjacent buildings have a front lot line on the same street: the lesser of 4.5 metres or the average front setbacks of adjacent buildings - Where 1 adjacent building has a front lot line on the same street: the lesser of 4.5 metres or the average of 4.5 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 4.5 metres - Despite (a), (b) and (c), where a building existed as of the date of passing of this By-law and the front setback is less than required, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback |
| 5. Minimum rear setback (metres) | - residential buildings: — - non-residential buildings: equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | - Where 1 adjacent building has a front lot line on the same street: the lesser of 4.5 metres or the average of 4.5 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 4.5 metres - Despite (a) and (b), where a building existed as of the date of passing of this By-law and the exterior setback is less than required, the minimum exterior setback for the existing |

| Zoning Provision | all permitted uses |
| --- | --- |
|  | building and any further development that enlarges or alters the building is the existing exterior setback |
| 7. Minimum interior setback (metres) | - residential buildings: 0.6 - non-residential buildings: 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |
| 8. Minimum aggregate of interior setbacks | - residential buildings: 3.6 metres - non-residential buildings: — |
| 9. Minimum landscaped open space | 30% |
| 10. Maximum number of principal buildings per lot | - residential buildings: 1.0 - non-residential buildings: — |
| 11. Maximum building depth (metres) | - residential buildings: 18.0 - non-residential buildings: — - Despite (a), the rear wall of the principal building must not be closer than 7.5 metres to the rear lot line |
| 12. Maximum number of principal dwelling units per lot including accessory houses | - house, semi-detached house: 4.0 - all other: 6.0 |
| 13. Maximum density | 69 dwelling units per net hectare |

(By-Law Number 2022-62; 2024-332; 2024-333)

### Urban Multi-Residential Zone 4 (URM4)

    - The use of any lot or building in the URM4 Zone must comply with the provisions of Table 12.5.1.

#### Table 12.5.1. – URM4 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | - house: 335.0 - semi-detached house, townhouse: 270.0 per lot |
| 2. Minimum lot frontage (metres) | 18.0 |
| 3. Maximum height | — |
| 4. Minimum front setback (metres) | 7.5 |
| 5. Minimum rear setback (metres) | 7.5 metres for buildings up to 5 storeys, then additional 1.2 metres for every storey above 5 |
| 6. Minimum exterior setback (metres) | 7.5 |
| 7. Minimum interior setback (metres) | - non-residential buildings: equal to half the height of the building - residential buildings that are 1 storey: 1.8 - residential buildings that are 2 storeys: 3.0 - residential buildings that are greater than 2 storeys: 3.0 metres plus 1.2 metres for each additional storey above 2 - Despite (b), (c) and (d), for townhouses, where a common party wall is located along a lot line, then 0 metres |
| 8. Minimum landscaped open space | 30% |
| 9. Maximum density | apartment building: 69 dwelling units per net hectare |
| 10. Maximum number of dwelling units per lot including accessory houses | - house, semi-detached house, townhouse: 4.0 - all other permitted uses: see density requirement above |

(By-Law Number 2022-62; 2024-332; 2024-333)

### Urban Multi-Residential Zone 5 (URM5)

    - The use of any lot or building in the URM5 Zone must comply with the provisions of Table 12.6.1.

#### Table 12.6.1. – URM5 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Maximum height | - flat roof: lesser of 9.0 metres or 3 storeys - all other: lesser of 10.7 metres or 3 storeys |
| 2. Minimum front setback (metres) | 7.5 |
| 3. Minimum rear setback (metres) | - residential buildings: Where parking spaces are provided in the rear yard: 12.0 - residential buildings: Where parking spaces are not provided in the rear yard: 7.5 - residential buildings: Despite (b), where the rear yard is adjacent to the interior yard or a park on an adjacent lot: 6.0 - non-residential buildings: equal to the height of the rear wall |
| 4. Minimum exterior setback (metres) | 7.5 |
| 5. Minimum interior setback (metres) | - residential buildings: 3.0 - non-residential buildings: 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres (c) Despite (a), for townhouses, where a common party wall is located along a lot line, then 0 metres |
| 6. Minimum landscaped open space | 30% |
| 7. Maximum number of principal dwelling units per building | 12.0 |
| 8. Maximum density | 69 dwelling units per net hectare |
| 9. Maximum floor space index | 3.5 |

| Zoning Provision | all permitted uses |
| --- | --- |
| 10. Maximum number of principal dwelling units per lot including accessory houses | - house, semi-detached house, townhouse: 4.0 - all other permitted uses: see density requirement above |

(By-Law Number 2022-62; 2024-332; 2024-333)

#### Additional Provisions for Lots Zoned URM5

    - In addition to the provisions of Table 12.6.1., the use of any lot or building in the URM5 Zone must comply with the following provisions:

The minimum separation distance between residential buildings on the same lot is 4.5 metres; and

2.  The minimum separation distance between the rear wall of a building on a lot in a URM5 Zone and the rear wall of a residential building located on a different lot is 15.0 metres.

### Urban Multi-Residential Zone 6 (URM6)

    - The use of any lot or building in the URM6 Zone must comply with the provisions of Table 12.7.1.

#### Table 12.7.1. – URM6 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum front setback (metres) | 7.5 |
| 2. Minimum rear setback (metres) | equal to the height of the building |
| 3. Minimum exterior setback (metres) | 7.5 |
| 4. Minimum interior setback (metres) | - where adjacent to a house or semi- detached house: equal to the height of the building - all other: equal to 50% of the height of the building |
| 5. Minimum aggregate of interior setbacks | equal to 150% of the height of the building |
| 6. Minimum landscaped open space | 30% |
| 7. Maximum density | 123 dwelling units per net hectare |
| 8. Maximum floor space index | 1.0 |

(By-Law Number 2022-62; 2024-332)

#### Additional Provisions for Lots Zoned URM6

    - In addition to the provisions of Table 12.7.1., the use of any lot or building in the URM6 Zone must comply with the following provisions:

Where an interior lot line is adjacent to a lot with a house or semi-detached house a privacy fence with a minimum height of 1.8 metres must be provided. Such privacy fence must extend from the intersection of the interior lot line with the rear lot line to the intersection of the interior lot line with the required front setback.

(By-Law Number 2022-62; 2024-332)

### Urban Multi-Residential Zone 7 (URM7)

    - The use of any lot or building in the URM7 Zone must comply with the provisions of Table 12.8.1.

Table 12.8.1. – URM7 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Maximum height | lesser of 13.5 metres or 4 storeys |
| 2. Minimum front setback (metres) | - Where 2 adjacent buildings have a front lot line on the same street: the greater of 2.0 metres or the average front setbacks of adjacent buildings - Where 1 adjacent building has a front lot line on the same street: the greater of 2.0 metres or the average of 1.0 metre and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 3.5 metres |
| 3. Minimum rear setback (metres) | 7.5 |
| 4. Minimum exterior setback (metres) | 3.5 |
| 5. Minimum interior setback (metres) | 3.0 |
| 6. Minimum landscaped open space | 30% |
| 7. Maximum density | 123 dwelling units per net hectare |
| 8. Maximum floor space index | 1.0 |

### Urban Multi-Residential Zone 8 (URM8)

    - The use of any lot or building in the URM8 Zone must comply with the provisions of Table 12.9.1.

#### Table 12.9.1. – URM8 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | apartment building, mixed use building, stacked townhouse: 1,480.0 |
| 2. Maximum height (metres) | Lesser of 20.0 metres or 6 storeys, excluding a basement storey |
| 3. Minimum streetwall height (metres) | 12.0 |
| 4. Minimum front setback (metres) | 2.0 |
| 5. Minimum rear setback (metres) | 10.0 |
| 6. Minimum exterior setback (metres) | 3.0 |
| 7. Minimum interior setback (metres) | 3.0 |
| 8. Minimum stepbacks (metres) | Where a main wall faces Johnson Street or Brock Street, on storeys 5 and 6: minimum 2.0 metres from the exterior wall of the 4th storey |
| 9. Minimum landscaped open space | 10% |
| 10. Maximum lot coverage | 55% |
| 11. Maximum floor space index | 3.2 |

Additional Provisions for Lots Zoned URM8

    - In addition to the provisions of Table 12.9.1., the use of any lot or building in the URM8 Zone must comply with the following provisions:

      - Within the required rear setback, a minimum 2.0 metre wide planting strip must be provided along the full length of the rear lot line and must comply with Subclause 4.16.1.2.;

      - Balconies are only permitted above the second storey, to a maximum projection of 1.5 metres from the main wall;

      - Parking spaces are prohibited in the front yard or exterior yard;

      - Despite Clause 1.8.5., the minimum lot area required by Table 12.9.1. must be provided for a lot to be developed with an apartment building, a mixed use building, or stacked townhouses; and

      - Uses that legally existed prior to the passage of this By-law are deemed to be permitted uses and must comply with the following provisions:

        - For lots in the URM8 Zone located west of Albert Street, development of such uses must comply with the provisions of the UR5 Zone; and

        - For lots in the URM8 Zone located east of Albert Street, development for such uses must comply with the provisions of the URM3 Zone.

### Urban Multi-Residential Zone 9 (URM9)

    - Despite the definitions of front lot line and rear lot line, in the URM9 Zone:

      - Where a lot has a street line adjacent to Portsmouth Avenue and/or Woodstone Crescent, such street lines are deemed to be a front lot line; and

      - Where a lot line is adjacent to a street reserve, that lot line is deemed to be a

#### front lot line.

    - The use of any lot or building in the URM9 Zone must comply with the provisions of Table 12.10.1.

#### Table 12.10.1. – URM9 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | apartment building, stacked townhouse: 1,600.0 |
| 2. Maximum height (metres) | Lesser of 12.0 metres or 4 storeys, excluding a basement storey |
| 3. Minimum front setback (metres) | 3.0 |
| 4. Minimum rear setback (metres) | 10.0 |
| 5. Minimum exterior setback (metres) | 3.0 |
| 6. Minimum interior setback (metres) | 3.0 |
| 7. Minimum landscaped open space | 12% |
| 8. Maximum lot coverage | 55% |
| 9. Maximum floor space index | 2.2 |

Additional Provisions for Lots Zoned URM9

    - In addition to the provisions of Table 12.10.1., the use of any lot or building in the URM9 Zone must comply with the following provisions:

      - Balconies are only permitted above the second storey, to a maximum projection of 1.5 metres from the main wall;

      - Parking spaces are prohibited in the front yard or exterior yard;

      - A maximum of 1 driveway is permitted per lot;

      - Despite Clause 1.8.5., the minimum lot area required by Table 12.10.1. must be provided for a lot to be developed with an apartment building or stacked townhouses; and

      - Uses that legally existed prior to the passage of this By-law are deemed to be permitted uses and development of such uses must comply with the provisions of the UR8 Zone.

### Urban Multi-Residential Zone 10 (URM10)

    - The use of any lot or building in the URM10 Zone must comply with the provisions of Table 12.11.1.

#### Table 12.11.1. – URM10 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | apartment building, stacked townhouse: 950.0 |
| 2. Maximum height (metres) | Lesser of 12.0 metres or 4 storeys, excluding a basement storey |
| 3. Minimum front setback (metres) | 3.0 |
| 4. Minimum rear setback (metres) | 10.0 |
| 5. Minimum exterior setback (metres) | 3.0 |
| 6. Minimum interior setback (metres) | 3.0 |
| 7. Minimum landscaped open space | 10% |
| 8. Maximum lot coverage | 55% |
| 9. Minimum lot depth (metres) | 32.0 |
| 10. Maximum floor space index | 2.2 |

Additional Provisions for Lots Zoned URM10

    - In addition to the provisions of Table 12.11.1., the use of any lot or building in the URM10 Zone must comply with the following provisions:

      - Within the required rear setback, a minimum 2.0 metre wide planting strip must be provided along the full length of the rear lot line and must comply with Subclause 4.16.1.2.;

      - Balconies are only permitted above the second storey, to a maximum projection of 1.5 metres from the main wall;

      - Parking spaces are prohibited in the front yard or exterior yard;

      - Despite Clause 1.8.5., the minimum lot area required by Table 12.11.1. must be provided for a lot to be developed with an apartment building or stacked townhouses; and

      - Uses that legally existed prior to the passage of this By-law are deemed to be permitted uses and development of such uses must comply with the provisions of the UR10 Zone.

### Urban Multi-Residential Zone 11 (URM11)

    - The use of any lot or building in the URM11 Zone must comply with the provisions of Table 12.12.1.

#### Table 12.12.1. – URM11 Provisions

| Zoning Provision | All permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height | The lesser of: - 4 storeys - 13.5 metres |
| 4. Minimum front setback (metres) | 4.5 |
| 5. Minimum rear setback (metres) | 7.5 |
| 6. Minimum exterior setback (metres) | 4.5 |
| 7. Minimum interior setback (metres) | 1.2 metres for the first three storeys, plus an additional 1.2 metres for the fourth storey |
| 8. Minimum landscaped open space | 30% |
| 9. Maximum lot coverage | 45% |
| 10. Maximum residential density | 90 dwelling units per net hectare |

(By-Law Number 2022-62; 2025-118)

#### Additional Provisions for Lots Zoned URM11

    - In addition to the provisions of Table 12.12.1., uses in the URM11 Zone must comply with the following provisions:

      - Loading spaces, parking spaces, except for accessible spaces, and

parking lots are not permitted in a front yard or exterior yard.

      - Residential uses that legally existed prior to the passage of this By-law are deemed to be permitted uses and must comply with the provisions of the UR5 Zone.”

(By-Law Number 2022-62; 2025-118)

# Section 13: Heritage Zones

### All Heritage Zones

    - For the purposes of this By-law, Heritage Zones include Heritage Zone 1 - Village of Barriefield (HCD1), Heritage Zone 2 - Market Square (HCD2), and Heritage Zone 3 – Old Sydenham (HCD3).

    - Uses permitted in Heritage Zones are limited to the uses identified in Table 13.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 13.1.2., the following corresponding provisions apply:

      - Is permitted only as an accessory use to a principal use on the lot and must be located at or above the second storey.

      - No portion of any lot within 30 metres of the street line of Highway 15 may be used for any purpose other than passive recreation.

#### Table 13.1.2. - Permitted Uses in the Heritage Zones

| Use | HCD1 | HCD2 | HCD3 |
| --- | --- | --- | --- |
| Residential apartment building | — | — | ● |
| dwelling unit in a mixed use building | — | ●1 | — |
| semi-detached house | ●2 | — | ● |
| house | ●2 | — | ● |
| stacked townhouse | — | — | — |
| townhouse | ●2 | — | ● |
| Non-residential animal care | — | ● | — |
| banquet hall | — | ● | — |
| building supply store | — | ● | — |
| commercial parking lot | — | ● | — |
| community centre | ●2 | ● | ● |

| Use | HCD1 | HCD2 | HCD3 |
| --- | --- | --- | --- |
| club | — | ● | — |
| creativity centre | — | ● | — |
| day care centre | — | ● | — |
| department store | — | ● | — |
| elementary school | ●2 | ● | ● |
| entertainment establishment | — | ● | — |
| financial institution | — | ● | — |
| fitness centre | — | ● | — |
| grocery store | — | ● | — |
| hotel | — | ● | — |
| laundry store | — | ● | — |
| library | ●2 | ● | ● |
| museum | ●2 | ● | ● |
| office | — | ● | — |
| personal service shop | — | ● | — |
| place of worship | ●2 | ● | ● |
| public market | — | ● | — |
| recreation facility | — | ● | — |
| repair shop | — | ● | — |
| restaurant | — | ● | — |
| retail store | — | ● | — |
| training facility | — | ● | — |
| wellness clinic | — | ● | — |

By-Law Number 2022-62; 2024-332)

### Heritage Zone 1 – Village of Barriefield (HCD1)

    - The following definitions apply to lots in the HCD1 Zone:

      - Heritage Building means a building that existed as of the date of passing of this By-law on a heritage lot;

      - Heritage Lot means the following lots, as they existed as of the date of passing of this By-law:

        - 6-8, 10, 13, 14 and 16 Drummond Street;

        - 7 and 9 George Street;

        - 218 Green Bay Road;

        - 228, 230, 232, 234, 236, 238, 244, 246 and 248 James Street;

        - 202, 207, 210, 215, 217-219, 223-225, 226-228, 233, 239, 247, 249, 268

and 275 Main Street;

        - 404, 406, 407, 412, 413, 414, 415, 419, 421 and 423 Regent Street;

        - 2 Sharman’s Lane; and

        - 404, 406 and 412 Wellington Street; and

      - Non-Heritage Lot means all lots in the HCD1 Zone, excluding heritage lots.

    - The use of any lot or building in the HCD1 Zone must comply with the provisions of Table 13.2.2.

#### Table 13.2.2. – HCD1 Provisions

| Zoning Provision | Heritage Lot | Non-Heritage Lot |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | Lot area existing as of the date of passing of this By-law | 370.0 |
| 2. Minimum lot frontage (metres) | Lot frontage existing as of the date of passing of this By-law | 12.0 |
| 3. Maximum height (metres) | Height existing as of the date of passing of this By-law | (a) Where 2 adjacent lots with a front lot line on the same street are heritage lots: the lesser of 10.0 metres or the average of the height |

| Zoning Provision | Heritage Lot | Non-Heritage Lot |
| --- | --- | --- |
|  |  | of the heritage buildings on the adjacent heritage lots (b) For all other lots: 10.0 |
| 4. Minimum front setback (metres) | Front setback existing as of the date of passing of this By-law | - Where 2 adjacent buildings have a front lot line on the same street: the lesser of 3.0 metres or the average front setbacks of adjacent buildings - Where 1 adjacent building has a front lot line on the same street: the lesser of 3.0 metres or the average of 3.0 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 3.0 metres - Despite (a), (b) and (c), where a building existed as of the date of passing of this By-law and the front setback is less than required, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback |
| 5. Minimum rear setback (metres) | Rear setback existing as of the date of passing of this By-law | 7.0 |
| 6. Minimum exterior setback (metres) | Exterior setback existing as of the date of passing of this By-law | 3.0 |
| 7. Minimum interior setback (metres) | Interior setback existing as of the date of passing of this By-law | - 1.2 metres - Despite (a), where a common party wall is located along a lot line: 0 metres |
| 8. Minimum aggregate of interior setbacks | Aggregate of interior setbacks that existed as | 6.0 |

| Zoning Provision | Heritage Lot | Non-Heritage Lot |
| --- | --- | --- |
|  | of the date of passing of this By-law |  |
| 9. Minimum landscaped open space | Landscaped open space existing as of the date of passing of this By-law | 30% |
| 10. Maximum lot coverage | Lot coverage existing as of the date of passing of this By-law | 25% |
| 11. Maximum number of principal buildings per lot | 1 | 1 |
| 12. Maximum number of storeys | Number of storeys existing as of the date of passing of this By-law | 2 |
| 13. Maximum number of dwelling units per lot including accessory houses | 4.0 | 4.0 |

(By-Law Number 2022-62; 2024-332; 2024-333)

#### Additional Provisions for Lots Zoned HCD1

    - In addition to the provisions of Table 13.2.2., the use of any lot or building in the HCD1 Zone must comply with the following provisions:

      - The maximum height of any fence or wall in the front yard is 1.0 metre;

      - Any portion of a lot within 30.0 metres of the street line of Highway 15 must be maintained as landscaped open space;

      - Buildings must be setback a minimum of 30.0 metres from the street line of Highway 15;

      - Parking is not permitted in the front yard of any lot; and

      - The maximum gross floor area of a non-residential building is 275.0 square metres.

    - An addition to a heritage building or a new building on a heritage lot must comply with the provisions that apply to a non-heritage lot, except the maximum height of an addition must not exceed a height that is 0.5 metres less than the height of the heritage building.

(By-Law Number 2022-62; 2024-332)

### Heritage Zone 2 – Market Square (HCD2)

    - The use of any lot or building in the HCD2 Zone must comply with the provisions of Table 13.3.1.

#### Table 13.3.1. – HCD2 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Minimum height | See subclause 13.3.2.1. |
| 4. Maximum height (metres) | See subclause 13.3.2.1. |
| 5. Minimum front setback (metres) | Average of the existing front setback of immediately adjacent buildings fronting on the same street |
| 6. Minimum rear setback (metres) | 0.0 |
| 7. Minimum exterior setback (metres) | 0.0 |
| 8. Minimum interior setback (metres) | 0.0 |
| 9. Minimum landscaped open space | 0.0 |
| 10. Minimum lot coverage | 50% |
| 11. Maximum lot coverage | 100% |
| 12. Maximum number of dwelling units per lot | 123 dwelling units per net hectare |

Additional Provisions for Lots Zoned HCD2

    - In addition to the provisions of Table 13.3.1., uses in the HCD2 Zone must comply with the following provisions:

      - Height provisions:

        - The height of buildings that existed as of the date of passing of this By- law must not be increased;

        - The maximum height of any new building replacing a building that existed as of the date of passing of this By-law is equal to the height of the building that existed as of the date of passing of this By-law;

        - The maximum height of any new building on a lot that was vacant as of the date of passing of this By-law is equal to the height of the tallest building located within the same block and zoned HCD2;

        - For buildings located on corner lots, the top of the highest projection along the main wall must not exceed 0.5 metres above the height of the tallest building or the height of the tallest parapet on a building located in the same block and zoned HCD2;

        - For buildings located on interior lots, the top of the highest projection along the main wall must not exceed 0.5 metres above the higher parapet line of the two adjacent buildings;

        - For buildings with a flat roof, the roof line must be lower than the parapet;

        - The tower on the property municipally known as 200 Ontario Street is not defined as a parapet or a roof; and

        - The minimum height of any new building replacing a building that existed as of the date of passing of this By-law is equal to the height of the building that existed as of the date of passing of this By-law.

      - Despite the provisions of Section 7:

        - No loading spaces are required; and

        - Parking spaces are not permitted in the front yard.

      - Any building erected on or before September 10, 1996 and located in the HCD2 Zone may be converted to contain one or more dwelling units, subject to the provisions of the HCD2 Zone, provided that:

        - All dwelling units must be located at or above the second storey;

        - Expansion or enlargement of the external walls or roof of the existing

building is not permitted;

        - Provisions regulating minimum setbacks, amenity area and maximum

lot coverage do not apply;

        - Drive-throughs are prohibited; and

        - The maximum gross floor area of a grocery store is 275.0 square metres.

#### Ground Floor Commercial Uses

      - Buildings are required to have ground floor commercial uses on the first storey where any portion of the lot aligns with the area identified as “Required Ground Floor Commercial” on Schedule 3.

      - Where ground floor commercial uses are required by Subclause 4., the entire streetwall of the first storey, excluding areas devoted to a lobby or other shared entrances/exits for other permitted uses, must be occupied by commercial uses. Portions of the floor area of the first storey that do not have an exterior wall facing a street line may be occupied by uses that service the building such as loading spaces, waste management facilities and rooms, mechanical rooms, bike parking facilities and other similar uses.

### Heritage Zone 3 – Old Sydenham (HCD3)

    - The use of any lot or building in the HCD3 Zone must comply with the provisions of Table 13.4.1.

#### Table 13.4.1. – HCD3 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | 370.0 |
| 2. Minimum lot frontage (metres) | 10.0 |
| 3. Maximum height | - flat roof: lesser of 9.0 metres or 3 storeys - all other: lesser of 10.7 metres or 3 storeys |
| 4. Minimum front setback (metres) | - Where 2 adjacent buildings have a front lot line on the same street: the lesser of 3.5 metres or the average front setbacks of adjacent buildings - Where 1 adjacent building has a front lot line on the same street: the lesser of 3.5 metres or the average of 3.5 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 3.5 metres - Despite (a), (b) and (c), where a building existed as of the date of passing of this By-law and the front setback is less than required, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback |
| 5. Minimum rear setback (metres) | - residential buildings: — - non-residential buildings: equal to the height of the rear wall |
| 6. Minimum exterior setback (metres) | - Where 1 adjacent building has a front lot line on the same street: the lesser of 3.5 metres or the average of 3.5 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 3.5 metres - Despite (a) and (b), where a building existed as of the date of passing of this By-law and the exterior setback is less than required, the minimum exterior setback for the existing building |

| Zoning Provision | all permitted uses |
| --- | --- |
|  | and any further development that enlarges or alters the building is the existing exterior setback |
| 7. Minimum interior setback (metres) | - semi-detached house, townhouse: 3.5, except where a common party wall is located along a lot line, then 0 metres - other residential buildings where there are openings in the main wall facing the interior lot line: 1.2 - other residential buildings where there are no openings in the main wall facing the interior lot line: 0.6 - non-residential buildings: 3.0 metres plus 0.3 metres for each additional 0.6 metres in height above 4.6 metres |
| 8. Minimum landscaped open space | 30% |
| 9. Maximum number of principal buildings per lot | - residential buildings: 1.0 - non-residential buildings: — |
| 10. Maximum number of principal dwelling units per lot | - residential buildings: 6.0 - non-residential buildings: — |
| 12. Maximum building depth (metres) | - residential buildings: 18.0 - non-residential buildings: — - Despite (a), the rear wall of the principal building must not be closer than 7.5 metres to the rear lot line |
| 13. Maximum density | 69 dwelling units per net hectare |
| 14. Maximum number of principal dwelling units per lot including accessory houses | - house, semi-detached house, townhouse: 4.0 - all other permitted uses: see density requirement above |

(By-Law Number 2022-62; 2024-332, 2024-333)

#### Additional Provisions for Lots Zoned HCD3

    - In addition to the provisions of Table 13.4.1., uses in the HCD3 Zone must comply with the following provisions:

      - Dormers are permitted, provided that:

        - The front wall of the dormer is setback a minimum of 0.4 metres from the main wall;

        - The side wall of the dormer is setback a minimum of 1.0 metre from the edge of the roof on which it is located; and

        - The maximum cumulative length of all dormers on the same portion of a sloped roof is the lesser of 4.6 metres or 50% of the length of the roof on which it is located.

      - Parking is not permitted in the front yard of any lot.

# Section 14: Institutional Zones

### All Institutional Zones

    - For the purposes of this By-law, Institutional Zones include Institutional Minor Zone (IN1), Institutional Major Zone (IN2), Correctional Facility Zone (G1) and Military Installation Zone (G2).

    - Uses permitted in Institutional Zones are limited to the uses identified in Table 14.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 14.1.2., the following corresponding provisions apply:

      - Is permitted only as an accessory use to a principal use on the lot.

      - Dwelling units and/or co-living units are permitted only where they are associated with a post-secondary institution.

#### Table 14.1.2. - Permitted Uses in the Institutional Zones

| Use | IN1 | IN2 | G1 | G2 |
| --- | --- | --- | --- | --- |
| Residential dwelling units in a mixed use building | ●1 | ●2 | ●1 | ●1 |
| house | — | ●2 | — | — |
| semi-detached house | — | ●2 | — | — |
| townhouse | — | ●2 | — | — |
| apartment building | — | ●2 | — | ●2 |
| Non-residential animal care | ● | — | — | ● |
| banquet hall | ● | ● | ● | ● |
| cemetery | ● | — | — | ● |
| community centre | ● | ● | ● | ● |
| correctional college | — | — | ● | — |
| correctional institution | — | — | ● | — |

| Use | IN1 | IN2 | G1 | G2 |
| --- | --- | --- | --- | --- |
| day care centre | ●1 | ●1 | ●1 | ●1 |
| hospital | — | ● | — | ● |
| institutional use | ● | ● | — | — |
| laboratory | — | ● | — | — |
| library | ● | ● | ● | ● |
| military installation | — | — | — | ● |
| museum | ● | ● | ● | ● |
| place of worship | ● | ● | ● | ● |
| post-secondary institution | — | ● | — | ● |
| recreation facility | ● | ● | — | ● |
| restaurant | — | — | — | ● |
| retail store | — | — | — | ● |
| secondary school | ● | ● | ● | ● |
| special needs facility | ● | ● | ● | ● |
| wellness clinic | ● | — | — | ● |

(By-Law Number 2022-62; 2024-332)

### Institutional Minor Zone (IN1)

    - The use of any lot or building in the IN1 Zone must comply with the provisions of Table 14.2.1.

Table 14.2.1. – IN1 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | 23.0 |
| 4. Minimum front setback (metres) | 6.1 |
| 5. Minimum rear setback (metres) | - Where the rear lot line abuts a lot in a Commercial Zone or Institutional Zone: 7.5 - Where the rear lot line abuts a lot in all other Zones: 10.5 |
| 6. Minimum exterior setback (metres) | 7.6 |
| 7. Minimum interior setback (metres) | - Where the interior lot line abuts a lot in a Commercial Zone or Institutional Zone: 0.0 - Where the interior lot line abuts a lot in all other Zones: 6.1 |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | — |
| 10. Maximum number of dwelling units per lot | 1.0 |

### Institutional Major Zone (IN2)

    - The following definitions apply to lots in the IN2 Zone:

      - Area A means buildings that are adjacent to:

        - Collingwood Street between Queen’s Crescent and Union Street;

        - Union Street between Collingwood Street and Alfred Street;

        - Alfred Street between Union Street and Johnson Street;

        - Johnson Street between Alfred Street and Barrie Street; or

        - Barrie Street between Johnson Street and Clergy Street.

      - Area B means buildings that are adjacent to:

        - Barrie Street between Clergy Street and King Street;

        - King Street between Barrie Street and Collingwood Street; and

        - Collingwood Street between King Street and Queen’s Crescent.

      - Area C means all other buildings in an IN2 Zone.

    - The use of any lot or building in the IN2 Zone must comply with the provisions of Table 14.3.2.

#### Table 14.3.2. – IN2 Provisions

| Zoning Provision | house, semi-detached house, townhouse | all other permitted uses |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | - house: 370.0 - semi-detached house: 330.0 per lot - townhouse: 295.0 per lot | — |
| 2. Minimum lot frontage (metres) | - house: 10.0 - semi-detached house: 9.0 per lot - townhouse: 8.0 per lot | — |
| 3. Maximum height (metres) | - flat roof: lesser of 9.0 metres or 3 storeys - all other: lesser of 10.7 metres or 3 storeys | — |

| Zoning Provision | house, semi-detached house, townhouse | all other permitted uses |
| --- | --- | --- |
| 4. Minimum front setback (metres) | - Where 2 adjacent buildings have a front lot line on the same street: the lesser of 4.5 metres or the average front setbacks of adjacent buildings - Where 1 adjacent building has a front lot line on the same street: the lesser of 4.5 metres or the average of 4.5 metres and the front setback of adjacent building - Where no adjacent buildings have a front lot line on the same street: 4.5 metres - Despite (a), (b) and (c), where a building existed as of the date of passing of this By-law and the front setback is less than required, the minimum front setback for the existing building and any further development that enlarges or alters the building is the existing front setback | - Area A: 1.5 times the height of the building - Area B: 7.5 - Area C: 7.5 |
| 5. Minimum rear setback (metres) | — | - Area A: 1.5 times the height of the building - Area B: 7.5 - Area C: the greater of 7.5 metres or 25% of the lot depth |
| 6. Minimum exterior setback (metres) | (a) Where 1 adjacent building has a front lot line on the same street: the lesser of 4.5 metres or the average of 4.5 metres and the front setback of adjacent building (b) Where no adjacent buildings have a front lot line on the same street: 4.5 metres | - Area A: 1.5 times the height of the building - Area B: 7.5 - Area C: 50% of the height of the building |

| Zoning Provision | house, semi-detached house, townhouse | all other permitted uses |
| --- | --- | --- |
|  | (c) Despite (a) and (b), where a building existed as of the date of passing of this By-law and the exterior setback is less than required, the minimum exterior setback for the existing building and any further development that enlarges or alters the building is the existing exterior setback |  |
| 7. Minimum interior setback (metres) | - house: 0.6 - semi-detached house and townhouse: where a common party wall is located along a lot line: 0 metres from the lot line with the common party wall and 1.2 metres from the other interior lot line | - Area A: 1.5 times the height of the building - Area B: 7.5 - Area C: 50% of the height of the building |
| 8. Minimum aggregate of interior setbacks | - house: 3.6 - semi-detached house and townhouse: — | — |
| 9. Minimum landscaped open space | 30% | — |
| 10. Maximum lot coverage | — | — |
| 11. Maximum number of principal buildings per lot | 1 | — |
| 12. Maximum building depth (metres) | - 18.0 - Despite (a), the rear wall of the principal building must not be closer than 7.5 metres to the rear lot line | — |
| 13. Maximum density of habitation units | — | 519 habitation units per net hectare |
| 14. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

#### Additional Provisions for Lots Zoned IN2

    - In addition to the provisions of Table 14.3.2., uses in the IN2 Zone must comply with the following provisions:

      - Despite Clause 7.1.1., where a residential use is associated with a post- secondary institution on a lot in an IN2 Zone, the provisions of Table 7.1.1. do not apply;

      - Short-term bike spaces for post-secondary institutions are exempt from Subclause 7.3.9.3.; and

      - Where a residential use is associated with a post-secondary institution, the maximum number of principal buildings per lot in Table 14.3.2. do not apply.

### Correctional Facility Zone (G1)

    - The use of any lot or building in the G1 Zone must comply with the provisions of Table 14.4.1.

Table 14.4.1. – G1 Provisions

| Zoning Provision | dwelling unit in a mixed use building | all other permitted uses |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | 2,050.0 | — |
| 2. Minimum lot frontage (metres) | 30.0 | 30.0 |
| 3. Maximum height (metres) | 12.0 | 12.0 |
| 4. Minimum front setback (metres) | 6.1 | 6.1 |
| 5. Minimum rear setback (metres) | 7.5 | 7.5 |
| 6. Minimum exterior setback (metres) | 6.1 | 6.1 |
| 7. Minimum interior setback (metres) | - Where the interior lot line abuts a lot in a Commercial Zone or Institutional Zone: 0.0 - Where the interior lot line abuts a lot in all other Zones: 7.6 | - Where the interior lot line abuts a lot in a Commercial Zone or Institutional Zone: 0.0 - Where the interior lot line abuts a lot in all other Zones: 7.6 |
| 8. Minimum landscaped open space | 30% | 30% |
| 9. Maximum lot coverage | — | — |
| 10. Maximum number of dwelling units per lot | 1.0 | 1.0 |

### Military Installation Zone (G2)

    - The use of any lot or building in the G2 Zone must comply with the provisions of Table 14.5.1.

Table 14.5.1. – G2 Provisions

| Zoning Provision | dwelling unit in a mixed use building | all other permitted uses |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | 2,050.0 | 8,000.0 |
| 2. Minimum lot frontage (metres) | 30.0 | 90.0 |
| 3. Maximum height (metres) | 12.0 | 23.0 |
| 4. Minimum front setback (metres) | 7.5 | 7.5 |
| 5. Minimum rear setback (metres) | - Where the rear lot line abuts a lot in a Commercial Zone or Institutional Zone: 7.5 - Where the rear lot line abuts a lot in all other Zones: 10.5 | - Where the rear lot line abuts a lot in a Commercial Zone or Institutional Zone: 7.5 - Where the rear lot line abuts a lot in all other Zones: 10.5 |
| 6. Minimum exterior setback (metres) | 7.5 | 7.5 |
| 7. Minimum interior setback (metres) | - Where the interior lot line abuts a lot in a Commercial Zone or Institutional Zone: 0.0 - Where the interior lot line abuts a lot in all other Zones: 9.0 | - Where the interior lot line abuts a lot in a Commercial Zone or Institutional Zone: 0.0 - Where the interior lot line abuts a lot in all other Zones: 9.0 |
| 8. Minimum landscaped open space | 30% | 30% |
| 9. Maximum lot coverage | — | — |

# Section 15: Commercial Zones

### All Commercial Zones

    - For the purposes of this By-law, Commercial Zones include Neighbourhood Commercial Zone (CN), Arterial Commercial Zone (CA), District Commercial Zone (CD), Regional Commercial Zone (CR), General Commercial Zone (CG), Marine Commercial Zone (CW) and Harbour Zone (HB).

    - Uses permitted in Commercial Zones are limited to the uses identified in Table 15.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 15.1.2., the following corresponding provisions apply:

      - Is permitted only as an accessory use to a principal use on the lot and must be located on or above the second storey.

      - Is permitted only as an accessory use to a principal use on the lot.

#### Table 15.1.2. - Permitted Uses in the Commercial Zones

| Use | CN | CA | CD | CR | CG | CW | HB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Residential dwelling unit in a mixed use building | ● | — | — | — | ● | ●2 | — |
| house | — | — | — | — | — | ●2 | — |
| Non-residential agricultural sales establishment | — | ● | — | — | — | — | — |
| animal care | — | ● | ● | — | ● | — | — |
| automobile sales establishment | — | ● | — | — | ● | — | — |
| banquet hall | — | ● | ● | ● | ● | — | — |
| building supply store | — | ● | — | ● | — | — | — |
| carwash | — | — | — | — | ● | — | — |
| commercial parking lot | — | — | ● | ● | ● | — | — |
| community centre | ● | ● | ● | ● | ● | ● | — |
| club | — | ● | ● | ● | ● | — | — |

| Use | CN | CA | CD | CR | CG | CW | HB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| creativity centre | ● | ● | ● | ● | ● | — | — |
| day care centre | ● | ● | ● | ● | ● | — | — |
| department store | — | — | ● | ● | — | — | — |
| entertainment establishment | — | ● | ● | ● | — | — | — |
| ferry terminal | — | — | — | — | — | — | ● |
| financial institution | ● | ● | ● | ● | ● | — | — |
| fitness centre | ● | ● | ● | ● | ● | — | — |
| service station | — | ● | ● | ● | ● | — | — |
| grocery store | ● | ● | ● | ● | ● | — | — |
| funeral establishment | — | ● | — | — | ● | — | — |
| garden centre | — | ● | ● | ● | — | — | — |
| hotel | — | ● | — | — | — | ● | — |
| institutional use | — | — | — | — | ● | — | — |
| laboratory | — | — | — | — | — | — | — |
| laundry store | ● | ● | ● | ● | ● | — | — |
| library | ● | ● | ● | ● | ● | ● | — |
| marina | — | — | — | — | — | ● | ● |
| museum | ● | ● | ● | ● | ● | ● | — |
| office | ● | ●1 | ● | ● | ● | — | — |
| outdoor storage | — | — | — | — | — | ●2 | — |
| personal service shop | ● | ● | ● | ● | ● | — | — |
| place of worship | ● | ● | ● | ● | ● | ● | — |
| public market | — | — | — | — | — | — | — |
| recreation facility | — | ● | ● | ● | ● | — | — |
| recreational vehicle sales establishment | — | ● | — | — | ● | — | — |
| repair shop | — | ● | — | — | ● | ● | — |
| restaurant | ● | ● | ● | ● | ● | ● | ● |
| retail store | ● | ● | ● | ● | ● | ● | ● |
| secondary school | — | ● | — | — | — | — | — |

| Use | CN | CA | CD | CR | CG | CW | HB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| special needs facility | — | ● | ● | — | ● | — | — |
| training facility | — | — | ● | ● | ● | — | — |
| transportation depot | — | — | — | — | — | — | — |
| transportation terminal | — | — | — | — | — | — | — |
| wellness clinic | ● | ● | ● | ● | ● | — | — |
| wholesale establishment | — | ● | — | — | — | — | — |

(By-Law Number 2022-62; 2024-332)

### Neighbourhood Commercial Zone (CN)

    - The use of any lot or building in the CN Zone must comply with the provisions of Table 15.2.1.

#### Table 15.2.1. – CN Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | 12.0 |
| 4. Minimum front setback (metres) | — |
| 5. Minimum rear setback (metres) | 6.0 |
| 6. Minimum exterior setback (metres) | 1.8 |
| 7. Minimum interior setback (metres) | 0.0 |
| 8. Minimum landscaped open space | 10% |
| 9. Maximum lot coverage | — |
| 10. Maximum number of dwelling units per lot | 1 |

Additional Provisions for Lots Zoned CN

    - In addition to the provisions of Table 15.2.1., uses in the CN Zone must comply with the following provisions:

      - The maximum permitted gross floor area of a fitness centre, financial institution, laundry store, personal service shop, repair shop, retail store, restaurant or grocery store is 185.0 square metres;

      - The maximum permitted gross floor area of a building containing more than one commercial use is 930.0 square metres;

      - Buildings are required to have ground floor commercial uses on the first storey where any portion of the lot aligns with the area identified as “Required Ground Floor Commercial” on Schedule 3;

      - Despite the maximum number of dwelling units per lot in Table 15.2.1., where a building is required to have ground floor commercial uses, such lot is exempt from the maximum number of dwelling units per lot if it complies with Clause 4.4.1.; and

      - Where ground floor commercial uses are required by Subclause 3., the entire streetwall of the first storey, excluding areas devoted to a lobby or other shared entrances/exits for other permitted uses, must be occupied by commercial uses. Portions of the floor area of the first storey that do not have an exterior wall facing a street line may be occupied by uses that service the building such as loading spaces, waste management facilities and rooms, mechanical rooms, bike parking facilities and other similar uses.

### Arterial Commercial Zone (CA)

    - The use of any lot or building in the CA Zone must comply with the provisions of Table 15.3.1.

#### Table 15.3.1. – CA Provisions

| Zoning Provision | dwelling unit in a mixed use building | all other permitted uses |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | 1393.0 | — |
| 2. Minimum lot frontage (metres) | — | — |
| 3. Maximum height (metres) | 13.7 | 13.7 |
| 4. Minimum front setback (metres) | 3.0 | 3.0 |
| 5. Minimum rear setback (metres) | 7.5 | 7.5 |
| 6. Minimum exterior setback (metres) | 3.1 | 3.1 |
| 7. Minimum interior setback (metres) | 0.01 | 0.01 |
| 8. Minimum landscaped open space | 20% | 20% |
| 9. Maximum lot coverage | — | — |
| 10. Maximum number of dwelling units per lot | 1.0 | — |

Additional Provisions for Lots Zoned CA

    - In addition to the provisions of Table 15.3.1., uses in the CA Zone must comply with the following provisions:

      - The minimum interior setback where an interior lot line abuts a Zone other than a Commercial Zone or Institutional Zone is 6.0 metres; and

      - Where a parking lot is adjacent to a sidewalk or walkway, a minimum 3.0 metre wide planting strip must be provided between such parking lot and sidewalk or walkway.

### District Commercial Zone (CD)

    - The use of any lot or building in the CD Zone must comply with the provisions of Table 15.4.1.

#### Table 15.4.1. – CD Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | 60.0 |
| 3. Maximum height (metres) | The greater of: - 13.7 metres - 1 storey |
| 4. Minimum front setback (metres) | 3.0 |
| 5. Minimum rear setback (metres) | 15.0 |
| 6. Minimum exterior setback (metres) | 3.0 |
| 7. Minimum interior setback (metres) | 0.01 |
| 8. Minimum landscaped open space | 10% |
| 9. Maximum lot coverage | — |
| 10. Maximum number of dwelling units per lot | — |

Additional Provisions for Lots Zoned CD

    - In addition to the provisions of Table 15.4.1., uses in the CD Zone must comply with the following provisions:

      - The minimum interior setback where an interior lot line abuts any Zone other than a Commercial Zone is 15.0 metres; and

      - Outdoor storage is prohibited.

### Regional Commercial Zone (CR)

    - The use of any lot or building in the CR Zone must comply with the provisions of Table 15.5.1.

#### Table 15.5.1. – CR Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (hectares) | 14.0 |
| 2. Minimum lot frontage (metres) | 60.0 |
| 3. Maximum height (metres) | — |
| 4. Minimum front setback (metres) | 3.0 |
| 5. Minimum rear setback (metres) | 15.0 |
| 6. Minimum exterior setback (metres) | 3.0 |
| 7. Minimum interior setback (metres) | 15.0 |
| 8. Minimum landscaped open space | 10%2 |
| 9. Maximum lot coverage | — |
| 10. Maximum number of dwelling units per lot | — |

Additional Provisions for Lots Zoned CR

    - In addition to the provisions of Table 15.5.1., uses in the CR Zone must comply with the following provisions:

      - The minimum gross leasable area is 30,000 square metres;

      - Where the lot is developed in phases, the required landscaped open space must be provided on a pro rata basis with the construction of gross leasable area; and

      - No outdoor storage is permitted, except for the sole purpose of a garden centre.

### General Commercial Zone (CG)

    - The use of any lot or building in the CG Zone must comply with the provisions of Table 15.6.1.

#### Table 15.6.1. – CG Provisions

| Zoning Provision | dwelling unit in a mixed use building | service station | all other permitted uses |
| --- | --- | --- | --- |
| 1. Minimum lot area (square metres) | 1,393.6 | 1,161.2 | 464.5 |
| 2. Minimum lot frontage (metres) | 30.0 | 30.0 | 30.0 |
| 3. Maximum height (metres) | 10.7 | 10.7 | 10.7 |
| 4. Minimum front setback (metres) | 3.1 | 3.1 | 3.1 |
| 5. Minimum rear setback (metres) | 7.5 | 7.5 | 7.5 |
| 6. Minimum exterior setback (metres) | 3.1 | 15.2 | 3.1 |
| 7. Minimum interior setback (metres) | 0.01 | 9.1 | 0.01 |
| 8. Minimum landscaped open space | 10% | 5% | 10% |
| 9. Maximum lot coverage | — | 20% | — |
| 10. Maximum number of dwelling units per lot | 1.0 | — | — |

Additional Provisions for Lots Zoned CG

    - In addition to the provisions of Table 15.6.1., uses in the CG Zone must comply with the following provisions:

      - The minimum interior setback where abutting any Zone other than a Commercial Zone or an Institutional Zone is 3.1 metres;

      - The maximum gross leasable area of a building containing more than one commercial use is 2,000.0 square metres, not including any floor area which is exclusively devoted to institutional, office or residential use; and

      - Outdoor storage is prohibited.

### Marine Commercial Zone (CW)

    - The use of any lot or building in the CW Zone must comply with the provisions of Table 15.7.1.

#### Table 15.7.1. – CW Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | 560.0 |
| 2. Minimum lot frontage (metres) | 23.0 |
| 3. Maximum height (metres) | 10.7 |
| 4. Minimum front setback (metres) | 3.0 |
| 5. Minimum rear setback (metres) | 15.0 |
| 6. Minimum exterior setback (metres) | 3.0 |
| 7. Minimum interior setback (metres) | 0.01 |
| 8. Minimum landscaped open space | 10% |
| 9. Maximum lot coverage | 40% |
| 10. Maximum number of dwelling units per lot | 1.0 |

(By-Law Number 2022-62; 2024-332)

#### Additional Provisions for Lots Zoned CW

    - In addition to the provisions of Table 15.7.1., uses in the CW Zone must comply with the following provisions:

      - The minimum interior setback where an interior lot line abuts a Zone other than a Commercial Zone is 9.0 metres; and

      - Outdoor storage is only permitted in the rear yard, except for outdoor storage associated with a marina, which is permitted in any yard.

### Harbour Zone (HB)

    - The use of any lot or building in the HB Zone must comply with the provisions of Table 15.8.1.

#### Table 15.8.1. – HB Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | 10.7 |
| 4. Minimum front setback (metres) | 0.0 |
| 5. Minimum rear setback (metres) | 0.0 |
| 6. Minimum exterior setback (metres) | 0.0 |
| 7. Minimum interior setback (metres) | 0.0 |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | 50% |

Additional Provisions for Lots Zoned HB

    - In addition to the provisions of Table 15.8.1., uses in the HB Zone must comply with the following provisions:

      - No loading spaces are required;

      - Despite Clause 4.23.1., the minimum separation distance from the high water mark of a waterbody for any use or building is 10.0 metres;

      - No outdoor storage is permitted, except for outdoor storage associated with a marina, which is permitted in any yard;

      - Drive-throughs are prohibited;

      - Buildings are required to have ground floor commercial uses on the first storey where any portion of the lot aligns with the area identified as “Required Ground Floor Commercial” on Schedule 3; and

      - Where ground floor commercial uses are required by Subclause 5., the entire streetwall of the first storey, excluding areas devoted to a lobby or other shared entrances/exits for other permitted uses, must be occupied by

commercial uses. Portions of the floor area of the first storey that do not have an exterior wall facing a street line may be occupied by uses that service the building such as loading spaces, waste management facilities and rooms, mechanical rooms, bike parking facilities and other similar uses.

# Section 16: Employment Zones

### All Employment Zones

    - For the purposes of this By-law, Employment Zones include Business Park Zone (M1), General Industrial Zone (M2), Heavy Industrial Zone (M3), Employment Service Zone (M4) and Waste Management Zone (M5).

    - Uses permitted in Employment Zones are limited to the uses identified in Table 16.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 16.1.2., the following corresponding provisions apply:

      - Is required to operate within an enclosed building.

      - Is permitted only as an accessory use to a principal use on the lot and is limited to a maximum gross floor area of 25% of the gross floor area of the principal use.

      - Is limited only to convenience commercial uses in a retail store, in accordance with the complementary use policies of the Official Plan.

      - Is permitted only as an accessory use to a principal use on the lot.

      - Is permitted as a principal use and may occupy 100% gross floor area on the

lot, in accordance with the complementary use policies of the Official Plan.

Table 16.1.2. - Permitted Uses in the Employment Zones

| Use | M1 | M2 | M3 | M4 | M5 |
| --- | --- | --- | --- | --- | --- |
| animal care | — | — | — | ●5 | — |
| animal shelter | — | — | — | ● | — |
| automobile body shop | — | ● | ● | — | — |
| automobile repair shop | — | ● | ● | — | — |
| banquet hall | — | — | — | ●5 | — |
| call centre | ● | — | — | ●1 | — |

| Use | M1 | M2 | M3 | M4 | M5 |
| --- | --- | --- | --- | --- | --- |
| catering service | ● | ● | — | ●1 | — |
| contractor’s yard | ●1 | ● | ● | ●1 | — |
| day care centre | — | — | — | ●5 | — |
| equipment rental | — | ● | ● | — | — |
| feedmill | — | ● | — | — | — |
| financial institution | — | — | — | ●5 | — |
| fitness centre | — | — | — | ●5 | — |
| heavy equipment or truck repair shop | — | ● | ● | — | — |
| heavy industrial uses | — | — | ● | — | — |
| hotel | — | — | — | ●5 | — |
| industrial repair shop | — | ● | ● | — | — |
| laboratory | ● | — | — | ●1 | — |
| light industrial use | ●1 | ● | ● | ●1 | — |
| office | ● | ●4 | ●4 | ●1 | ●4 |
| outdoor storage | ●4 | ●4 | ●4 | — | ● |
| personal service shop | — | — | — | ●5 | — |
| place of worship | — | — | — | ●5 | — |
| production studio | ● | — | — | ●1 | — |
| recreation facility | — | — | — | ●5 | — |
| repair shop | ●1 | ● | ● | ●1 | — |
| research establishment | ● | — | — | ●1 | — |
| restaurant | — | — | — | ●5 | — |
| retail store | ●2 | ●2 | ●2 | ●3 | ●2 |
| salvage yard | — | — | — | — | ● |
| self-service storage facility | — | ● | — | — | — |
| towing compound | — | ● | ● | — | ● |
| training facility | ●1 | — | — | ●1 | — |
| transportation depot | ●1 | ● | ● | ●1 | — |

| Use | M1 | M2 | M3 | M4 | M5 |
| --- | --- | --- | --- | --- | --- |
| warehouse | ●1 | ● | ● | ●1 | — |
| waste disposal area | — | — | — | — | ● |
| waste processing site | — | — | — | — | ● |
| waste transfer station | — | — | — | — | ● |
| wastewater treatment facility | — | — | ● | — | ● |
| water supply plant | — | — | ● | — | — |
| wellness clinic | — | — | — | ●5 | — |
| wholesale establishment | ●1 | ● | ● | ●1 | — |
| workshop | ●1 | ● | ● | ●1 | — |

### Business Park Zone (M1)

    - The use of any lot or building in the M1 Zone must comply with the provisions of Table 16.2.1.

#### Table 16.2.1. – M1 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | 25.0 |
| 3. Maximum height (metres) | 15.0 |
| 4. Minimum front setback (metres) | 6.03 |
| 5. Minimum rear setback (metres) | 7.53 |
| 6. Minimum exterior setback (metres) | 6.03 |
| 7. Minimum interior setback (metres) | 6.03 |
| 8. Minimum landscaped open space | 15% |
| 9. Maximum lot coverage | — |

Additional Provisions for Lots Zoned M1

    - In addition to the provisions of Table 16.2.1., uses in the M1 Zone must comply with the following provisions:

      - Outdoor storage is not permitted on a lot that is adjacent to a lot in an Urban Residential Zone, Urban Multi-Unit Residential Zone or Heritage Zone or on a lot that is adjacent to a lot with a residential use. Where permitted, outdoor storage must:

        - comply with the setback provisions as if such use were a building;

        - not be located in a front yard or exterior yard, and must not be located in a rear yard that abuts a street line;

        - be provided with a visual screen a minimum height of 1.8 metres in such a manner that the outdoor storage is not visible from a street or a lot that is located in any Zone other than an Employment Zone; and

        - have an area no greater than 15% of the lot area.

      - Despite Subclause 1., when outdoor storage is established under the provisions of this By-law, the subsequent construction of a new residential use on a separate lot does not render such outdoor storage non-complying.

      - Where a lot line abuts a lot in any Zone other than an Employment Zone:

        - A minimum 3.0 metre wide planting strip must be provided along the portion of the lot line that abuts a lot in a Zone that is not an Employment Zone; and

        - The minimum height for the row of trees or a continuous hedgerow of evergreens or shrubs located in the planting strip is 1.5 metres;

        - Driveways, walkways and similar features are permitted to cut across the

planting strip perpendicularly; and

        - Where the lot line is an interior lot line or rear lot line, the minimum

setback from the lot line is 15.0 metres.

      - Where a lot has a street line that is located on the opposite side of the street

from any Zone other than an Employment Zone:

        - A minimum 3.0 metre wide planting strip must be provided along the portion of the street line that is opposite to the Zone that is not an Employment Zone; and

        - The minimum height for the row of trees or a continuous hedgerow of evergreens or shrubs located in the planting strip is 1.5 metres; and

        - Driveways, walkways and similar features are permitted to cut across the

planting strip perpendicularly.

      - All parking lots must be surfaced with asphalt, concrete, lockstone or a similar material approved by the City.

      - It is intended that complementary uses may be authorized through a minor variance in accordance with the policies of the Official Plan, and subject to, and in accordance with, the provisions of the Planning Act.

(By-Law Number 2022-62; 2024-332)

### General Industrial Zone (M2)

    - The use of any lot or building in the M2 Zone must comply with the provisions of Table 16.3.1.

#### Table 16.3.1. – M2 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | 21.0 |
| 3. Maximum height (metres) | 15.0 |
| 4. Minimum front setback (metres) | 8.0 |
| 5. Minimum rear setback (metres) | 7.53 |
| 6. Minimum exterior setback (metres) | 6.0 |
| 7. Minimum interior setback (metres) | 6.03 |
| 8. Minimum landscaped open space | 15% |
| 9. Maximum lot coverage | — |

Additional Provisions for Lots Zoned M2

    - In addition to the provisions of Table 16.3.1., uses in the M2 Zone must comply with the following provisions:

      - Outdoor storage is not permitted on a lot that is adjacent to a lot in an Urban Residential Zone, Urban Multi-Unit Residential Zone or Heritage Zone or on a lot that is adjacent to a lot with a residential use. Where permitted, outdoor storage must:

        - comply with the setback provisions as if such use were a building;

        - not be located in a front yard or exterior yard, and must not be located in a rear yard that abuts a street line;

        - be provided with a visual screen a minimum height of 1.8 metres and in such a manner that the outdoor storage is not visible from a street or a lot that is located in any Zone other than an Employment Zone; and

        - have an area no greater than 30% of the lot area;

      - Despite Subclause 1., when outdoor storage is established under the provisions of this By-law, the subsequent construction of a new residential use on a separate lot does not render such outdoor storage non-complying;

      - Where an interior lot line or rear lot line abuts a lot in any Zone other than an Employment Zone, the minimum setback from such lot line is 15.0 metres;

      - Heavy industrial uses existing as of the date of passing of this By-law are deemed to be permitted uses in the M2 Zone. Any development associated with an existing heavy industrial use must comply with the provisions in Table

16.3.1.; and

      - It is intended that complementary uses may be authorized through a minor variance in accordance with the policies of the Official Plan, and subject to, and in accordance with, the provisions of the Planning Act.

### Heavy Industrial Zone (M3)

    - The use of any lot or building in the M3 Zone must comply with the provisions of Table 16.4.1.

#### Table 16.4.1. – M3 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | 25.0 |
| 3. Maximum height (metres) | 15.0 |
| 4. Minimum front setback (metres) | 8.0 |
| 5. Minimum rear setback (metres) | 7.53 |
| 6. Minimum exterior setback (metres) | 6.0 |
| 7. Minimum interior setback (metres) | 6.03 |
| 8. Minimum landscaped open space | 15% |
| 9. Maximum lot coverage | — |

Additional Provisions for Lots Zoned M3

    - In addition to the provisions of Table 16.4.1., uses in the M3 Zone must comply with the following provisions:

      - Outdoor storage is not permitted on a lot that is adjacent to a lot in an Urban Residential Zone, Urban Multi-Unit Residential Zone or Heritage Zone or on a lot that is adjacent to a lot with a residential use. Where permitted, outdoor storage must:

        - comply with the setback provisions as if such use were a building;

        - not be located in a front yard or exterior yard, and must not be located in a rear yard that abuts a street line; and

        - be provided with a visual screen a minimum height of 1.8 metres and in such a manner that the outdoor storage is not visible from a street or a lot that is located in any Zone other than an Employment Zone;

      - Despite Subclause 1., when outdoor storage is established under the provisions of this By-law, the subsequent construction of a new residential use on a separate lot does not render such outdoor storage non-complying;

      - Where an interior lot line or rear lot line abuts a lot in any Zone other than an Employment Zone, the minimum setback from such lot line is 15.0 metres; and

      - It is intended that complementary uses may be authorized through a minor variance in accordance with the policies of the Official Plan, and subject to, and in accordance with, the provisions of the Planning Act.

### Employment Service Zone (M4)

    - The use of any lot or building in the M4 Zone must comply with the provisions of Table 16.5.1.

#### Table 16.5.1. – M4 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | 25.0 |
| 3. Maximum height (metres) | 15.0 |
| 4. Minimum front setback (metres) | 6.0 |
| 5. Minimum rear setback (metres) | 7.51 |
| 6. Minimum exterior setback (metres) | 6.0 |
| 7. Minimum interior setback (metres) | 6.01 |
| 8. Minimum landscaped open space | 15% |
| 9. Maximum lot coverage | — |

(By-Law Number 2022-62; 2024-332)

#### Additional Provisions for Lots Zoned M4

    - In addition to the provisions of Table 16.5.1., uses in the M4 Zone must comply with the following provisions:

      - Where an interior lot line or rear lot line abuts a lot in any Zone other than an Employment Zone, the minimum setback from such lot line is 15.0 metres;

      - Outdoor storage is prohibited;

      - All parking lots must be surfaced with asphalt, concrete, or lockstone or a similar material approved by the City; and

      - It is intended that complementary uses may be authorized through a minor variance in accordance with the policies of the Official Plan, and subject to, and in accordance with, the provisions of the Planning Act.

### Waste Management Zone (M5)

    - The use of any lot or building in the M5 Zone must comply with the provisions of Table 16.6.1.

#### Table 16.6.1. – M5 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | 15.0 |
| 4. Minimum front setback (metres) | 15.0 |
| 5. Minimum rear setback (metres) | 15.02 |
| 6. Minimum exterior setback (metres) | 15.0 |
| 7. Minimum interior setback (metres) | 15.02 |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | — |

Additional Provisions for Lots Zoned M5

    - In addition to the provisions of Table 16.6.1., uses in the M5 Zone must comply with the following provisions:

      - Outdoor storage must be provided with a visual screen a minimum height of

1.8 metres and in such a manner that the outdoor storage is not visible from a

street or a lot that is located in any Zone other than an Employment Zone;

      - Where an interior lot line or rear lot line abuts a lot in any Zone other than an Employment Zone, the minimum setback from such lot line is 30.0 metres;

      - All facilities for managing solid waste must be located within an enclosed building and constructed in such a manner that the waste material is not visible from a street or an adjacent non-industrial property; and

      - It is intended that complementary uses may be authorized through a minor variance in accordance with the policies of the Official Plan, and subject to, and in accordance with, the provisions of the Planning Act.

# Section 17: Transportation and Utilities Zones

### All Transportation and Utilities Zones

    - For the purposes of this By-law, Transportation and Utilities Zones include Airport Zone (TA), Transportation and Railway Zone (TR) and Utility Installation or Corridor Zone (TU).

    - Uses permitted in Transportation and Utilities Zones are limited to the uses

identified in Table 17.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 17.1.2., the following corresponding provisions apply:

      - Is permitted only as an accessory use to a principal use on the lot; and

      - An existing golf course and driving range is a permitted principal use on all lands in the TA Zone.

Table 17.1.2. - Permitted Uses in the Transportation and Utilities Zones

| Use | TA | TR | TU |
| --- | --- | --- | --- |
| agricultural use | — | — | ●1 |
| airport facilities | ● | — | — |
| club | ● | — | — |
| ferry terminal | — | ● | — |
| golf course | ●2 | — | — |
| outdoor storage | — | — | ●1 |
| personal service shop | — | ●1 | — |
| restaurant | — | ●1 | — |
| retail store | — | ●1 | — |
| transformer station | — | — | ● |
| transportation terminal | — | ● | — |
| water supply plant | — | — | ● |

### Airport Zone (TA)

    - The use of any lot or building in the TA Zone must comply with the provisions of Table 17.2.1.

Table 17.2.1. – TA Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | 15.2 |
| 4. Minimum front setback (metres) | 21.3 |
| 5. Minimum rear setback (metres) | 7.6 |
| 6. Minimum exterior setback (metres) | 21.3 |
| 7. Minimum interior setback (metres) | 3.1 |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | — |
| 10. Maximum number of dwelling units per lot | — |

### Transportation and Railway Zone (TR)

    - The use of any lot or building in the TR Zone must comply with the provisions of Table 17.3.1.

#### Table 17.3.1. – TR Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | 18.0 |
| 4. Minimum front setback (metres) | 7.5 |
| 5. Minimum rear setback (metres) | 0.01 |
| 6. Minimum exterior setback (metres) | 7.5 |
| 7. Minimum interior setback (metres) | 4.5 |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | — |
| 10. Maximum number of dwelling units per lot | — |

Additional Provisions for Lots Zoned TR

    - In addition to the provisions of Table 17.3.1., uses in the TR Zone must comply with the following provisions:

      - Bus stations must comply with the following:

        - A minimum 3.0 metre wide planting strip must be provided along all lot lines.

      - Train stations must comply with the following:

        - A minimum 7.5 metre wide planting strip must be provided along all lot lines.

### Utility Installation or Corridor Zone (TU)

    - The use of any lot or building in the TU Zone must comply with the provisions of Table 17.4.1.

#### Table 17.4.1. – TU Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | — |
| 4. Minimum front setback (metres) | 7.5 |
| 5. Minimum rear setback (metres) | 7.5 |
| 6. Minimum exterior setback (metres) | 6.0 |
| 7. Minimum interior setback (metres) | 6.0 |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | — |
| 10. Maximum number of dwelling units per lot | — |

Additional Provisions for Lots Zoned TU

    - In addition to the provisions of Table 17.4.1., uses in the TU Zone must comply with the following provisions:

      - A minimum 4.5 metre wide planting strip must be provided along all street lines;

      - Where a lot abuts a lot in an Institutional Zone or Commercial Zone, a minimum 3.0 metre wide planting strip must be provided adjacent to any such lot line, which must have a minimum height of 1.5 metres for the row of trees or a continuous hedgerow of evergreens or shrubs;

      - Where a lot is located on the opposite side of the street from an Institutional Zone or Commercial Zone, a minimum 3.0 metre wide planting strip must be provided along the portion of the street line that is opposite to such Zone;

      - No planting strip is required adjacent to a lot line that abuts an Employment Zone, Transportation and Utilities Zone, or any combination of such Zones; and

      - A minimum 4.5 metre wide planting strip must be provided along all other lot lines, which must have a minimum height of 1.5 metres for the row of trees or a continuous hedgerow of evergreens or shrubs.

# Section 18: Open Space Zones

### All Open Space Zones

    - For the purposes of this By-law, Open Space Zones include Protected Open Space Zone (OS1), General Open Space Zone (OS2) and Development Reserve Zone (DR).

    - Uses permitted in Open Space Zones are limited to the uses identified in Table 18.1.2., and are denoted by the symbol “●” in the column applicable to each Zone and corresponding with the row for a specific permitted use. Where the symbol “— “ is identified in the table, the use is not permitted in that Zone.

    - Where a permitted use includes a reference number in superscript beside the “●”

symbol in Table 18.1.2., the following corresponding provisions apply:

      - Is permitted only as an accessory use to a principal use on the lot;

      - Is permitted if the principal building legally existed on the lot as of the date of passing of this By-law; and

      - Is permitted in the rural area.

Table 18.1.2. - Permitted Uses in the Open Space Zones

| Use |  |  |  | OS1 | OS2 | DR |
| --- | --- | --- | --- | --- | --- | --- |
| Residentia house |  |  | l | — | — | ●2 |
| Non-resid agricultur |  |  | ential al use | — | — | ●3 |
| campgro |  | und |  | — | ● | — |
| cemetery |  |  |  | — | ● | — |
| club |  |  |  | — | ● | — |
| communi |  | ty centre |  | — | ● | — |
| elementa |  | ry school |  | — | ● | — |
| fitness ce |  | ntre |  | — | ● | — |
| forestry | use |  |  | — | ● | — |
| golf cour |  | se |  | — | ● | — |
| library |  |  |  | — | ● | — |

| Use | OS1 | OS2 | DR |
| --- | --- | --- | --- |
| marina | — | ● | — |
| museum | — | ● | — |
| office | — | ●1 | — |
| park | ● | ● | ● |
| place of worship | — | — | — |
| recreation facility | — | ● | — |
| secondary school | — | ● | — |

### Protected Open Space Zone (OS1)

    - The use of any lot or building in the OS1 Zone must comply with the provisions of Table 18.2.1.

Table 18.2.1. – OS1 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | — |
| 4. Minimum front setback (metres) | — |
| 5. Minimum rear setback (metres) | — |
| 6. Minimum exterior setback (metres) | — |
| 7. Minimum interior setback (metres) | — |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | — |
| 10. Maximum number of dwelling units per lot | — |

### General Open Space Zone (OS2)

    - The use of any lot or building in the OS2 Zone must comply with the provisions of Table 18.3.1.

Table 18.3.1. – OS2 Provisions

| Zoning Provision | all permitted uses |
| --- | --- |
| 1. Minimum lot area (square metres) | — |
| 2. Minimum lot frontage (metres) | — |
| 3. Maximum height (metres) | 23.0 |
| 4. Minimum front setback (metres) | 7.5 |
| 5. Minimum rear setback (metres) | 7.5 |
| 6. Minimum exterior setback (metres) | 7.5 |
| 7. Minimum interior setback (metres) | 7.5 |
| 8. Minimum landscaped open space | — |
| 9. Maximum lot coverage | — |
| 10. Maximum number of dwelling units per lot | — |

### Development Reserve Zone (DR)

    - The use of any lot or building in the DR Zone must comply with the provisions of Table 18.4.1.

#### Table 18.4.1. – DR Provisions

| Zoning Provision | house | all other permitted uses |
| --- | --- | --- |
| 1. Minimum lot area (square metres) | — | — |
| 2. Minimum lot frontage (metres) | — | — |
| 3. Maximum height (metres) | — | — |
| 4. Minimum front setback (metres) | 7.6 | 6.1 |
| 5. Minimum rear setback (metres) | 7.6 | 7.6 |
| 6. Minimum exterior setback (metres) | 7.6 | 6.1 |
| 7. Minimum interior setback (metres) | 7.6 | 7.6 |
| 8. Minimum landscaped open space | — | — |
| 9. Maximum lot coverage | 20% | 20% |
| 10. Maximum number of dwelling units per lot including accessory houses | 4.0 | — |

(By-Law Number 2022-62; 2024-332; 2024-333)

# Section 19: Environmental Zones

### Environmental Protection Area Zone (EPA)

    - No use, building or any part of a private sewage system is permitted in the EPA Zone, except:

      - Water quality management uses;

      - Flood control components or works;

      - Erosion control components or works;

#### Passive recreation;

      - Educational uses, generally of a passive nature, excluding associated

#### buildings;

      - New public or private works or utilities such as pipelines, roads, bridges or parking areas, where such facilities are not feasible outside of the Environmental Protection Area;

#### Marine facilities;

      - Water based facilities associated with existing permitted marinas; and

      - Parks and conservation uses.